3GPP TSG-WG SA2 Meeting #155	S2-2302291

# Athens, Greece, February 20 – 24, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4120 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Informative Annex for UDM sending CH controlled list of SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, vivo, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-08 has concluded that the home network UDM may determine to update UE with prioritized list of hosting network information (i.e. CH controlled list of SNPN with validity conditions in the normatvie phase) using SoR procedure. The triggers can be UE location and UE subscription data change, e.g. via external parameter provisioning. In the 154AHE meeting, we have added an Annex clause to capture the informative description for localized services. Therefore, this CR aims to add a new subclause in the annex to provide informative description about how home network UDM is triggered to update UE with the CH controlled list of SNPNs with validity condition based on the TR conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new subclause in the annex part to provide informative description about how UDM is triggered to update the CH controlled list of SNPNs. The description is the guideline which is implementation specific. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear when the UDM need to provide the CH controlled list of SNPNs with validity condition to UE for accessing localized service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

x.y	Triggers for UDM to update UE with CH controlled list of SNPNs for accessing to localized services

When the UDM in the Credentials Holder supports to update UE with CH controlled list of SNPN with validity conditions for accessing to localized services via a SNPN, it is assumed that there is agreement between the Credentials Holder and the Localized Service Provider (LSP).

The UDM in the Credentials Holder may obtain the information related to the UE subscribed localized service (e.g. validity conditions and the SNPNs support providing access to the localized service) via external parameter provisioning. UDM in the Credentials Holder can determine to update UE with CH controlled list of SNPN with validity conditions via SoR procedure to enable UE to select and access to a SNPN for localized services.

If the validity condition includes location information (e.g. geographical location), existing mechanism can be implemented to trigger the UDM in the Credentials Holder to update the CH controlled list of SNPN with validity condition based on UE location so that the UE can use the list for SNPN selection when the UE is near the geographical location, which can reduce the frequency of scaning whether the SNPN is available. Such mechanism is implementation-specific.

# * * * * End of changes * * * *