3GPP TSG-SA WG2 Meeting #155	S2-2302436

20 – 24 February 2023, Athens, GR	(revision of S2-2301767)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3956 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transfer of Satellite Coverage Data to a UE and AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Qualcomm Incorporated,] Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusions for FS_5GSAT_Ph2 in TR 23.700-28, include supporting the the transfer of satellite coverage information to a UE and AMF. This CR introduces the support as proposed in a comparison of alternative solutions in S2-2300668. Changes since the agreed rev 1 from SA2#154-AHE: There are Editor’s notes on how AMF/MME receives coverage information, e.g. whether both O&M and AF options are to be supported. The option where AF provides coverage information to AMF/MME has not been discussed in much detail, and the TR conclusions do not contain any agreements on how this should be done (e.g. procedure, impacts to NEF services, impacts EPS and 5GS). It is therefore proposed in this CR to use only the O&M based approach in rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for transfer of coverage data to a UE and to an AMF. Changes since the agreed rev 1 from SA2#154-AHE: Clairfy that the AMF/MME receives coverage data from O&M. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transfer of enhanced satellite coverage data to a UE compared to support for EPS in Release 17 and to an AMF would not be supported. Changes since the agreed rev 1 from SA2#154-AHE: Remaining Editor’s notes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X.Y (new), 5.4.X.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change (all new) ****

5.4.X Support of discontinuous network coverage

#### 5.4.X.Y UE coverage information provisioning to the UE

A UE may use satellite coverage information for satellite access to support discontinuous coverage operations. Coverage information can be provided to a UE by an external server by user plane or control plane in case NIDD is used.

Editor’s Note: 	It is FFS whether only UP approach is sufficient or NIDD is also to be supported.

#### 5.4.X.Z UE Coverage information provisioning to the AMF

The AMF may use UE satellite coverage information to support satellite access by UEs with discontinuous coverage operation. Coverage information may be provisioned to the AMF by O&M.

**** End of Changes ****