3GPP TSG-SA2 Meeting #155	S2-2302500

February 20-24, 2023, Athens

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3992 | Rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling Access to Localized Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Samsung], Ericsson, [Deutsche Telekom, Futurewei, LG Electronics] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-08 has the below conlsuon for KI#4 : Conclusion for how localized service is accessed in hosting network per agreed conditions 1. Validity conditions provided to the UE as part of the localized service information can be used to restrict the UE's access of the hosting network. 2. Existing methods, such as network slicing, forbidden area restriction, service area restriction, CAG, LADN, URSP rules can also be utilized to restrict UE's access, i.e. no need for additional normative work for access control in hosting network. For KI#5 :Enabling access to localized services via a specific hosting network The following conclusions are agreed for normative work: - When the hosting network is an SNPN which provides localized services and the home network is an HPLMN, the architecture specified in clause 5.30.2.7 of TS 23.501 [3] is reused for the UE to access both home network services (using the SNPN as underlay network) and localized services (via the SNPN). - If the hosting network is a PNI-NPN, existing mechanisms (e.g. roaming architecture, network slicing, etc.) are reused for the UE to access home network services. - The existing URSP rules or LADN feature can be re-used for a UE to access the Localized services after the UE has registered to a Hosting network. URSP rules are pre-configured in the UE, provisioned by the PCF of the home network or provisioned by the PCF of the hosting network according to existing principles. This CR documents how to apply existing functionality to enable the UE getting localized service information needed to access the localized service from hosting network and how the hosting network restrict the access using existing LADN principle. Rev2: Updated reference to CR3842 in other comments in cover sheet Added time to S-NSSAI restriction as time based S-NSSAI restriction is progressing as part of eNS_Ph3. Removed Editor’s note, as there are no normative work progressing in other work items making SM procedures for a DNN to be time restricted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Informative Annex including: 1 ) UE gets the localized service information (either from HPLMN) or from hosting network using URSP and LADN 2) The UE behaviour and AMF/SMF action to restrict the UE as per existing LADN feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how localized services can be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex x.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is to be implemented after CR3842 (adding the new annex so this clause goes under the new Annex). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# x.4	Enabling the UE access to localized services

The access to a localized service is made available in a specific area and/or a specific period of time.

After the UE has successfully registered to PNI-NPN/SNPN providing access to the localized service, the UE can be configured (e.g. by HPLMN or hosting network) with URSP rules using existing principles (see TS 23.503 [45] clause 6.6.2.2).

The URSP rules can include an association between the UE application and the DNN/S-NSSAI which is meant for a particular localized service. The URSP rules can also include "Route Selection Validitation Criteria" as described in TS 23.503 [45] table 6.6.2.1-3, with the time/location defined for the particular localized service.

The existing LADN feature per the clause 5.6.5 can also be used for enabling the UE access to localized service which is defined by a LADN DNN. The S-NSSAI used for a localized service can be restricted to a specific area and time as described in clause 5.15.

END OF CHANGE