3GPP TSG-SA WG2#155	S2- 2302507

Athens, Greece, 20-24 February 2023	(was S2-230xxxx)

Source:	Qualcomm Incorporated

Title:	Way forward for PEGC route selection policies

Document for:	Discussion and Agreement

Agenda Item:	9.3.2

Work Item / Release:	PIN / Rel-18

Abstract of the contribution: This paper addresses the question whether to introduce new PIN Route Selection Policies (PRSP) or to extend URSP, analyses which parameters can be used to enable route selection by PEGC and proposes a way forward.

# 1.	Discussion

## 1.1	Overview

During the SA2#154 meeting, SA2 decided – based on show of hands – that new UE policies will be introduced for UEs acting as PEGC to determine the PDU Session to be used for PIN traffic. In line with this, the following conclusion was documented in TR 23.700-88 [1]:

11)	If AF for PIN is used, the AF may provide necessary parameters to 5GC which may be considered by PCF to generate the PIN Route Selection Policy for PDU Session selection by PEGC(s) and to generate the URSP accordingly for PEMC(s).

It is worth noting that no conclusions have been made which information needs to be included in the PIN Route Selection Policy.

During SA2#154AHE the conclusion to introduce new UE policies has been challenged by different companies based on the argument that any additional PIN related traffic descriptor information could simply be added to URSP and that this way URSP would also be applicable to PIN scenarios. In addition, it was claimed that existing URSP traffic descriptors would equally apply to PIN scenarios.

This paper analyses whether existing URSP traffic descriptors can apply to PIN scenarios and discusses which new traffic descriptors are needed for PIN scenarios. Based on this analysis, this paper will conclude with a way forward proposal.

## 1.2	Existing URSP traffic descriptor components are inapplicable to PIN traffic

Thus far, URSP focuses on determining "if a detected application can be associated to an established PDU Session " (TS 23.503 [3] clause 6.1.2.2.1). Similarly, TS 23.503 [3] clause 6.6.2.3 (UE procedure for associating applications to PDU Sessions based on URSP) states that "For every newly detected application the UE evaluates the URSP rules in the order of Rule Precedence and determines if the application is matching the Traffic descriptor of any URSP rule.".

Observation 1: URSP as it is defined focuses on detected applications.

The assumption is that applications can provide the UE with application information before PDU Session Establishment. This information is then matched against the Traffic descriptors of the URSP rules in the UE. Based on this, the UE can then determine the related Route Selection Descriptor (RSD) and finally establish a PDU Session for the application (unless there is already a PDU Session with the same RSD).

Observation 2: URSP relies on applications providing the UE with application information before PDU Session Establishment.

Table 1: URSP Traffic descriptor components
(Source: TS 23.503 [3] Table 6.6.2.1-2)

| Information name | Description |
| --- | --- |
| Rule Precedence | Determines the order the URSP rule is enforced in the UE. |
| Traffic descriptor | This part defines the Traffic descriptor components for the URSP rule. |
| Application descriptors | It consists of OSId and OSAppId(s). (NOTE 2) |
| IP descriptors (NOTE 5) | Destination IP 3 tuple(s) (IP address or IPv6 network prefix, port number, protocol ID of the protocol above IP). |
| Domain descriptors | FQDN(s) or a regular expression which are used as a domain name matching criteria (NOTE 6). |
| Non-IP descriptors (NOTE 5) | Descriptor(s) for destination information of non-IP traffic |
| DNN | This is matched against the DNN information provided by the application. |
| Connection Capabilities | This is matched against the information provided by a UE application when it requests a network connection with certain capabilities. (NOTE 4) |

Table 1 lists traffic descriptor components supported by URSP against which information provided by applications can be matched.

For applications running on a UE it is possible to define an internal interface to enable applications to provide such information to the UE (e.g., to a connection manager component of the UE) in a similar way as Traffic descriptor information can be provided using AT commands between TE and MT (see TS 27.007 [2] clause 10.1.75).

Instead, applications running on PIN Elements cannot provide application information to the UE acting as PEGC because no mechanism has been studied to enable PIN Elements to provide information such as OSId/AppID, Domain descriptors, DNN, or Connection capabilities to the UE acting as PEGC.

As a result, a UE cannot evaluate URSP rules with Application descriptors, Domain descriptors, DNN or Connection Capabilities for PIN scenarios.

Also, even if it was possible to provide Application descriptors, Domain descriptors, DNN and Connection Capabilities from PIN Elements to PEGC before PDU Session Establishment, it is unclear whether MNOs would like to rely on such information as information provided by PIN Elements cannot be trusted in the same way as information provided by applications running on the UE (which are typically installed through managed platforms, e.g., app stores).

Observation 3: Application descriptors, Domain descriptors, DNN and Connection Capabilities are not applicable to PIN traffic because PIN Elements cannot provide the related information to the UE. Also, it is unlikely that MNOs would like to rely on such information provided by PIN Elements as those cannot be trusted in the same way as information provided by applications running on the UE (which are typically installed through managed platforms, e.g., app stores)

For IP descriptors one could argue that the PEGC can potentially analyse traffic sent by PIN Elements and match the destination IP address and port information of IP packets sent by PIN Elements against IP descriptor information. However, it is important to re-emphasize that this would possibly need to happen before PDU Session Establishment. The issue is that at this point PIN Elements do not have a globally routable IP address yet because a routable address, e.g., an IPv6 prefix, can only be assigned after PDU Session Establishment. This implies that the PEGC would need to assign private IP addresses to PINEs and translate PINE IP packets so that the source IP address of the packet from the PIN element matches the routable IP address or IPv6 prefix assigned for the PDU Session.

In summary, this can only work if the PEGC performs Network Address Translation (NAT) for PINE traffic (including for IPv6).

Moreover, even if NAT was supported by PEGC, the other issue is that typically applications perform a DNS query before establishing a connection to a specific server. If an application running on a PIN element sends a DNS query (over UDP or HTTPS), then the DNS query would already trigger the PEGC to establish a PDU session for the PIN traffic.

It is also important to highlight that NAT support is not supported for PIN at least for IPv6 according to the following conclusion in TR 23.700-88 clause 8.4, which states that IPv6 prefix delegation will be used for IP address assignments to PINEs:

IPv6 Prefix Delegation as described in clause 4.6.2.3 of TS 23.316 [5], or DHCP proxy by PEGC, or Framed Routing as described in clause 5.6.14 of TS 23.501 [2] are applied for IP address allocation of PINEs connected to PEGC.

Observation 4: IP descriptors are not applicable to PIN traffic as a basis for PDU Session Establishment.

Given Observations 1-4 we can conclude that existing URSP traffic descriptor components are not applicable to PIN traffic.

Conclusion 1: Existing URSP traffic descriptor components are not applicable to PIN traffic.

## 1.3	Enabling route selection for PIN scenarios based on PIN ID

Given the analysis in the previous section, using information provided by PINEs to determine the route to select is not an option as no mechanism has been studied for PINEs to provide such information to PEGC. Similarly, analysing traffic sent by PINEs as a basis to determine the PDU Session to use is not an option as argued above.

An alternative approach is to assume that

-	the MNO assigns a PIN ID for each PIN of a subscriber and provides it to the subscriber using mechanism beyond the scope of 3GPP. (Note that the PIN ID only needs to be unique per subscriber.)

-	PINs are configured and identified in the PEGC based on the PIN ID.

Route selection policies for PIN scenarios can then be based on the PIN ID as depicted in Figure 1.

Figure 1: Determining the PDU Session to use based on a PIN ID in the Route selection policy

In summary, this paper proposes to introduce a new PIN ID traffic descriptor component, which can then be used by the PEGC to identify the PDU session to use for traffic of a particular PIN identified by the related PIN ID.

Proposal 1: Perform route selection in PEGC based on a new PIN ID traffic descriptor component where the PIN ID is assumed to identify a specific PIN supported by the PEGC.

## 1.4	Introduce new UE policies for PIN scenarios or extend URSP

Given the analysis in section 1.2, existing URSP traffic descriptor components are not applicable to PIN scenarios. For this reason, the previous section proposed to introduce a new PIN ID traffic descriptor component.

As depicted in Figure 2, this implies that the existing URSP traffic descriptor components only apply to route selection for applications running on the UE whereas the new PIN ID traffic descriptor component only applies to PIN scenarios.

Figure 2: Existing URSP traffic descriptor components and new PIN ID are used for different purposes in the UE.

In other words, the existing URSP traffic descriptor components and the new PIN ID traffic descriptor component are evaluated differently within the UE. While existing URSP traffic descriptor components are evaluated based on information provided by applications running on the UE (i.e., based on interaction with applications running on the UE), the PIN ID traffic descriptor is evaluated based on configured PINs.

To simplify enforcement, it is therefore essential to clearly separate route selection policies that are used for applications running on the UE and route selection policies that are used for PINs.

Two options can be distinguished to reach this goal:

-	Option 1: Introduce new PIN Route Selection Policies (PRSP), which only supports a PIN ID traffic descriptor component.

-	Option 2: Extend URSP with a PIN ID traffic descriptor component and an indication, which identifies whether the rule applies to traffic from an application on the UE or to traffic from an application running on a device attached to the UE (i.e., a PIN element).

The benefit of Option 1 is that it provides a clear separation between rules for applications and rules for PIN scenarios. With respect to Option 1 it is also important to highlight that the impact to the system is limited to UE and PCF because the mechanism to deliver UE policies from PCF to the UE as defined in TS 24.501 [4] is agnostic to the actual UE policy and can therefore be fully reused. The same was for instance done when V2X or 5G ProSe policies were introduced.

Proposal 2: Enable route selection for PIN scenarios be either

-	(Option 1) introducing PIN Route Selection Policies (PRSP), which only support a PIN ID traffic descriptor component; or by

-	(Option 2) extending URSP with a PIN ID traffic descriptor component and an indication, which identifies whether the rule applies to traffic from an application on the UE or to traffic from an application running on a device attached to the UE (i.e., a PIN element).

# 2	Way forward

This paper concludes the following:

Conclusion 1: Existing URSP traffic descriptor components are not applicable to PIN traffic.

In addition, this paper makes the following two proposals.

Proposal 1: Perform route selection in PEGC based on a new PIN ID traffic descriptor component where the PIN ID is assumed to identify a specific PIN supported by the PEGC.

Proposal 2: Enable route selection for PIN scenarios be either

-	(Option 1) introducing PIN Route Selection Policies (PRSP), which only supports a PIN ID traffic descriptor component, or by

-	(Option 2) extending URSP with a PIN ID traffic descriptor component and an indication, which identifies whether the rule applies to traffic from an application on the UE or to traffic from an application running on a device attached to the UE (i.e., a PIN element).

With respect to Proposal 2 only one option is expected to be selected during the SA2 meeting. CRs have been submitted for Option 1 ([5] and [6]) and for Option 2 in [7].

# 3.	References

[1]	3GPP TR 23.700-88: "Study on architecture enhancements for Personal IoT Network (PIN)".

[2]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[3]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	Qualcomm: "23.503: PIN Route Selection Policy"; S2-2302509.

[6]	Qualcomm: "23.502: AF guidance of PRSP configuration "; S2-2302514.

[7]	Qualcomm: "URSP handling for PIN"; S2-2302516.