3GPP TSG-WG SA2 Meeting #155	S2-2302670

# Athens, Greece, February 20 – 24, 2023		(revision of S2-2301374)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0835 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Policy update to support policy control enhancements for multi-modal services. |
| --- | --- |
| Source to WG: | [China Telecom], [Ericsson], Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the KI#1&KI#2 conclusions in TR 23.700-60, multi-modal flows related to single UE/multiple UEs require policy control enhancements. Therefore, the interaction between PCF and AF needs to be enhanced to provide new requirements and parameters for XR services, the QoS monitoring capabilities need to be enhanced to realize multi-modal services, and the PCC rule should be updated to support policy coordination among a group of associated flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The interaction between PCF and AF is updated to include the requirements and paramters agreed in the KI#1&KI#2 conclusions. The QoS monitoring function is updated to help realize multi-modal services. The PCC rule is updated to support policy coordination among a group of associated flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to support policy control enhancements for XR traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.1.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR ... TS 23.502 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

6.1.3.X Support of eXtended Reality and Interactive Media Services

6.1.3.X.a	Support for delivery of multi-modal services

Enablers to support XR services and multi-modal communication services are defined in TS 23.501 [2] clause 5.37.x.

For the delivery of multi-modal services, the AF may send a request to the NEF to set up a data session with specific QoS requirements, as described in clause 6.1.3.22, including the following additional attributes where the AF provides specific information for multi-modal applications:

- Multi-modal Service Requirements: Services requirements for multi-modal services (e.g. the QoS monitoring requirements for multiple IP data flows associated to a multi-modal service).

- Multi-modal flows belonging to the same multi-modal service instance share the same Multi-modal Service ID.

The NEF authorizes the AF request, and interacts with the PCF via the Npcf_PolicyAuthorization service. The request to the PCF includes Multi-modal Service ID as well as service requirements for each media flow that comprise the multi-modal service. The request to the PCF includes also the Multi-modal Service Requirements if received from the AF.

The AF may request at the same time multiple media flows, to a UE or multiple UEs, to be setup with a specific QoS and priority handling each. Such request can be implemented in one or multiple AF requests. Multi-modal Service ID is used to bind the media flows belonging to the same multi-modal service instance in one or multiple AF requests.

PCF authorizes the service requirements of media flows with the same Multi-modal Service ID in the same AF request together. When receives further AF request with the same Multi-modal Service ID and PCF authorization fails, PCF indicates to the AF. Application may decide how to deal with the media flows with already authorized AF request with the same Multi-modal Service ID (e.g., stop them, adjust the AF request).

PCF coordinates the QoS policies of multi-modal flows sharing the same Multi-modal Service ID based on Multi-modal Service Requirements, and reports to AF whether the QoS requirements of these media flows are fulfilled according to the Multi-modal Service Requirements. When PCF receives further AF request with the same Multi-modal Service ID, PCF may adjust the QoS policies of previous multi-modal flows with the same Multi-modal Service ID. If receives the Multi-modal Service Requirements in the further AF request, PCF replaces the previous received one with the same Multi-modal Service ID. If does not receive the Multi-modal Service Requirements in the further AF request, PCF uses the previous received one with the same Multi-modal Service ID.

# * * * * End of changes * * * *