3GPP TSG-SA WG2 Meeting #155	S2-2302726

# February 20 – 24, 2023, Athens, Greece	(revision of S2-2300938)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0665 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF analytics to detect URSP enforcement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusions in KI#2 of clause 8.2 of TR 23.700-85 agreed that a new analytic ID is required to be supported by the NWDAF to monitor the traffic in one or multiple PDU sessions for statistics of URSP enforcement. The analytics request includes the expected traffic according to provisioned URSP rules via the S-NSSAI/DNN. The NWDAF reports statistics of UEs that route traffic to a PDU session of a specific S-NSSAI/DNN according to a provisioned URSP rules and a list of UEs that route traffic that it is not expected to be routed via a specific S-NSSAI/DNN according to provisioned URSP rule(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description and procedure of new analytics to be supported by the NWDAF to monitor known traffic for statistics of URSP enforcement. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions of KI#2 as part of the UEPO study will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

****************************** START OF CHANGE (ALL NEW TEXT)****************************

## 6.X	Statistics for traffic monitoring of known traffic according to provisioned URSP rule(s)

### 6.X.1	General

This clause specifies the procedure for an NWDAF to provide statistics on whether traffic of UEs via one or multiple PDU sessions is according to provisioned URSP rules.

The NWDAF collects traffic flow information of UE traffic via PDU session(s) established for a specific S-NSSAI and/or DNN and provides statistics of UEs that route traffic according to a specific URSP rule and UEs that route traffic which is not expected according to a provisioned URSP rule.

The assumption is that traffic according to a URSP rule is known (e.g. an Application ID or a known flow description). and that there are associated Packet Detection Rule(s) for the known traffic configured in the UPF. It is also assumed that such analytics would be requested only for the case where a URSP rule includes S-NSSAI and/or DNN in the Route Selection Descriptor of the provisioned URSP rule.

NOTE x: How traffic from different applications/URSP rules over the same S-NSSAI/DNN and/or PDU session is discerned is implementation-specific

The service consumer may be a PCF.

The consumer of these analytics may indicate in the request:

-	Analytics ID = "URSP statistics";

-	Target of Analytics Reporting: UE, group of UE or any UE;

-	information corresponding to the known traffic to be routed via a PDU session based on URSP traffic descriptor list (e.g. Flow Descriptors in URSP rule) of the provisioned URSP rule ;

-	Analytics Filter Information per URSP traffic descriptor list containing:

-	Area of Interest (i.e. the location of UEs where URSP rule enforcement is monitored) ;

-	S-NSSAI corresponding to a Route Selection Descriptor of the URSP rule ;

-	DNN corresponding to a Route Selection Descriptor of the URSP rule ;

-	An Analytics target period indicates the time period over which the statistics are requested.

-	Optionally, maximum number of objects.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

### 6.X.2	Input Data

NWDAF collects input data from the SMF and/or UPF. The detailed data are described in Table 6.X.2-1.

Table 6.X.2-1: Input data to detect traffic based on a URSP rule

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF, UPF | UE ID for the UE that established PDU sessions to an S-NSSAI/DNN |
| S-NSSAI | SMF, UPF | The S-NSSAI for which the PDU session is established |
| DNN | SMF, UPF | The DNN for which the PDU session is established |
| Application ID (NOTE X) | SMF, UPF | Application Identifier of the traffic according to the Traffic Descriptor of the URSP rule |
| Traffic Filtering Information (NOTE X, NOTE Y) | SMF, UPF | Information on Service Data Flows to report according to the Traffic Descriptor of the URSP rule |
| NOTE X: Application ID and Traffic Filtering are exclusive NOTE Y: Traffic Filtering Information includes also an indication to report traffic not matching traffic corresponding to a Traffic Descriptor of a URSP rule |  |  |

Editor’s Note: Additional input data from AF and other Data Sources are FFS.

The NWDAF collects input data from the UPF either indirectly via the SMF, or directly from the UPF using the "UserDataUsageMeasures" event exposure event as described in clause 4.15.4.x of 3GPP TS 23.502 [x]. Further details about input parameters are described in Table 4.15.4.x.1 of 3GPP TS 23.502 [x].

### 6.X.3	Output Analytics

The NWDAF collects input data from the UPF and provides statistics of UEs that route traffic over a PDU session of specific S-NSSAI/DNN according to a provisioned URSP rules and UEs that route traffic not according to a provisioned URSP rule

The output analytics is shown in Table 6.X.3-1.

Table 6.X.3-1: URSP Statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytic information is provided |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| URSP statistics | Identifies UE that use the provisioned URSP rules |
| > SUPI list (0..SUPImax) | List of SUPI(s) that route traffic according to provisioned URSP rule. |
| Traffic which is not expected according to a URSP rule | Identifies UEs that route traffic not expected according to provisioned URSP rules |
| > SUPI list (0..SUPImax) | List of SUPI(s) that route traffic that is not expected based on provisioned URSP rule. |
| >> Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol). |
| >> Volume | measures of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) |

### 6.X.4	Procedures

The procedure for deriving traffic monitor statistics is shown below.

Figure 6.X.4-1: NWDAF providing traffic monitoring statistics

1.	The Consumer NF (i.e. PCF) requests or subscribes to the NWDAF to request URSP statistics. The consumer includes within analytics filter the expected traffic via a PDU session according to the Traffic Descriptors within provisioned URSP rules to the UE and the S-NSSAI/DNN of the PDU session. It is assumed that there are associated Packet Detection Rule(s) for the expected traffic listed in the analytics request, i.e. the expected traffic is known.

2.	The NWDAF determines to collect data, either directly from the UPF or indirectly via the SMF and identifies the SMF and/or UPF to retrieve input data according to the S-NSSAI/DNN and UE information in the analytics request. The NWDAF retrieves data from the UPF where the UE has established a PDU session to the DNN/S-NSSAI included in the analytics request.

3.	The NWDAF collects traffic information using the "UserDataUsageMeasures" event exposure event.

4.	The NWDAF derives statistics indicating a list of UEs that route traffic according to the provisioned URSP rule and a list of UEs which route traffic that it is not expected according to the provisioned URSP rule.

5.	The NWDAF notifies URSP statistics.

************************************ END OF CHANGE ****************************************