3GPP TSG-WG SA2 Meeting #155	S2-2302869

February 20-24, 2023  Athens														 	(revision of S2-2300447)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0630 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNA_Ph3 DCCF relocation in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is concluded in TR 23.700-81 for KI#4 as follow: 7. When a DCCF and optionally MFAF is no longer able to serve a data consumer due to data source change, procedures for DCCF and optionally MFAF re-location (respectively re-selection) are supported according to solutions 12 and 66 (respectively 44). The selection of the appropriate procedure is determined by the DCCF based on an optional “No relocation” indicator provided by the DCCF service consumer: when the indicator is absent or set to “False”, the DCCF executes the relocation procedure, when set to “True”, the DCCF does not execute the relocation. Therefore, the procedure description of DCCF (re-)selection initiated by consumer and DCCF and MFAF relocation initiated by DCCF needs to be reflect in TS23.288 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Clause 6.2.6.3.1 - Introduce general information about DCCF relocation 2.Clause 6.2.6.3.Y - Introduce DCCF relocation initiated by DCCF 3.Clause 6.2.6.3.X - Introduce DCCF relocation initiated by consumer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCCF relocation is not updated to reflect conclusions in TR23.700-81 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.3.1, 6.2.6.3.X (new), 6.2.6.3.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

6.2.6.3.1		General

This clause specifies procedures for data collection using the DCCF described in clause 5A for cases other than obtaining analytics from an NWDAF (which is specified in clause 6.1.2). Two options are supported: data delivered via the DCCF, according to clauses 6.2.6.3.2 and 6.2.6.3.3 and data delivered via a messaging framework according to clauses 6.2.6.3.4 and 6.2.6.3.5. Which option to be used is determined by DCCF configuration.

Due to e.g. UE mobility, the source DCCF or MFAF may no longer be able to serve the UE. In such cases, the data consumer may select a new DCCF as described in clause 6.2.6.3.X, or source DCCF may execute the DCCF relocation procedure as described in clause 6.2.6.3.Y. The data consumer may indicate in the subscription request whether the DCCF should execute the relocation procedure or send a notification to the data consumer as specified in the reselection procedure. If the data consumer does not send an indicator in the subscription request allowing DCCF relocation initiated by the DCCF (clause 6.2.6.3.Y) or DCCF relocation initiated by the consumer (6.3.6.3.X),  the consumer may execute DCCF relocation based on internal logic. If the DCCF is Rel-18, it may help the consumer to select a new DCCF.

When data is collected for a group of UEs or any UE and one or more of the UEs require DCCF reselection (e.g. because of UE mobility outside of the area served by the DCCF), the consumer may select one or more Target DCCFs and initiate a new subscription for the UEs. When data is collected for a group of UEs or any UE and one or more of the UEs require DCCF or MFAF Relocation, the DCCF may select one or more Target DCCFs or Target MFAFs and transfer contexts for the UEs to the Target DCCF or Target MFAF. The target DCCF or MFAF subsequently sends notifications to the data consumer for UE(s) whose contexts have moved to the target DCCF or MFAF.

* * * Second Change (all new text) * * *

6.2.6.3.X	DCCF (re-)selection initiated by consumer

The procedure depicted in Figure 6.2.6.3.X-1 is used by a data consumer (e.g. NWDAF or central DCCF) to obtain UE(s) data, be notified by the DCCF when the DCCF can no longer serve the UE(s) and reselect the DCCF.

Figure 6.2.6.3.X-1: Procedure for DCCF relocation initiated by consumer

0.	The data consumer subscribes to source DCCF. The data consumer may indicate in the subscription request with an indicator that the DCCF is allowed to execute the relocation procedure.

NOTE:	If the source DCCF or target DCCF does not support relocation, the consumer may execute DCCF relocation based on internal logic.

1.	Source DCCF may notify the data consumer that it cannot serve the subscription anymore, e.g., when location of UE(s) falls outside the serving area of the DCCF. A cause code is added with the notification (e.g., UE(s) moved outside DCCF serving area). The DCCF may send pending data to the data consumer.

2.	The data consumer for the DCCF determines to select a new instance of DCCF. The data consumer discovers and selects the target DCCF as described in clause 6.3.19 of TS 23.501 [2]. The data consumer may perform the DCCF selection due to internal triggers, notification of a  UE mobility event or by receiving the notification from the source DCCF in step 1.

3.	The data consumer sends a subscription request to the target DCCF using Ndccf_DataManagement_Subscribe request.

4.	The data consumer may unsubscribe from the source DCCF.

5.	Target DCCF may subscribe to relevant data source(s), if not yet subscribed.

* * * Third Change (all new text) * * *

6.2.6.3.Y	DCCF and MFAF relocation initiated by DCCF

The procedure depicted in Figure 6.2.6.3.Y-1 is used by a data consumer (e.g. NWDAF or central DCCF) to obtain UE(s) data and to support DCCF and MFAF reselection when the DCCF or MFAF can no longer serve the UE(s).

Figure 6.2.6.3.Y-1: Procedure for DCCF relocation initiated by DCCF

- The data consumer subscribes to source DCCF. The data consumer may indicate in the subscription request with an indicator that the DCCF may execute the relocation procedure.

NOTE:	If the source DCCF or target DCCF does not support relocation, the consumer may execute DCCF relocation based on internal logic.

1.	Source DCCF subscribes UE mobility events from AMF. The UE ID is provided by the data consumer in step 0.

2.	If UE(s) moves out of the service area of the source DCCF, source DCCF determines UE(s) DCCF subscription context to be transferred to target DCCF(s), e.g., triggered by a UE mobility event notification from AMF. If UE(s) moves out of the service area of the source MFAF, source DCCF determines UE(s) MFAF data subscription to be transferred to target MFAF(s).

3.	Source DCCF may query the NRF to discover and select the target DCCF(s) and/or MFAF(s), e.g. based on the UE location information received from AMF.

Conditional on MFAF transfer:

4.	Source DCCF uses the Nmfaf_3daDataManagement service to request transfer of UE(s) MFAF data subscription context to the target MFAF

5.	Target MFAF retrieves MFAF subscription context from Source MFAF

6. 	Target MFAF accepts the data subscription(s) context transfer

7.	Target MFAF responds to the DCCF indicating the transfer is complete. The response may contain a Transaction Reference ID from the Target MFAF.

Conditional on DCCF transfer

8.	Source DCCF using Ndccf_DataManagement_Transfer Request service operation to transfer of UE(s) data subscription context to the target DCCF(s)

9.	Target DCCF(s) accepts the data subscription(s) context transfer.

10.	If an MFAF is being used, the Target DCCF uses the Nmfaf_3daDataManagement_Configure service to configure the target MFAF.

11.	Target DCCF confirms UE(s) data subscription context transfer to the source DCCF. The confirmation includes the Subscription Correlation ID used by the Target DCCF.

For DCCF or MFAF transfer:

12.	[Optional] Target DCCF subscribes to the relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the data subscription context and repeat step 1 to subscribes for UE mobility events from AMF.

NOTE 2: for MFAF context transfer without DCCF context transfer, the Target DCCF is also the Source DCCF.

13.	Source DCCF informs the data consumer about the successful UE DCCF or MFAF data subscription context transfer using a Ndccf_DataManagement_Notify message. The notification may contain a Subscription Correlation ID provided by the target DCCF.

14.	[Optional] Source DCCF unsubscribes with the data source(s) that are no longer needed for the remaining UE(s) data subscriptions.

NOTE 3:	At this point, UE(s) data subscription transfer is deemed completed.

15-17.	Target DCCF or MFAF collects data from the data source(s) and notifies the data consumer using a Ndccf_DataManagement_Notify message.

* * *End of Changes * * *