SA WG2 Meeting #155	S2-2302971

February 20 – 24, 2023, Athens, Greece

Source:	Ericsson

Title:	Aspects related to support of limited AoS slices not matching deployed TAs

Document for:	Agreement

Agenda Item:	9.11.2

Work Item / Release:	eNS_Ph3 / Rel-18

Abstract of the contribution: This contribution discusses aspects related to support of limited AoS slices not matching deployed TAs and proposes way forward for how to handle the areas.

# Discussion

GST attribute Area of service can be seen as corresponding to coverageArea.

The meaning of the two are as follows:

GSMA NG.116 v7.0:

Area of service

This attribute specifies the area where the UEs can access a particular network slice.

Therefore, the attribute specifies the list of the countries where the service will be provided. The list is specific to NSPs and their roaming agreements.

In case the list comprises more than one entry, roaming agreements between the HPMN and the VPMNs are required.

As a part of region specification, requirements for altitude can be provided. Altitude is needed for airborne devices communication where the devices can be from simple UEs or Uncrewed Aerial Systems (UAS) at lower aviation height and up to helicopters and small aeroplanes at higher aviation height. The values for the height may be impacted by the regional regulation and other aspects limiting the use of the full range of the indicated values in practice.

3GPP TS 28.541:

| coverageArea | An attribute specifies the coverage area of the network slice, i.e. the geographic region where a 3GPP communication service is accessible, see Table 7.1-1 of TS 22.261 [28]) and NG.116 [50]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| --- | --- | --- |
| coverageAreaTAList | An attribute specifies a list of Tracking Areas for the network slice . allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. | type: Integer multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None allowedValues: N/A isNullable: False |

Observation 1: Both area of service (NG.116) and coverageArea (28.541) specifies an area where the service is accessible and the service will be provided, but coverage area is defined with TA granularity while AoS is not restricted to any area granularity.

As per the TR 23.700-41 we have the following conclusion related to the area:

“Reconfiguration of TAs while keeping the uniform support of S-NSSAIs in cells within the TA unchanged, but if an operator do not want to change the TA borders, the operator configures the cells of a TA that are outside AoS to have no or limited resources using existing NG-RAN OAM configuration.”

The benefit of allocating some/limited resources to some cells outside the AoS can e.g. be to avoid the UE fully losing UP of PDU Sessions of the slice when the UE exits shortly the AoS.

Observation 2: The cells outside the AoS can be allocated some resources.

Figure 1: An example TA.   Dashed cells: AoS for factory slice. (Micro cells at separate freq.)

In the example in Figure 1, the operator has a requirement to fulfil the services for an factory in the coverage area of the dashed micro cells, e.g. the microcells corresponds to where operator have fully allocated resources to all the cells to fulfil the requirements for the network slice. This corresponds to the Area of Service, and this is where the KPI’s of the slice will be met.

In this example, there is a coverage hole that is not covered by the micro cells. How the operator treats that hole, depends on policy, KPIs and charging model.

In this example, the factory owner is only paying for the provided micro cells. For the operator it may be important that the user will notice degradation outside the AoS, since the factory owner does not pay for another microcell covering the hole and extending the AoS to cover the whole enterprise area.

However, if there are many UEs passing in and out of the hole, and the UP is taken down every time, this will correspond to excessive signalling. For legacy UEs possible also the PDU session would be taken down, and the slice removed from the Allowed NSSAI and added to the Rejected NSSAI (or even removed from the Configured NSSAI), every time the UE enters the coverage hole and is served by the macro cell, so that would cause is even more signalling.

The middle macro cell is therefore configured with a small amount of resources for the slice. The resources are sufficient to keep the UP, but QoS requirements will not be fulfilled, and any GBR flows may be closed.

The other macro cells are configured with no resources for the slice, and there will be no UP for PDU sessions on the slice.

Another possible policy could be that the UEs on the slice also may have access to the slice services in the middle macro cell, in case there is capacity left. In that case, the slice would be configured to have access to shared resources in the middle macro cell, with low priority, so that the UE may be served with acceptable QoS at low load. Also in this case, it is not required that the KPI’s are fulfilled while outside the AoS.

Observation 3: There are scenarios where the operator may want to allocate more than 0 resources for a slice in some cells outside the AoS.

Observation 4: If an area where cells allocated more than zero resources but not enough for meeting the KPIs of the network slice is used, it is not obvious whether that area needs to be captured separately in GSMA and SA5 specs as it is likely not to be part of the AoS.

Proposal 1: AoS is used to describe the area where the KPI’s of the slice are required to be fulfilled.

It is obvious that there is a need to monitor KPIs for the network slice according to the area where the customer defined its requirements and that would correspond to the dashed microcells in the figure 1.

Proposals been submitted at multiple previous SA2 meetings where the SMF subscribes to the AoI corresponding to the AoS and indicates in the reports to CHF whether the UE is in or out of that area. Such info will enable the operator to monitor whether KPIs related to user data are met.

Proposal 2: Define that SMF reports to CHF whether the UE is in or out of the area where the network slice is to fulfil the KPI requirements.

If there is a need for a term defining where more than zero resources is allocated for a slice, another term than AoS should be used until SA2 work is stable and seek feedback from GSMA and SA5 on the mapping to their terminologies and whether extensions are needed. One way can be to use Slice Area of Availability (SaoA) for the coverage area of cells that have non-zero resources allocated for the slice.

# Proposal

Observation 1: Both area of service (NG.116) and coverageArea (28.541) specifies an area where the service is accessible and the service will be provided, but coverage area is defined with TA granularity while AoS is not restricted to any area granularity.

Observation 2: The cells outside the AoS can be allocated some resources.

Observation 3: There are scenarios where the operator may want to allocate more than 0 resources for a slice in some cells outside the AoS.

Observation 4: If an area where cells allocated more than zero resources but not enough for meeting the KPIs of the network slice is used, it is not obvious whether that area needs to be captured separately in GSMA and SA5 specs as it is likely not to be part of the AoS.

Proposal 1: AoS is used to describe the area where the KPI’s of the slice are required to be fulfilled.

Proposal 2: Define that SMF reports to CHF whether the UE is in or out of the area where the network slice is to fulfil the KPI requirements.