3GPP TSG-WG SA2 Meeting #155 	S2-2303152

Athens, Greece, February 20 – 24, 2023

Source:	Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon

Title:	Defining GMLC, LMF and NEF in Ranging and Sidelink Positioning

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This pCR is to identify and add the role of GMLC, NEF, and LMF in Ranging and Sidelink positioning in high level view.

# 1. Introduction/Discussion

It is proposed to specify GMLC and NEF and LMF be enhanced to support the (MT-LR) session of Ranging and Sidelink operations.

Otherwise, without the GMLC role, there could be following consequences:

1.	There could be incompatible call flow with relative position request in Ranging_SL and LCS request.

2.	GMLC features for legacy LCS is not reused and possibly duplicated in other network element(s)

3.	Without the LMF resolving UE_ID, LMF can not idetify UEs inolved in ranging

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change * * * *

### 4.3.7	NEF

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the NEF supports the following:

-	To enable AFs to provide service specific information to the 3GPP network, the NEF supports additional service parameters for Ranging/SL Positioning policy which can use the description if the clause 6.2.5 in TS 23.304 [7].

-	Interacting with GMLC for Ranging/SL Positioning, e.g., providing additional service parameters in the request to a GMLC to perform MT-LR.

-	Interacting with AFs and UEs for Ranging/SL Positioning procedures to perform MT-LR, e.g., providing the Ranging/SL Positioning result.

# * * * * Second change * * * *

### 4.3.8	LMF

In addition to the functions defined in TS 23.273 [8], the LMF supports the following:

-	The Network assisted SL Positioning;

-	Trigger Ranging/Sidelink positioning.

-	Exchange the Ranging/Sidelink Positioning capability.

-	Exchange the Ranging/Sidelink Positioning assistant data.

-	Exchange Ranging/Sidelink positioning signal measurement data/result.

-	Support of receiving stored Ranging/Sidelink Positioning capability from AMF and support of providing updated Ranging/Sidelink Positioning capability to AMF.

-	Determine the location of the Target UE based on the ranging/SL positioning measurement data or result reported by Target UE.

-	Delivering the Ranging/Sidelink positioning service request/response for the Ranging service exposure to UE.

-	Resolve the UE_ID of the UE involved in Sidelink Positioning

# * * * * Third change * * * *

### 4.3.9	GMLC

In addition to the functions defined in TS 23.273 [8], the GMLC supports the following:

-	enable AFs and NFs to perform MT-LR by accessing GMLC directly with additional service parameters for Ranging/SL Positioning.

-	determine the serving AMF instances for the UEs involved in the Ranging/SL Positioning request and forward the request to respective serving AMF

-	receive response from AMF and event reports from and return Ranging/SL Positioning result to NFs and AFs.

# * * * * End of changes * * * *