3GPP TSG SA WG2 Meeting #155	S2-2303168

# February 20 - 24, 2023, Athens										revision S2-2302228, S2-2301449)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1263 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture of IMS supporting data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, China Mobile, Nokia, Nokia Shanghai Bell, ZTE, Vivo, T-Mobile, Samsung, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the conclusion of FS_NG_RTC, IMS network architecture and procedures need to be enhanced to support data channel services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new annex is proposed to document IMS network enhancement to support data channel services. Changes fron Previous Revision: - Editorial Changes highlighted in yellow - XX.3 new Clause documenting call flows, protocol stack and various IMS Data Channel related considerations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.228 CR#1264 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Annex AA is from CR 1264 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

Annex XX (normative):
Support of Data Channel services in IMS

# XX.1	General

This annex describes IMS architecture enhancements to support data channel services.

Data channels are always established in the context of an IMS MMTel session.

Service based interfaces applicable to data channel services are specified in AA.2.3

The architecture in this annex supports separation of signalling function and media function supporting data channel services. A network function for data channel management signalling, Data Channel Signalling Function (DCSF), is specified. The network function to handle data channel media can be provided in two ways:

-	by enhancing the existing IMS MRF (called MRF) to perform media functions related to DC, which interacts with the IMS AS via enhanced Mr'/Cr interfaces; and

-	by introducing a new network function called Data Channel Media Function (DCMF), which interacts with the IMS AS via the service-based interface DC2.

NOTE:	In this release of the specification the IMS AS supporting data channel services is collocated with the TAS.

Editor’s Note: It is FFS whether and how IMS Data Channel is supported in roaming cases.

# XX.2	Architecture and functions

### XX.2.1	Architecture

Figure XX.2.1-1 shows the data channel architecture when using the service-based media function DCMF.

Figure XX.2.1-1: Architecture option of IMS supporting DC usage with DCMF

Figure XX.2.1-2 shows the data channel architecture when using the IMS MRF.

Figure XX.2.1-2: Architecture option of IMS supporting DC usage with MRF

NOTE 1:	MDC2 may connect different IMS security domains.

NOTE 2:	DC5 reference point is not specified in 3GPP.

NOTE 3:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 4:	DC3 and DC4 reference points are not specified in this release.

NOTE 5:	Enhancements to the N33 reference point for support of data channel services are not specified in this release.

Editor's note:	Enhancements to other interfaces (e.g., N71/Sh, N70/Cx, Gm, Mw, ISC, Iq, Mb) for support of IMS data channel service are FFS.

### XX.2.2	Functional entities

#### XX.2.2.1	Data Channel Signalling Function (DCSF)

The DCSF is the signalling control function that provides data channel control logic. The DCSF is not involved in SIP signalling. The DCSF supports the following functionalities:

-	The DCSF receives event reports from the IMS AS and decides whether data channel service is allowed to be provided during the IMS session;

-	The DCSF manages bootstrap data channel and (if applicable) application data channel resources at the DCMF or MRF via the IMS AS;

-	The DCSF supports HTTP web server functionality to download data channel applications (bootstrapping) via DCMF and/or MRF to the UE based on UE subscription.

-	The DCSF downloads data channel applications from the Data Channel Application Repository;

NOTE 1:	The detailed procedures to download data channel applications to the UE are out of scope of this specification.

-	The DCSF interacts with NEF for data channel capability exposure via N33;

NOTE 2:	Interaction with NEF is not specified in this release of the specification.

-	The DCSF interacts with the DC Application Server for DC resource control via DC4/DC3, and for traffic forwarding via MDC3/MDC2.

NOTE 3:	Interaction with the DC Application Server for resource control is not specified in this release of the specification.

#### XX.2.2.2	Data Channel Application Repository (DCAR)

The Data Channel Application Repository stores data channel applications which are retrieved by the DCSF when required.

#### XX.2.2.3	Data Channel Media Function (DCMF) / MRF

The DCMF and MRF provide the media resource management and forwarding of data channel media traffic. The DCMF and MRF provides the following functionalities:

-	The DCMF and MRF manage the data channel media resources (bootstrap and application data channel resources, if applicable) under the control of the IMS AS;

-	The DCMF and MRF terminate the bootstrap data channel from the UE and forward HTTP traffic between the UE and DSCF via MDC1;

-	The DCMF and MRF may anchor the application data channel in P2P scenarios, if required, and forward application data traffic from/to the UEs;

-	The DCMF and MRF relays traffic on the A2P/P2A application data channel between the UE and the DC Application Server via MDC2.

#### XX.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications;

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the DCMF via DC2 or wit MRF via Mr'/Cr for data channel media resource management;

### XX.2.3	Reference points

The following service-based reference points are defined to support data channel service in IMS:

-	DC1:	Reference point between the DCSF and the IMS AS.

-	DC2:	Reference point between the IMS AS and DCMF.

-	DC3:	Reference point between the DCSF and NEF.

NOTE 1:	DC3 reference point is not specified in this release.

- N72/Sc:	Reference point between the DCSF and HSS.

Editor's note:	It is FFS whether a new reference point between DCSF and HSS is required or N71/Sh reference point can be re-used.

The following service-based reference points are updated to support data channel signalling control in IMS:

- N70/Cx/Dx:	Reference point between the CSCF and HSS.

- N71/Sh:	Reference point between the IMS AS and HSS.

The following reference points are defined for data channel media handling:

-	MDC1:	Reference point for transport of data channel media between data channel media function (either DCMF or MRF) and DCSF.

-	MDC2:	Reference point for transport of data channel media between data channel media function (either DCMF or MRF) and DC Application Server.

-	MDC3:	Reference point for transport of data channel media between DCSF and DC Application Server.

The following reference point is updated to support data channel media handling:

-	Mr'/Cr:	SIP based reference point between IMS AS and MRF.

## XX.3	 IMS Data Channel Service Subscription

Subscribers entitled to use IMS Data Channel shall be allocated in HSS a Data Channel service profile. The Data Channel service profile is forwarded to the IMS AS during a third-Party Registration.

Editor’s Note: The exact nature of the Data Channel Service profile, and how it is used is FFS.

Editor's note:	It is FFS whether the Data Channel service profile is downloaded to the S-CSCF during initial IMS Registration or from the IMS AS via N71/Sh.

### XX.4 	IMS DC Channel Setup

The following principles apply when an IMS Data Channel is established: -

- UE can establish an IMS Data Channel simultaneously while establishing an IMS audio/video session or upgrade an ongoing IMS audio/video session through a re-INVITE to an IMS Data Channel.

Editor’s Note: It is FFS whether it needs to be recommended that a UE initiates the IMS session audio/video session firstbefore updating the IMS session to an IMS Data Channel session.

NOTE X: 	An IMS Data Channel established simultaneously with an IMS audio/video session can have impact to the IMS session setup time.

- The trigger for a UE to initiate a DC establishment request is left for UE implementation.

Editor’s Note: It is FFS whether and how the UE is configured with a service profile to initiate a DC establishment request.

NOTE Y: Other means for configuring the UE with the needed trigger information to initiate an IMS Data Channel are also possible.

- An IMS AS that detects an IMS Data Channel established simultaneously with an IMS audio/video IMS session by a non-subscribed UE proceeds with the establishment of the audio/video IMS session establishment, discarding the Data Channel request.

- The DCSF provides a URL to the DCMF or MRF via the IMS AS during data channel media resource reservation for the bootstrap data channel that enables downloading a subscriber specific graphical user interface to the UE via the Bootstrap Data Channel. Once the DCMF or MRF, acting as HTTP proxy, receive a HTTP GET Request containing the root ("/") URL through the bootstrap data channel, the DCMF or MRF replaces the root ("/") URL in the HTTP GET Request with the subscriber specific URL and forward the request to the DCSF for downloading the subscriber specific graphical user interface to the UE. The UE shall maintain the HTTP session state when interacting with the DCMF.

## XX.5 	Data Channel Media Setup

DCMF and the MRF provides media anchoring when needed for the IMS Bootstrap Data Channel, and the IMS Application Data Channel. To that effect, they support the following capabilities:

- HTTP Proxy:  In this configuration, DCMF/MRF supports terminating DTLS with HTTP traffic being transparent to DCMF/MRF. This mode is deployed both for Bootstrap Data Channel, and HTTP Application Data Channels. Figure XX.5-1 illustrates the protocol stack for the HTTP proxy configuration mode for the Bootstrap Data Channel case.

When used with HTTP Application Data Channel, the downloaded Data Channel application will in this case communicate with the DC Application Server, using basic HTTP, within the same bootstrap SCTP association as in which this application was downloaded. UDP, TCP and SCTP should be supported as the transport layer protocols for MDC2.

Figure XX.5-1: DCMF/MRF “HTTP Proxy” Media Configurations

### UDP Proxy: In this configuration, DCMF/MRF transparently proxies HTTP traffic to its target. This mode is deployed only for Application Data Channels.  Figure XX.5-2 illustrates the protocol stack for the UDP proxy configuration mode for the Application Data Channel case, providing a Person2Application/Application2Person/Person2Person Data Channel Application.

Figure XX.5-2: DCMF/MRF “UDP Proxy” Media Configurations

### XX.6	Data Channel Procedures

### XX.6.1. Bootstrap Setup Signalling procedure

Figure XX.6.1-1depicts a signalling flow diagram for establishing a Bootstrap Data Chanel in a person-to-person use case. The DCMF anchors the Bootstrap Data Channel, and the originating network is offering a Bootstrap Data Channel to the remote peer as well for application download.

In the call flow the two UEs have already established an IMS audio session, and the originating UE is updating the IMS audio/video session to an IMS Data Channel session.

NOTE X: Some SIP signalling not relevant for the procedure, is not shown in the call flow below.

Figure XX.6.1-1: Boostrtap Data Channel set up Signalling Prcoedure

The steps in the call flow are as follows:

- Step 1: UE#1 sends the SIP INVITE request with an initial SDP to AS, through originating network P-CSCF and S-CSCF. The initial SDP contains offers for the Bootstrap Data Channel establishment requirement. In this example procedure, SDP contains both Bootstrap Data Channel offers for originating side and terminating side.

NOTE:	This SIP INVITE can also be a SIP re-INVITE performed after the initial IMS audio session is setup.

- Step 2: IMS AS validates user subscription data to determine whether the Data Channel  call request should be conveyed to DCSF.

If the IMS AS determined, based on the user profile, that Data Channel call request needs to be sent to DCSF, the IMS AS selects a DCSF for this user based on local configuration or discovery and selection of a DCSF instance via NRF.

If the IMS AS determined, based on the user profile, that the Data Channel call request need not sent to DCSF, or DCSF decides that DC request is not allowed, the IMS AS proceeds with normal IMS procedures to setup the MMTel session without performing Data Channel  bootstrapping.

-	Step 3: IMS AS notifies the DCSF of the call event sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, CallingID, CalledID, 
SessionCase, SessionID, MediaInfoList) Notify request to the DCSF .

-	Step 4: After receiving the DC control request, the DCSF determines the policy about how to process the Bootstrap Data Channel establishment requirement based on the related parameters in the Data Channel control request and/or operator strategy.

-	Step 5: Since the SessionEstablishmentRequestEvent indicates that served user is offered local bootstrap media, DCSF, based on its policies prepares originating side MDC1 media information, as well as the terminating side MDC1 remote bootstrap media (targeting remote UE).

-	Step 6: DCSF invokes the Nimsas_MediaControl_MediaInstruction (SessionID, MediaInstructionSet) operation based on its policies instructing the IMS AS to connect DCMF both for originating and terminating side.

-	Step 7: The IMS AS selects a DCMF based on local configuration or discovers and selects a DCMF instance or MRF supporting DC media function via NRF.

-	Step 8:	 IMS AS invokes DCMF service to instruct DCMF to allocate a Mb resource to terminate the media descriptor specified by the DCSF for the originating side. . For IMS AS to stand alone DCMF, DC1 is used to reserve data channel media resources reserved based on DCSF media information created in step 5. If MRF is used, IMS AS uses Mr'/Cr to the MRF to reserve data channel media resources.

-	Step 9: IMS AS invokes DCMF service to instruct DCMF to allocate a Mb resource to terminate the media specified by the DCSF for the terminating side. For IMS AS to stand alone DCMF, DC2 is used to reserve data channel media resources based on DCSF received in step 5. If MRF is used, IMS AS uses Mr'/Cr to the MRF to reserve data channel media resources.

NOTE Z:	The DCMF media resource allocation could be done with one or multiple service invocations.

-	Step 10: IMS AS responds to the MediaInstruction request received in step 6. The response may include the atomic success of operation received by the IMS AS and includes also media resource information.

-	Step 11: The DCSF stores the media resource information and responds to the Notify Request received in step 3.

-	Steps 12-14: IMS AS sends the INVITE which includes the updated SDP offer adding media information of DCMF or MRF via the  originating S-CSCF to remote network side and UE#2. The SDP offer for bootstrap data channel to UE#2 will in this case be included.

UE#2 and terminating network returns an 18X response with SDP answer to bootstrap DC to originating network. If the terminating network decides to establish another bootstrap DC to UE#1 and UE#2, the corresponding data channel media description is also included in SDP answer.

-	Steps 24-25: UE#2 and terminating network returns a 200 OK response.

-	Step 26: IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, SessionID, MediaInfoList)  to DCSF.

-	Step 27: The DCSF responds to the notification.

-	Steps 28-30: 200 OK forwarded to UE#1.

-	Steps 31~32:	The bootstrap data channels have been established between originating DCMF or MRF and UE#1/UE#2. The UEs send application request messages to DCMF or MRF via bootstrap data channel with its data channel capabilities. The DCMF or MRF forwards the message to received media point of DCSF. The DCSF provides the proper data channel applications to UE#1 and UE#2 based on their data channel capabilities through DCMF or MRF.

The Bootstrap Data hannels have been established between terminating DCMF or MRF and UE#1/UE#2. The data channel application is requested and downloaded to UE#1 and UE#2 from terminating DCSF

### XX.6.2. Application Data Channel Setup Signalling Procedure

### XX.6.2.1 Person to Person (P2P) Application Data Channel Setup

Figure XX.6.2.1-1 depicts a signalling flow diagram for establishing an Application Data Chanel in a person-to-person use case. The DCMF is not used to anchor the Application data channel.

In the call flow the UEs have already established an IMS audio session, and the originating  UE is updating the IMS audio/video session to an IMS Data Channel session.

Figure XX.6.2.1-1: Person-to-Person Application Data Channel set up Signalling Prcoedure

The steps in the call flow are as follows:

-	Step 0:	  IMS session and bootstrap data channel have been established. Data channel applications have been downloaded to UE#1 and UE#2.

-	Step 1: UE#1 sends the SIP reINVITE request with an updated SDP to IMS AS, through originating network P-CSCF and S-CSCF. The updated SDP contains the Bootstrap data channel information, as well as the Application Data Channel establishment requirement per TS 26.114 [X].

-	Step 2: IMS AS validates user subscription data to determine whether the media change request should be triggered to DCSF.

-	Step 3: IMS AS notifies the DCSF, via Nimsas_SessionEventControl_Notify (MediaChangeRequest Event, SessionID, EventDirection, MediaInfoList) of the media update request event.

-	Step 4: After receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment requirement based on the related parameters in the notification and/or operator policy.

-	Step 5: DCSF determines that the added Application Data Channel media of the offer takes UE#2 as target endpoint not requiring anchoring in the local DCMF or MRF. If DCMF or MRF needs to anchor application data channel, DCMF would have used the Nimsas_ MediaControl service.

-	Step 6: DCSF responds to the notification received in step 3.

-	Steps 7-8: IMS AS sends the re-INVITE to originating S-CSCF and then to remote network side and UE#2.

-	Steps 9-11: UE#2 and terminating network returns a 200 OK response with SDP answer for application DC to originating network.

-	Step 12: IMS AS notifies the DCSF of the successful data channel modification.

-	Step 13: DCSF responds to the notification.

-	Steps 14-15: The IMS AS sends 200 OK response to S-CSCF and P-CSCF.

-	Step 16: The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

-	Step 17: P-CSCF returns the 200 OK response to UE#1.

-	Step 18: UE#1 send ACK to the terminating network.

-	Step 19: The application data channel between UE#1 and UE#2 is established, in this example, not anchored in DCMF/ MRF.

#### XX.6.2.2 Person-to-Application (P2A) Application Data Channel Setup

Figure XX.6.2.2-1 depicts a signalling flow diagram for establishing an Application Data Chanel in a person to application  use case.

Figure XX.6.2.2-1: Person-to-Application (P2A) Application Data Channel set up Signalling Prcoedure

The steps in the call flow are as follows:

- Steps 0-3: Steps 0-3 of clause XX.6.2.1 applies.

- Step 4:	 After receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment requirement based on the related parameters in the notification and/or operator policy.

- Step 5: DCSF determines that the added Application Data Channel media of the offer takes DC Application Server as target endpoint requiring anchoring in the local DCMF or MRF.

- Step 6: DCSF invokes Nimsas MediaControl service to instruct IMS AS to terminate the media flow of the originating UE to DCMF. The instruction also includes information to be consumed by the DCMF that the data channel media shall be relayed via the MDC2 interface.

- Step 7: IMS AS invokes DCMF service to instruct DCMF on Data Channel establishment and data channel media resource reservation based on the DC media information received from DCSF. For IMS AS to stand alone DCMF, DC2 is used to reserve data channel media resources. If MRF is used, IMS AS uses Mr'/Cr to enhance MRF to reserve data channel media resources.

- Step 8: IMS AS notifies the MediaControl instruction control response to DCSF.

- Step 9:	The DCSF stores the media resource information and sends a P2A connection establishment request (including the MDC2 SDP offer received from DCMF) to the DC Application Server via DC3/DC4.

- Step 10: DC Application Server accepts the P2A communication returning an MDC2 SDP answer and is prepared for UE#1 traffic through MDC2.

NOTE X: Details on how DCSF communicates with the DC Application Server is out of scope of this Release.

- Step 11: DCSF requests IMS AS to update the DCMF resource with MDC2 media endpoint information of DC Application Server.

- Step 12: IMS AS updates the DCMF resource.

- Step 13: IMS AS notifies the MediaControl instruction control response to DCSF.

- Step 14: DCSF replies to the Nimsas notification request.

- Steps15-16: IMS AS sends the re-INVITE to remote network side and UE#2, via the originating S-CSCF, and which does not include the SDP.

- Steps 17-19: UE#2 and terminating network returns a 200 OK response with SDP answer for audio/video.

- Step 20: IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

- Step 21: DCSF replies to the notification.

- Steps 22-23: The IMS AS includes SDP answer for application data channels to UE#1 in 200 OK response and sends 200 OK response to S-CSCF and P-CSCF.

- Step 24: The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

- Step 25: CSCF returns the 200 OK response to UE#1.

- Step 26: UE#1 send ACK to the terminating network.

- Step 27: The application data channel between UE#1 and DC Application Server is established via DCMF or MRF. DCMF or MRF forwards data channel traffic between UE#1 and DC Application Server based on MDC2 media point information received in step 9 and 12.

### XX.6.3	Person-to-Application and Application-to-Person (P2A2P) Procedure

### Editor’s Note: The relationship between XX.6.3 and XX.6.2 needs further clarification.

This procedure enables originating and terminating UE to establish application data channels for the same application to communicate with the same Data Channel Application Server.

Figure XX.6.3-1: Symmetric Application Data Channel Establishment

0.	IMS session and bootstrap data channels are established. Data channel applications are downloaded to UE#1 and UE#2.

1.	UE#1 sends SIP re-INVITE request with an updated SDP to IMS AS. The updated SDP contains the bootstrap data channel and the application data channel information.

2.	IMS AS validates the data channel media description information and/or user subscription data to determine whether the DCSF needs to be notified.

3.	IMS AS selects and notifies the DCSF about the call event and data channel establishment request.

4.	The DCSF determines how to process the application data channel establishment request based on the parameters in the notification from DCSF and/or operator policies.

5.	DCSF determines that the added application data channel media in the SDP offer requires the DC Application Server is the endpoint for both originating and terminating UE and that the application DC must be anchored at the DCMF or MRF. DCSF communicates with the DC Application Server for DC resource control. Once the application data channel is established, tha DC Application Server will send/receive traffic to/from UE#1 through the MDC2 interface.

NOTE X:	Details on how the DCSF communicates with the DC Application Server are out of scope of Release 18.

6.	DCSF invokes Nimsas MediaControl service to send data channel control request to IMS AS, including information how to relay data channel media via the MDC2 interface.

7.	IMS AS reserves data channel media resources at the DCMF via DC2 or at the MRF via Mr’/Cr based on the DC media information received from DCSF.

8.	IMS AS notifies the DCSF about MediaControl instruction control response.

9.	The DCSF stores the media resource information and establishes MDC1 connections with DCMF or MRF for application DC traffic to UE#1 and replies to the Nimsas notification request.

10-11.	IMS AS sends re-INVITE which include the SDP offer from DCMF or MRF for the application data channel to the originating S-CSCF and then to the remote network and UE#2.

12-15.	UE#2 and terminating network return 200 OK with SDP answer for audio/video and for the application data channel. The terminating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK.

16. IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

17.	DCSF replies to IMS AS.

18-19.	The IMS AS includes SDP answer for application data channel to UE#1 in 200 OK and sends 200 OK to S-CSCF and P-CSCF.

20.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK.

21.	P-CSCF returns the 200 OK to UE#1.

22.	UE#1 sends ACK to the terminating network.

23.	The application data channel between UE#1 and DC Application Server is established via DCMF or MRF. DCMF or MRF forwards data channel traffic between UE#1 and DC Application Server via MDC2.

24.	The application data channel between UE#2 and DC Application Server is established via DCMF or MRF. DCMF or MRF forwards data channel traffic between UE#2 and DC Application Server via MDC2.

### XX.6.4	NF service registration and discovery

If the NRF is used for NF registration and discovery,the DCSF and the DCMF shall register their services at the NRF before providing services to consumers, and existing NRF based mechanism needs to be extended with DC specific profiles according to clauses 4.17.2 and 4.17.3 of TS 23.502 [3].

#### XX.6.4.1	DCSF Registration and Discovery

The DCSF profile may include IMPU ranges for calling identities or called identities of users it serves.

The DCSF can update and deregister in the NRF after registration and the procedures are specified in clauses 4.17.2  and 4.17.3 of TS 23.502 [3].

#### XX.6.4.2	DCMF Registration and Discovery

The DCMF profile may include information to select a local DCMF to minimize transmission delays in the media path, this may include IP address or location.

### XX.6.5	Providing subscriber specific graphical user interface to the UE

According to clause 6.2.10.1 of TS 26.114 [76], the UE can send a HTTP GET Request for the root ("/") URL through the bootstrap data channel which is replied with a data channel application describing the graphical user interface and the logic needed to handle any further data channel usage beyond the bootstrap data channel itself. The graphical user interface can e.g., contain a menu of applications, from which the user can choose one or several. The graphical user interface should be subscriber specific and contain only applications for which the user has subscribed to and is authorized to use. To provide the graphical user interface containing subscriber specific data channel applications via the bootstrap data channel to the UE, the DCSF, when receiving a call event notification including subscriber specific information from the IMS AS, creates the URL identifying the graphical user interface for this subscriber.

NOTE X:	How the DCSF creates the URL is implementation specific.

**** End of Changes ****