3GPP TSG SA WG2 Meeting #155	S2-2302637

# February 20 - 24, 2023, Athens, Greece								(Revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1268 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS DC capability discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, HiSilicon, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusions are agreed for IMS DC capability discovery in the TR 23.700-87: - The network discovers the UE support of DC based on existing mechanism as specified in TS 26.114 [6] and TS 24.229 [8]. - The UE discovers home network support for DC via the Feature-Caps header field in 200 OK during registration, as specified in solution #4. - The discovery of UE capabilities required to use data channel applications is done via application level communication through the bootstrap data channel and is transparent to IMS. TS 23.228 needs to be updated to support mutual IMS DC capability discovery based on the above conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is proposed to document how UE and IMS network discovers the DC capability of each other. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

Annex XX (normative):
Support of Data Channel services in IMS

XX.Y	Procedures

XX.Y.Z	IMS DC capability negotiation

The IMS network and the UE need to mutually negotiate the capability of supporting IMS data channel mutually.

IMS data channel capabilitiy negotiation includes two aspects:

-	The network discovers the data channel capability of the UE.

-	The UE discovers the data channel capability of the network.

When the UE supporting IMS data channel registers on the IMS network, it includes the media feature tag as specified in TS 26.114 [76] in the Contact header field of the initial REGISTER request to allow the home IMS network discovers its IMS data channel capability.

If the IMS network supports IMS data channel, the S-CSCF includes a Feature-Caps header field indicating its data channel capability in the 200 OK response to the initial and any subsequent REGISTER request, which is used by the UE to discover the IMS data channel capability of its home IMS network.

NOTE x:	The UE can receive a Feature-Caps header field indicating its data channel capability in the 200 OK response to a subsequent REGISTER request when the network starts supporting IMS data channel after successful initial registration of the UE.

When the UE supporting IMS data channel initiates an IMS session, it includes the media feature tag as specified in TS 26.114 [76] in the Contact header field of the initial INVITE or a re-INVITE request to remote UE, regardless of data channel media being part of the SDP or not.

The UE shall not include data channel media description in the SDP offer of the initial INVITE request or any subsequent re-INVITE, if the S-CSCF has not included the data channel capability indication in the Feature-Caps header field in the 200 OK response either to the REGISTER or subsequent REGISTER request.

**** End of Changes ****