3GPP TSG-WG SA2 Meeting #155  	S2-2303182

Athens, Greece, February 20 – 24, 2023	(revision of S2-2303021)

Source:	Samsung, Interdigitial

Title:	Service exposure to AF and 5GC NF

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This contribution specifies ranging and sidelink positioning service exposure to AF and 5GC NF.

# 1. Introduction/Discussion

This contribution specifies ranging and sidelink positioning service exposure to AF and 5GC NF.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.586, clauses 5.6.2 and 5.6.3.

# * * * * First change (all new text) * * * *

### 5.6.2 Service Exposure to the Application Server

The 5GS Ranging/SL Positioning service may be exposed to the Application server either via direct interaction with the GMLC or via the NEF for an Application server in untrusted domain. Thus, the ranging application server may contact GMLC directly or via NEF to provide a ranging or sidelink positioning operation request to the 5GS.

The service exposure in clause 5.5 of TS 23.273 [8] is reused with enhancements. The request shall include one or multiple target UEs and it may include additional information such as the SL reference UE, the required positioning QoS, etc. The request may indicate with corresponding events if the server requires relative positioning/ranging of target UEs, absolute positioning of target UEs, or both. The request shall include one or multiple target UEs and optionally it may include additional information such as the SL reference UE for relative positioning/ranging, the required positioning QoS, etc. For relative positioning/ranging, at least two UE’s identities are included as attribute in the location service requests and accuracy and latency for distance and/or direction measurements are included in Requested QoS information. The result is returned by the 5GC to the Application server via NEF, if needed. The detailed procedure is specified in clause 6.1.X.

In addition, the LCS architecture specified in TS 23.273 [8] is re-used and the 5GC-MT-LR and 5GC-MO-LR procedures there specified are taken as baseline. LMF is the responsible 5GC NF to forward the Ranging/Sidelink Positioning request to the UE, and one Ranging/SL Positioning capable LMF is selected by the AMF for a Ranging/Sidelink Positioning session. The AMF serves either the Target UE or the SL Reference UE or both. In case the selected LMF does not have the context information for all involved UEs, a similar context transfer as specified in TS 23.273 [11] can be performed.

The Ranging/SL Positioning result calculation is performed by LMF or UEs. When absolute positioning is requested, LMF and target UE may decide whether SL positioning or Uu positioning needs to be performed. LMF may combine Ranging/Sidelink Positioning results and Uu positioning results before exposing the results to the Application server.

### 5.6.3 Service exposure to 5GC NF

The 5GS Ranging/SL Positioning service request may be initiated by a 5GC NF. In this case, the 5GC NF initially interacts with GMLC and the LCS architecture specified in TS 23.273 [8] may also be re-used as described in clause 5.6.2.

Editor’s note: Further details of service exposure to 5GC NF are FFS.

# * * * * End of changes * * * *