3GPP TSG-WG SA2 Meeting #155 	S2-2303224

February 20 – 24, 2023, Athens, Greece	(revision of S2-2303179, S2-2303006)

Source:	Philips International B.V, Interdigital

Title:	High Level Description for Located UE Discovery and Selection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes a high level description for Located UE Discovery & Selection for TS 23.586.

# 1. Introduction/Discussion

This paper proposes a high level description for Located UE Discovery & Selection for TS 23.586.

# 2. Text Proposal

It is proposed to capture the following new clause 5.X.Y (e.g. 5.2.2 as in draft table of contents) for TS 23.586.

# * * * * Start of changes * * * *

## 5.X.Y	Located UE Discovery & Selection

The discovery of Located UEs follows the same principles as specified in clause 5.2.1. The UE can indicate its role “Located UE” in its list of supported roles during discovery, if it is authorized to be a Located UE in a given PLMN as per the Authorization and Provisioning for Ranging/SL positioning service as specified in clause 5.1.

Editor’s Note: Privacy aspects of sharing the location of a Located UE (e.g. to any Target UE) and exposing information about such privacy aspects during discovery (e.g. to facilitate Located UE selection by a Target UE) needs to be aligned with SA WG3.

A Target UE may discover and select one or more Located UEs (and other Reference UEs) to be used in the Ranging/SL positioning procedures as specified in clauses 5.3 through 5.5. In this release of the specification, for network assisted ranging and SL positioning, the Target UE shall discover and select Located UEs that are in the same serving PLMN of the Target UE.

Editor’s Note: other selection criteria are FFS.

Editor’s Note: Whether LMF will select Located UEs for Ranging/SL positioning procedure will be decided by RAN.

Editor’s Note: It is FFS if the LMF can provide candidate list of Located UE to participate in the network assisted SL positioning for the target UE, e.g.to perform targeted discovery.

Editor’s Note: It is FFS whether an indication of whether a Located UE is static/mobile needs to be provided during discovery in line with SA1 requirements in TS 22.261, e.g. to facilitate Located UE selection in various coverage scenarios.

NOTE: 	The role of being “Located UE” is dynamic and can change over time, in particular if the Located UE is moving. Hence, the discovery results need to be refreshed if there is a (significant) delay between discovery and initiating of a ranging procedure with a discovered Located UE. How often this is done is up to UE implementation.

Editor’s Note: 	The criteria to determine if the location of a UE is sufficiently stable and accurate in order for the UE to announce itself as a “Located UE” needs to be aligned with RAN WGs.

# * * * * End of changes * * * *