3GPP TSG-SA WG2 Meeting #155	S2-2303359

# February 20 – 24, 2023, Athens, Greece									revision of S2-2302739

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0221 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | One Remote UE for emergency handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In LS S2-2205433, SA2 has the following assumption in Q4: SA2 assumes that a Relay UE may provide emergency service access to only one Remote UE at any time It can be seen that SA1 doesn’t object this assumption in their response, so SA2 can persue this assumption into SA2 normative work. But this assumption can be only applied to L3 relay, as L2 relay has no impacts with mutiple L2 remote UEs accessed at same time. From the technical perspective, the 5GC cannot establish multiple dedicated emergency voice QoS flows for the L3 relay UE if multiple L3 remote UEs accessing to the L3 relay UE for emergency voice service in parallel. To reduce further modification for the 5GC, we advice that only one L3 Remote UE is allowed to be accessed for emergency service at any time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specify that a 5G ProSe Layer-3 UE-to-Network Relay shall guarantee that only one 5G ProSe Layer-3 Remote UE is allowed to be accessed for emergency service at any time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The L3 relay UE can not serve multiple L3 remote UE for relaying emergency service in parallel. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

5.4.x	Support of Emergency Service for UE-to-Network Relaying

A 5G ProSe Layer-3 UE-to-Network Relay shall guarantee that only one 5G ProSe Layer-3 Remote UE is allowed to be accessed for emergency service at any time. Once a 5G ProSe Layer-3 Remote UE has been accessed for emergency service, the 5G ProSe Layer-3 UE-to-Network Relay shall stop the relay discovery procedure and reject or ingnore any other received connection request.

* * * * End of Changes * * * *