SA WG2 Meeting #S2-155	S2-2303639

# Athens, Greece, Feb 20 – 24, 2023,                                                          was S2-2301373

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3864 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Policy control enhancements to support multi-modal flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile,Nokia,Nokia Shanghai Bell, Samsung, Xiaomi, Ericsson, InterDigital Inc., China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The conclusions of KI#1 and KI#2 for FS_XRM are described in TR 23.700-60 in clauses 8.1 and 8.2. To include these aspects in TS 23.501, this paper proposes to introduce a new clause for the general description of policy enhancement to support delivery of multi-modality services, and some related clause update. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A general description about policy enhancement to support multi-modal services. Related clause update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-modal services delivery cannot be supported using PCC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes, all the texts are new >>>

5.37.X 	Policy control enhancements to support multi-modal services

Multi-modal services consist of several data flows (named as multi-modal flows) that related to each other and may come from different sources. Each data flow (single-modal data) may be seen as one type of data (for example audio, video, positioning, haptic data) associated with the same communication service. Data flows that comprise a multi-modal service may come from a single UE, either via a single device or via multiple devices connected to the single UE that can access the 5GS, or from multiple UEs.

For the single UE case, the expected multi-modal application behaviour is that those data flows that are closely related and require strong application coordination for correct delivery of the multi-modal application data, are transmitted in a single PDU session.

-	The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, service requirements, for each media that comprise the multi-modal service, a Multi-modal Service ID and QoS monitoring requirements for multiple IP data flows associated to a multi-modal service.

-	The AF may provide to the PCF a Multi-modal Service ID, as an explicit indication that the application traffics are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and apply QoS policies for data flows that are part of a specific multi-modal application.

-	The AF may provide QoS monitoring requirement for data flows associated to a multi-modal service to the PCF (either directly or via NEF) with in a certain period. The PCF generates the authorized QoS Monitoring policy for these service data flows.

NOTE: In order to strat the monitoring of the packet delay results for data flows associated to a multi-modal service with in a certain period,  the PCF needs to receive the data from AF accordingly.

-	The multi-modal services are carried in single PDU session for one UE.

In addition to the features that are provided for the case that the flows are associated with a single UE, the following features are provided for the case where the flows are associated with more than one UE:

-	The same DNN/S-NSSAI combination for the multi-modalservice should be selected among the multiple UEs. The URSP Rule evaluation framework is used to ensure that the same DNN/S-NSSAI is selected.

-	The AF may allocate the same Multi-modal Service ID to all UE PDU sessions that componse a multi-modal service. The PCF may take this information into account (e.g. to allocate a specific QoS profile) when processing each PDU session independently.  The data flows that contribute to the service experience, but may still be valid stand-alone, may be transmitted over separate PDU sessions from multipe UEs.

-	In case multiple PCFs are involved, the PCF(s) take policy decisions according to the input provided by the AF. There is no support for policy coordination among the multiple PCF(s). Policy decisions are taken by each PCF separately on a per PDU session basis.

<<< End of changes >>>