3GPP TSG-SA2 Meeting #155	S2-2303651

# Athens, Greece, February 20 – 24, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3893 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys - QoS filtering criteria in assistance to UE member selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Describe the procedures when the AF indicates a QoS filtering criteria for UE member selection |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added procedures for indicating the QoS filtering criteria for UE member selection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide a QoS filtering criteria assistance to UE member selection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.X.2, 4.15.X.3, 4.15.X.3.1, 4.15.X.3.2, 4.15.X.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.502 CR 3674 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_AIMLsys. 4.15.X. (this clause is introduced by CR 3674), |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.15.X.2	UE Member Filtering Criteria for 5GS assistance to UE member selection

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the selected UEs match or exceed the QoS of the filtering criteria |  | 4.15.X.3 |

* * * Next Change * * *

#### 4.15.X.3	Specific procedure for QoS UE member filtering criteria

#### 4.15.X.3.1	General

An AF may invoke Nnef_UEMemberSelectionAssistance_Subscribe service operation with a QoS filtering criteria for receiving a list of UEs that match or exceed such criteria.

In addition to the mandatory parameters, the AF also includes in the request:

- QoS filtering criteria.

- Optionally, an Area of Interest: location area of the candidate UEs.

- Optional time windows for selecting the candidate UEs: start time and stop time.

The QoS filtering criteria includes one or more:

- Uplink packet delay.

- Downlink packet delay.

- Round trip packet delay.

If one or more time windows are included in the request, the AF is expressing its intention to obtain the list of UEs that have been (statistics) or will be (predictions) matching or exceeding the QoS filtering criteria and have been or will be optionally located in the AoI, for the duration of the subscription. This is further described in clause 4.15.X.3.2.

If no time window is included in the request, the AF is expressing its intention to obtain the current list of UEs that match or exceed the QoS filtering criteria and are optionally located in the AoI in real time, for the duration of the subscription. This is further described in clause 4.15.X.3.3.

##### 4.15.X.3.2	UE Member Assistance with QoS filtering criteria requiring NWDAF interaction

If a time window is present in the Nnef_UEMemberSelectionAsistance_Subscribe request of step 1, NEF contacts NWDAF for determining the list of candidate UEs.

Figure 4.15.X.3-2: 5GC assistance to UE member selection for QoS filtering criterial requiring NWDAF interaction

1.	AF subscribes the UE member selection assistance functionality by sending Nnef_ UEMemberSelectionAssistance_subscribe request including the Application ID or Flow Description(s), a list of target UE(s), a ‘QoS’ filtering criteria, and one or more time window(s).

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executed based on the UE member QoS filtering criteria provided by the AF, e.g., analytics.

3.	If an Application ID is present in the request from the AF, the NEF derives the S-NSSAI and DNN to which this Application ID have access. Due to the presence of a time window in the request from the AF, NEF discovers the NWDAFs that are deployed matching the UE member QoS filtering criteria, including: target Analytics IDs, NWDAF service area (matching the area of interest), S-NSSAI, etc.

Editor’s Note: The actual Analytics IDs sent in this request are FFS.

4.	For each selected NWDAF, NEF sends either an Nnwdaf_AnalyticsSubscription_Subscribe request or an Nnwdaf_AnalyticsInfo_Request request (Analytics ID, S-NSSAI, DNN, AoI, time window(s)), also including, if applicable, the target UEs and the Area of Interest (if available).

5.	If required, NWDAF collects data as required by the Analytics ID, as per procedures specified in TS 23.288 [50].

6.	NWDAF sends an approviate Nnwdaf_AnalyticsSubscription_Notify request or Nnwdaf_AnalyticsInfo_Request response including an Analytics report.

7.	Based on the Analytics report received from NWDAF, NEF consolidates the results received from NWDAF and derives the list(s) of candidate UE(s) and additional information which fulfill the UE member QoS filtering criteria received in the AF request.

8.	NEF sends a Nnef_ UEMemberSelectionAssistance_Notify request to the AF including and the list(s) of candidate UE(s) and additional information.

##### 4.15.X.3.3	UE Member Assistance with QoS filtering criteria for real-time QoS Monitoring

If no time window is present in the Nnef_UEMemberSelectionAsistance_Subscribe request of step 1, NEF determines to do real-time QoS monitoring for determining the list of UEs that match or exceeds the QoS filtering criteria in real time.  QoS Monitoring must have been activated in SMF for that QoS Flow before (e.g. by AFSession with required QoS). NsmF_Event_Exposure consumer becomes an additional received of the QoS Monitoring information.

Editor´s Note: For QoS Flows other than QoS Flow associated to the default QoS rule, whether and how QoS monitoring is performed is FFS.

Figure 4.15.X.3-2: 5GC assistance to UE member selection for real-time QoS monitoring

1.	AF subscribes the member selection assistance functionality by sending Nnef_ UEMemberSelectionAssistance_subscribe request including the Application Identity, Flow Description(s) and the ‘QoS’ filtering criteria.

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executed based on the QoS UE member filtering criteria provided by the AF, e.g., QoS Monitoring events.

3.	If an Application ID is present in the request from the AF, the NEF derives the S-NSSAI and DNN to which this Application ID have access. Due to the absence of a time window in the request from the AF, NEF discovers querying UDM and NRF the SMFs that are deployed in the Area of Interest. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination (this S-NSSAI and DNN combination has been derived from the Application Id or AF Id).

4.	For each target SMF, NEF sends an Nsmf_EventExposure_Subscribe request (Event: ‘QoS monitoring’, target UEs, Application ID or Flow Description(s), S-NSSAI, DNN, …).

5.	The SMF many need to interact with UPF for requesting the QoS monitoring for certain flows.

6.	SMF sends an Nsmf_EventExposure_Notify request including an Event Exposure notification.

7.	Based on the Analytics report received from the SMFs, NEF consolidates the results received from the SMFs and derives the list(s) of candidate UE(s) and additional information which fulfill the QoS filtering criteria in the AF request.

8.	NEF sends a Nnef_ UEMemberSelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

* * * End of Changes * * *