3GPP TSG-SA2 Meeting #155	S2-2303697

# Athens,Greece, 20 – 24 February, 2023								       revision of S2-2303693

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3897 | rev | 10 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN communication configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in TR 23.700-88, PIN traffic QoS and routing differentiation need to be supported by 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe session management enhancement for the support of PIN service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PIN support is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 5.X.c.1, new 5.X.c.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | For reference to 5.X, see 23.501 CR3854 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Separate PDU Session Establishment and PDU Session Modification into different sub-clauses |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 5.X	Support for Personal IoT Networks

### 5.X.c	Session management enhancement for PIN service support

#### 5.X.c.1	PDU Session Establishment for PIN

When a PDU Session associated with a PIN is established by UE with PEGC, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The UE with PEGC may use IP address allocation methods as specified in clause 5.8.2.2 (e.g. IPv6 Prefix Delegation feature).

One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN if the PIN traffic is via PEGC/5GC. One PIN may be served by more than one PDU sessions in the PEGC.

Editor's note:	How and wether to handle the case where PINs share a PDU session and local switching is FFS.

Editor's note:	One PIN served by more than one PDU sessions in PEGC is FFS.

Editor's note:	The handling of the PEMC in 5GC in relation with PIN is FFS.

#### 5.X.c.2	Session management related policy control

For PIN traffic via PEGC/5GC with PDU session, the 5GC supports the policy control. The policy control is based on session management procedures as specified in TS 23.502 [3] and TS 23.503 [45].

A PEGC or an AF may provide QoS flow prameters to 5GC.  An AF supports PIN service may also influence traffic routing for PDU sessions for PIN traffic. The PIN traffic can be categorised into following types:

-	Between two PINEs, which is via 5G core network when the two PINEs connect to different PEGCs;

-	Between PINE and PEMC via a PEGC and 5G core network;

-	Between PINE and DN via a PEGC and 5G core network;

-	Between PEGC and DN via 5G core network.

Editor's note:	The handling of PIN deletion/deactivation/activation is FFS.

# * * * * End of change * * * *