3GPP TSG-WG SA2 Meeting #155 	S2-2303722

# 20 - 24 February, 2023, Athens, Greece	(revisions of S2-2303676)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3841 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on registration area impacted by the requeted or subscribed coverage area. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Address the EN on RA scope and TAs inclusion from 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the AF request ASTI time sync procedure and Subscription-based ATSI time sync procedure to reflect the impact to Registration area. from S2-2302392: AMF restrains/expands TAs that it will use for reporting UE’s Presence in an Area of Interest while the RA remains unchanged. In order to achieve this, the TSCTSF invokes the subscription to UE’s Presence in AoI with AMF using TAs from the TA list constituting the Spatial Validity Condition, but the AMF decides, if required, which TAs to add/remove based on the RA. Applying the same changes to (g)PTP case, see clause 4.15.9.3.2, and other related changes in clauses 4.15.9.3.1 and 5.2.2.3.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear the AMF behavoir when it receives the time sync coverage area. There is fig and description mis-alignment in clause 4.28.2.1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.9.3.1, 4.15.9.3.2, 4.15.9.4, 5.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

##### 4.15.9.3.1	General

This procedure can be used by the AF to activate, modify or deactivate the (g)PTP instances in 5GS.

The AF may activate the time synchronization service using the Nnef_TimeSynchronization_ConfigCreate service operation. The service operation creates a time synchronization configuration based on the service parameters as indicated in the create request. The AF may update the time synchronization configuration using the Nnef_TimeSynchronization_ConfigUpdate service operation. The AF may deactivate the time synchronization service using the Nnef_TimeSynchronization_ConfigDelete service operation, which deletes the corresponding time synchronization service configuration.

The Nnef_TimeSynchronization_ConfigCreate and Nnef_TimeSynchronization_ConfigUpdate request may contain the parameters as described in Table 4.15.9.3-1.

Table 4.15.9.3-1: Description of Time Synchronization service parameters

| Time Synchronization Parameter | Description |
| --- | --- |
| PTP instance type | Identifies the requested PTP instance type as described in clause 5.27.1.4 of TS 23.501 [2]. |
| Transport protocol | Identifies the requested transport protocol for PTP instance as described in clause 5.27.1.4 of TS 23.501 [2]. This is applicable for IEEE Std 1588 [76] Boundary Clock and Transparent Clock operation. |
| PTP Profile | Identifies the PTP profile for the PTP instance as requested by AF. |
| Grandmaster enabled | Indicates whether the AF requests the PTP instance in 5GS to be able to act as a grandmaster for PTP or gPTP (depending on the requested PTP instance type). This is applicable for IEEE Std 1588 [76] Boundary Clock or IEEE Std 802.1AS [75] operation. [optional] |
| Grandmaster priority | Indicates a priority used as defaultDS.priority1 when generating Announce message when 5GS acts as (g)PTP GM. Applicable only if the Grandmaster enabled = TRUE. If omitted, the default value as described in the PTP Profile is used. [optional] |
| Time Domain | (g)PTP domain of the PTP instance as defined in IEEE Std 1588 [76]. |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the time synchronization service for a PTP instance is active. [optional] |
| Spatial Validity Condition | Indicates a geographical area (e.g., a civic address or shapes) or a TA list in which time synchronization service is enabled (NOTE Z). [optional] |
| Time synchronization error budget | Indicates the time synchronization budget for the time synchronization service (as described in clause 5.27.1.9 of TS 23.501 [2]). [optional] |
| For each PTP port in the PTP instance |  |
| Either UE identity (for a DS-TT port), or "N6 interface" indication | Identifies the UE/DS-TT which the parameters below apply. "N6 interface" indicates that the parameters below apply to the N6 interface. If the "PTP port" needs to be identified, this field refers to the UE identity (GPSI or SUPI). If the N6 termination needs to be identified, then this field indicates "N6 interface" flag, instead of SUPI or GPSI. |
| PTP enabled | TRUE/FALSE. This is used to set the portDS.portEnable. If omitted, the default value as described in the PTP Profile is used. [optional] |
| Log Sync Interval | Specifies the mean time interval between successive Sync messages. This is applicable for IEEE Std 1588 [76] Boundary Clock or IEEE Std 802.1AS [75] operation. If omitted, the default value as described in the PTP Profile is used. [optional] |
| Use management settable Log Sync Interval | TRUE/FALSE. This is applicable if the PTP Profile is I IEEE Std 802.1AS [75]. When set to FALSE, the Log Sync Interval is used to set the initialLogSyncInterval as described in IEEE Std 802.1AS [75]. When set to TRUE, the Log Sync Interval is used to set the mgtSettableLogSyncInterval as described in IEEE Std 802.1AS [75]. If omitted, the default value as described in the IEEE Std 802.1AS [75] is used. [optional] |
| Log Announce Interval | Specifies the mean time interval between successive Announce messages. This is applicable for IEEE Std 1588 [76] Boundary Clock or IEEE Std 802.1AS [75] operation. If omitted, the default value as described in the PTP Profile is used. [optional] |
| Use management settable Log Announce Interval | TRUE/FALSE. This is applicable if the PTP Profile is IEEE Std 802.1AS [75]. When set to FALSE, the Log Announce Interval is used to set the initialLogAnnounceInterval as described in IEEE 802.1AS. When set to TRUE, the Log Announce Interval is used to set the mgtSettableLogAnnounceInterval as described in IEEE Std 802.1AS [75]. If omitted, the default value as described in the IEEE Std 802.1AS [75] is used. [optional] |
| NOTE Z: A geographical area requested by an AF includes at least one tracking area (TA), i.e., the granularity is restricted to a TA level in this release of the specifications. |  |

The AF may use Nnef_TimeSynchronization_CapsSubscribe service operation as described in clause 4.15.9.2 to learn the UE capabilities for time synchronization service. The Nnef_TimeSynchronization_CapsNotify service operation indicates the list of UE identities, User-plane Node ID and the Subscription Correlation ID. The AF can use the Subscription Correlation ID and the user-plane node ID received in the Nnef_TimeSynchronization_CapsNotify service operation as a target of the Nnef_TimeSynchronization_ConfigCreate request. The NEF uses the Subscription Correlation ID and user-plane node ID to determine the list of UEs and list of AF-sessions to which the Nnef_TimeSynchronization_ConfigCreate service operation is targeted to.

>>>> Next Change <<<<

##### 4.15.9.3.2	Time synchronization service activation

Figure 4.15.9.3.2-1: Time synchronization service activation

1.	The AF creates a time synchronization service configuration for a PTP instance by invoking Nnef_TimeSynchronization_ConfigCreate service operation. The request includes the parameters as described in Table 4.15.9.3-1. The request contains a Subscription Correlation ID and user-plane node ID as a reference to the target of the UEs and AF-sessions.

The create request creates also a subscription for the changes in the time synchronization service configuration.

2.	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_TimeSynchronization_ConfigCreate service operation with the corresponding TSCTSF, with the parameters as received from the AF.

If the request includes a spatial validity condition and if the AF uses a geographical area as a spatial validity condition, the NEF transforms this information into 3GPP identifiers (e.g. TAI(s)) based on pre-configuration.

The AF that is part of operator's trust domain may invoke the services directly with TSCTSF.

If the request includes a spatial validity condition and if the AF is within the operator's domain, the spatial validity condition shall comprise of a list of TA(s).

NOTE XX:	It is assumed that AFs within the operator’s domain is aware of TAs that can be used to formulate a spatial validity condition for Time Synchronization Coverage Area (see clause 5.27.1.10 of TS 23.501[2]).

3.	TSCTSF checks with the UDM if the UE is allowed to receive the time sync service. The TSCTSF responds with the Ntsctsf_TimeSynchronization_ConfigCreate response. The Ntsctsf_TimeSynchronization_ConfigCreate response includes a PTP instance reference.

4.	The NEF responds with the Nnef_TimeSynchronization_ConfigCreate response, including a reference to the time synchronization service configuration (PTP instance reference).

5-6.	The TSCTSF uses the Subscription Correlation ID and user-plane node ID in Ntsctsf_TimeSynchronization_ConfigCreate to determine the target UEs and corresponding AF-sessions. The TSCTSF uses the parameters (e.g. requested PTP instance type, transport protocol and PTP profile) in the Ntsctsf_TimeSynchronization_ConfigCreate request to determine suitable DS-TT(s) and corresponding AF-sessions among all AF-sessions that are associated with the Subscription Correlation ID and user-plane node ID in the request.

The TSCTSF maintains association between list of suitable AF-sessions, corresponding time synchronization configuration, the PTP instance reference in 5GS, PTP instance references in each involved DS-TT and NW-TT and Subscription Correlation ID and user-plane node ID as given in step 1.

NOTE 1:	The AF-sessions that are not associated with a time synchronization configuration, are available to be selected as suitable AF-sessions in another Ntsctsf_TimeSynchronization_ConfigCreate request.

The TSCTSF uses the procedures described in clause K.2.2 of TS 23.501 [2] to configure and initialize the PTP instance in the DS-TT(s) and NW-TT. The TSCTSF constructs a PMIC to each DS-TT/UE to activate the time synchronization service in DS-TT in respect to the service parameters in the request in step 2. The TSCTSF constructs PMIC(s) and UMIC to NW-TT to activate the time synchronization service in NW-TT in respect to the service parameters in the request in step 2.

Upon reception of responses from each DS-TT and NW-TT, the TSCTSF determines the state of the time synchronization configuration.

The TSCTSF constructs a PMIC to each DS-TT/UE to subscribe for the port management information changes in the DS-TT. The TSCTSF constructs PMIC(s) and UMIC to NW-TT to subscribe for the port management and user-plane management information changes in NW-TT. The TSCTSF retrieves the PMIC(s) and UMIC via means of Npcf_PolicyAuthorization service operations.

The create request creates also a subscription for notifications for the changes in the time synchronization service configuration.

If the Ntsctsf_TimeSynchronization_ConfigCreate request contains a temporal validity condition with a start-time and/or the stop-time that is in the future, the TSCTSF maintains the start-time and stop-time for the time synchronization service for the corresponding time synchronization configuration. If the start-time is in the past, the TSCTSF treats the request as if the time synchronization service was activated immediately. When the start-time is reached, the TSCTSF proceeds as described in this step above. When the stop-time is reached for active time synchronization service configuration, the TSCTSF proceeds as Ntsctsf_TimeSynchronization_ConfigDelete was received as described in clause 4.15.9.3.4.

If the Ntsctsf_TimeSynchronization_ConfigCreate request contains a spatial validity condition, then the TSCTSF performs the following operations:

-	For each target UE, TSCTSF determines whether the TSCTSF has subscribed for the UE presence in Area of Interest composed by the TA(s) in the spatial validity condition. If not, the TSCTSF discovers the AMF(s) serving the TA(s) that comprise the spatial validity condition, using the NRF discovery service (Nnrf_NFDiscovery_Request) with the list of TA(s). Then the TSCTSF subscribes to the AMF(s) to receive notifications about the UE presence in Area of Interest using Namf_EventExposure operation with the corresponding event filters as described in clause 5.2.2.3. The subscribed area of interest may be the same as the spatial validity condition or may be a subset of the spatial validity condition (e.g. a list of TAs) based on the latest known UE location.

-	In order to ensure that a TAI list specifying the AoI for the AMF is aligned with UE’s Registration Area (RA), the following steps shall be performed:

-	When invoking the subscription with the AMF(s), the TSCTSF may provide an indication, a new Parameter Type = “Adjust AoI based on RA”, that the AMF may adjust the received AoI depending on UE’s RA.

-	After receiving the Namf_EventExposure_Subscribe request from the TSCTSF with the Parameter Type = “Adjust AoI based on RA” and specified AoI, the AMF compares TAs from the AoI with the UE’s Registration Area (RA). If the AoI includes one or more TA(s) that are part of UE's current RA, the AMF reports the UE is inside the Area Of Interest, otherwise the AMF reports the UE is outside the Area Of Interest, as described in Annex D.

-	The AMF notifies the TSCTSF about the UE’s presence in the AoI using the Namf_EventExposure_Notify service operation.

-	Based on the notification from the AMF and spatial validity condition received in step 1, the TSCTSF determines whether to activate time synchronization service for this UE:

-	If the UE location is within the spatial validity condition, the TSCTSF determines to activate time synchronization service for the UE/DS-TT creating the PTP port in DS-TT and adding it to the PTP instance. The TSCTSF uses the procedures described in clause K.2.2 of TS 23.501 [2] to configure and initialize the PTP instance in the DS-TT(s) and NW-TT.

-	If the UE location is outside the spatial validity condition, the TSCTSF determines not to activate time synchronization service and not to create a PTP port in a DS-TT.

The TSCTSF uses the procedure in clause 4.15.9.4 to activate or modify the 5G access stratum time distribution for the UEs that are part of the impacted PTP instance.

7.	The TSCTSF notifies the NEF (or AF) with the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation, containing the PTP instance reference and the current state of the time synchronization service configuration.

If TSCTSF received spatial validity condition as part of the Ntsctsf_TimeSynchronization_ConfigCreate request, the TSCTSF notifies the NEF (or AF) with the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation, whenever the UE moves in or out of the Area of Interest. The notification contains the PTP instance reference and the current state of the time synchronization service configuration.

8.	The NEF notifies the AF with the Nnef_TimeSynchronization_ConfigUpdateNotify service operation, containing the PTP instance reference and the current state of the time synchronization service configuration.

9.	Upon a change in the PTP instance in the DS-TT or NW-TT, the DS-TT or NW-TT report the change via PMIC or UMIC to the TSCTSF as described in clause K.2.2 of TS 23.501 [2].

Upon PDU Session release indication from a PCF, the TSCTSF removes the corresponding AF-session from the list of AF-sessions associated with the time synchronization configuration. The TSCTSF uses the procedure in clause 4.15.9.4 to remove the 5G access stratum time distribution parameters for the UE that is removed from the impacted PTP instance.

Upon PDU Session Establishment as defined clause 4.3.2.2.1, steps 10-13 in Figure 4.15.9.2-1 are repeated for the new PDU Session and the TSCTSF may notify the NEF (or AF) for the Time Synchronization capability event, optionally with the updated time synchronization capabilities, as described in step 12 in Figure 4.15.9.2-1.

NOTE 2:	Upon receiving the notification, the NEF (or AF) can use the Ntsctsf_TimeSynchronization_ConfigUpdate service operation to add the DS-TT/UE to the existing PTP instance and corresponding time synchronization service configuration.

If TSCTSF received spatial validity condition as part of the Ntsctsf_TimeSynchronization_ConfigCreate request, upon a change in the UE presence in Area of Interest, the TSCTSF determines if the spatial validity condition shall trigger an activation or deactivation of the time synchronization service:

-	If the UE has moved outside the spatial validity condition, then the TSCTSF temporarily removes the UE/DS-TT port from the PTP instance:

-	If the DS-TT is configured to send Sync, Follow_Up and Announce messages for the related PTP instance, then TSCTSF deactivates the Grandmaster functionality in the DS-TT using PMIC (see also clause K.2.2.4 of TS 23.501 [2]).

-	If NW-TT is configured to send Sync, Follow_Up and Announce messages on behalf of the DS-TT, then TSCTSF deactivates the Grandmaster functionality on behalf of the DS-TT in NW-TT using UMIC (see also clause K.2.2.4 of TS 23.501 [2]).

-	If the UE has moved inside the spatial validity condition, then the TSCTSF adds the DS-TT PTP port to the PTP instance and also (re-)activates the Grandmaster functionality (described in clause K.2.2 of TS 23.501 [2]).

10.	The TSCTSF updates the state of the time synchronization configuration and may notify the NEF (or AF) with the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation, containing the PTP instance reference and the updated state of the time synchronization service configuration, including whether there was a change in the UE's presence in the spatial validity condition (in cases when the AF has requested the service for a specific spatial validity condition).

11.	The NEF notifies the AF with the Nnef_TimeSynchronization_ConfigUpdateNotify service operation, containing the reference to the time synchronization service configuration (PTP instance reference) and the updated state of the time synchronization service configuration.

>>>> Next Change <<<<

#### 4.15.9.4	Procedures for management of 5G access stratum time distribution

The AF can use the procedure to activate, update or delete the 5G access stratum time distribution for one UE or a group of UEs.

The AF may query the status of the 5G access stratum time distribution using Nnef_ASTI_Get service operation. The Nnef_ASTI_Create and Nnef_ASTI_Update request may contain the parameters as described in Table 4.15.9.4-1.

Table 4.15.9.4-1: Description of 5G access stratum time distribution parameters

| Parameter | Description |
| --- | --- |
| 5G access stratum time distribution indication (enable, disable) | Indicates that the access stratum time distribution via Uu reference point should be activated or deactivated for the associated UE identities. |
| Time synchronization error budget | Indicates the time synchronization error budget for the time synchronization service (as described in clause 5.27.1.9 of TS 23.501 [2]). [optional] |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the time synchronization service is active. [optional] |
| Spatial Validity Condition | Indicates a geographical area (e.g., a civic address or shapes) or a TA list in which time synchronization service is enabled (NOTE X). [optional] |
| NOTE X: A geographical area requested by an AF includes at least one tracking area (TA), i.e., the granularity is restricted to a TA level in this release of the specifications. |  |

Figure 4.15.9.4-1: Management of 5G access stratum time information

1.	AM Policy Association establishment as described in clause 4.16.1.

2.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	To create a new request, the AF provides access stratum time distribution parameters to the NEF using the Nnef_ASTI_Create service operation (together with the AF identifier and potentially further inputs as specified in table 4.15.9.4-1), including a target (one UE identified by SUPI or GPSI, a group of UEs identified by an External Group Identifier. The NEF forwards the GPSI or the External Group Identifier to the TSCTSF by including it inside the Ntsctsf_ASTI_Create request.

-	To update or remove an existing request, the AF invokes an Nnef_ASTI_Update or Nnef_ASTI_Delete service operation providing the corresponding time synchronization configuration id.

-	To query the status of the access stratum time distribution, the AF invokes Nnef_ASTI_Get service operation providing the target (a list of UE identities (SUPI or GPSIs) or an External Group Identifier).

The AF that is part of operator's trust domain may invoke the services directly with the TSCTSF and identifies the targeted UE(s) using SUPI(s) or an Internal Group Identifier.

If the request includes a spatial validity condition and the AF uses a geographical area as a spatial validity condition, the NEF transforms this information into 3GPP identifiers (e.g. TAI(s)) based on pre-configuration.

NOTE 1:	Steps 1 and 2 can occur in any order.

NOTE Y:	It is assumed that AFs within the operator’s domain is aware of TAs that can be used to formulate a spatial validity condition for Time Synchronization Coverage Area (see clause 5.27.1.10 of TS 23.501[2]).

3.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_ASTI_Create/Update/Delete/Get service operation with the TSCTSF discovered and selected as described in clause 6.3.24 of TS 23.501 [2].

-	The TSCTSF determines whether the targeted UE is part of a PTP instance in 5GS, if so the TSCTSF rejects the request (steps 4-10 are skipped).

-	The TSCTSF calculates the Uu time synchronization error budget as described in clause 5.27.1.9 of TS 23.501 [2]. This does not apply in case of Ntsctsf_ASTI_Delete service operation.

(When the procedure is triggered by PTP instance activation, modification, or deactivation in the TSCTSF):

-	The TSCTSF calculates the Uu time synchronization error budget as described in clause 5.27.1.9 of TS 23.501 [2] for corresponding SUPIs that are part of the PTP instance. This does not apply in case of Ntsctsf_ASTI_Delete service operation.

4.	If the AF targeted UE(s) are identified by GPSI(s) or an External/Internal Group Identifier, the TSCTSF uses the Nudm_SDM_Get request to retrieve the subscription information (SUPI(s)) from the UDM using each GPSI or the External Group Identifier as received from the NEF, or an Internal Group Identifier as provided by the AF directly.

5.	The UDM provides the Nudm_SDM_Get response containing SUPI(s) that are mapped from each received GPSI or the External/Internal Group Identifier and identify the targeted UEs.

6a.	If the Ntsctsf_ASTI_Create request in step 2 contains a spatial validity condition, then the TSCTSF performs the following operations:

-	For each target UE, TSCTSF determines whether the TSCTSF has subscribed for the UE presence in Area of Interest composed by the TAs list in the spatial validity condition. If not, the TSCTSF discovers the AMF(s), serving in the TAs that comprise the spatial validity condition, using the NRF discovery service (Nnrf_NFDiscovery_Request) with the list of TAs. Then the TSCTSF subscribes to the AMF(s) to receive notifications about the UE presence in Area of Interest using Namf_EventExposure operation with the corresponding event filters as described in clause 5.2.2.3. The subscribed area of interest may be the same as the spatial validity condition or may be a subset of the spatial validity condition (e.g. a list of TAs) based on the latest known UE location.

-	In order to ensure that a TAI list specifying the AoI for the AMF is aligned with UE’s Registration Area (RA), the following steps shall be performed:

-	When invoking the subscription with the AMF(s), the TSCTSF may provide an indication, a new Parameter Type = “Adjust AoI based on RA”, that the AMF may adjust the received AoI depending on UE’s RA.

-	After receiving the Namf_EventExposure_Subscribe request from the TSCTSF with the Parameter Type = “Adjust AoI based on RA” and specified AoI, the AMF compares TAs from the AoI with the UE’s Registration Area (RA). If the AoI includes one or more TA(s) that are part of UE's current RA, the AMF reports the UE is inside the Area Of Interest, otherwise the AMF reports the UE is outside the Area Of Interest, as described in Annex D.

-	The AMF notifies the TSCTSF about the UE’s presence in the AoI using the Namf_EventExposure_Notify service operation.

-	Based on the notification from the AMF and spatial validity condition received in step 1, the TSCTSF determines whether to activate time synchronization service for this UE:

-	If the UE location is within the spatial validity condition, the TSCTSF determines to enable access stratum time distribution for the UE.

-	If the UE location is outside the spatial validity condition, the TSCTSF determines to disable access stratum time distribution for the UE.

6b.	If for any targeted UE, the "AF request Authorization" in the Time Synchronization Subscription Data indicates that the AF is allowed to request 5G access stratum-based time distribution, the TSCTSF proceeds with ASTI service configuration. Otherwise, if the AF is not authorized, steps 7-11 are skipped for this UE.

7.	The TSCTSF searches the PCF for the UE using Nbsf_Management_Subscribe with a SUPI as an input parameter, indicating that it is searching for the PCF that handles the AM Policy Association of the UE.

8.	The BSF provides to the TSCTSF the identity of the PCF for the UE for the requested SUPI via an Nbsf_Management_Notify operation. If matching entries already existed in the BSF when step 6 is performed, this shall be immediately reported to the TSCTSF.

9.	The TSCTSF sends to the PCF for the UE its request for the AM policy of the UE (identified by SUPI) using Npcf_AMPolicyAuthorization request, containing the 5G access stratum time distribution indication (enable, disable) and optionally the calculated Uu time synchronization error budget.

10.	If the PCF receives multiple time synchronization error budgets for a given UE, then the PCF picks the most stringent budget. The PCF takes a policy decision and then the PCF may initiate an AM Policy Association Modification procedure for the UE as described in clause 4.16.2.2 to provide AMF the 5G access stratum time distribution parameters. As part of this, the AMF shall, if supported, store the 5G access stratum time distribution indication (enable, disable) and the Uu time synchronization error budget in the UE context in AMF and send the 5G access stratum time distribution indication (enable, disable) and the Uu time synchronization error budget, when they are available, to NG-RAN during mobility registration, AM policy modification, Service Request, N2 Handover and Xn handover as specified in TS 38.413 [10]. The NG-RAN node shall, if supported, store the information in the UE Context. Based on this information, the NG-RAN node provides the 5GS access stratum time to the UE according to the Uu time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN).

NOTE 2:	This release of the specification assumes that deployments ensure that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [30].

11.	The PCF of the UE replies to the TSCTSF with the result of Npcf_AMPolicyAuthorization operation.

12.	The TSCTSF responds the AF with the Ntsctsf_ASTI_Create/Update/Delete/Get service operation response.

13.	The NEF informs the AF about the result of the Nnef_ASTI_Create/Update/Delete/Get service operation performed in step 2.

14.	If TSCTSF received spatial validity condition as part of the Ntsctsf_ASTI_Create request in step 2, upon the reception of a change in the UE presence in Area of Interest notification, the TSCTSF determines if the spatial validity condition shall trigger an activation or deactivation of the access stratum time distribution:

-	If the UE has moved inside the spatial validity condition, then the TSCTSF determines to enable access stratum time distribution for the UE.

-	If the UE has moved outside the spatial validity condition, then the TSCTSF determines to disable access stratum time distribution for the UE.

15-16.	If TSCTSF determines to modify access stratum time distribution for the UE in step 14 for which the AF requested access stratum time distribution, the TSCTSF notifies the service status to AF.

>>>> Next Change <<<<

##### 5.2.2.3.1	General

Service description: This service enables an NF to subscribe and get notified about an Event ID.

Following UE access and mobility information event are considered (Event ID is defined in clause 4.15.1 and Table 4.15.3.1-1):

-	Location Report (TAI, Cell ID, N3IWF/TNGF node, UE local IP address and optionally UDP source port number);

-	UE moving in or out of a subscribed "Area Of Interest" as described in clauses 5.3.4.4 and 5.6.11 in TS 23.501 [2];

-	Number of UEs served by the AMF and located in "Area Of Interest";

-	Time zone changes (UE Time zone);

-	Access Type changes (3GPP access or non-3GPP access);

-	Registration state changes (Registered or Deregistered);

-	Connectivity state changes (IDLE or CONNECTED);

-	UE loss of communication;

-	UE reachability status;

-	UE indication of switching off SMS over NAS service;

-	Subscription Correlation ID change (implicit subscription);

-	UE Type Allocation code (TAC);

-	Frequent mobility re-registration;

-	Subscription Correlation ID addition (implicit subscription);

-	User State Information in 5GS, as described in clause 5.4.4 in TS 23.632 [68];

-	UE access behaviour trends (see clause 4.15.4.2);

-	UE location trends (see clause 4.15.4.2); and

-	Total number of Mobility Management transactions:

-	The Total number of Mobility Management transactions is used to collect the number of MM transactions of a SUPI or Internal Group ID, for example Dispersion Analytics as specified in TS 23.288 [50]. The Total number of transactions is incremented when the NAS signalling transactions from Authentication, Registration, De-Registration, Service Request and UE Configuration Update procedures is completed. Only the periodic reporting mode applies.

In addition to UE access and mobility information event, AMF exposes the "S-NSSAIs per TAI mapping" event providing, per TAI, the related access type and list of supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF). The Event Consumer may use as target of event reporting a list of TAIs, or "any TAI" and may use event filter information including a list of "S-NSSAIs". Whenever there is a change in supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF) for a TAI, the event notification is generated with the updated information.

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides some examples on how the conditions to match for event reporting can be specified for various Event IDs for AMF exposure.

NOTE:	The conditions to match can be set based on AMF-associated expected UE Behaviour parameter(s) to only notify the event when the UE's behaviour deviates from its expected UE behaviour as described in TS 23.288 [50].

Table 5.2.2.3.1-1: Example of Event Filters for AMF exposure events

| Event ID | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Location Report | <Parameter Type = LocationFilter, Value = TA1> |
| UE moving in or out of Area of Interest | <Parameter Type = TAI, Value = TA1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = NSI ID, Value = NSI ID1> <Parameter Type = PRA ID, Value = PRA ID value> <Parameter Type = Adjust AoI based on RA, Value = Yes or No> to indicate that AMF may adjust the received AoI depending on UE’s current Registration Area. Absence of this parameter in the request is interpreted as “AoI remains unchanged.” |
| Access Type | <Parameter Type=AN Type, Value=3GPP Access"> |
| Location | <Parameter Type=TAI, Value=wildcard> (to report any TAI change) |
| Location | <Parameter Type=TAI Value=abnormal> (to report only when the TAI deviates from expected values based on Expected UE Moving Trajectory). |
| Reachability Filter | Applicable to the event UE reachability. Value = UE reachability status change or UE reachable for DL traffic. Absence of this parameter in UE reachability event request is interpreted as "UE reachability status change". |
| Total number of Transactions | <Parameter Type = TAI, Value = TA1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| Number of UEs present in a geographical area | <Parameter Type = UE Type, Value = Aerial UE> <Parameter Type = PDU session status, Value = PDU session established for DNN subject to aerial services> |

The following service operations are defined for the Namf_EventExposure service:

-	Namf_EventExposure_Subscribe.

-	Namf_EventExposure_UnSubscribe.

-	Namf_EventExposure_Notify.

>>>> Next Change <<<<

Annex D (normative):
UE Presence in Area of Interest

# D.1	Determination of UE presence in Area of Interest by AMF

If the AMF has requested NG-RAN location reporting as specified in clause 4.10 for the Area Of Interest and UE is in CM-CONNECTED state, including RRC_CONNECTED and RRC_INACTIVE state, the AMF determines the UE presence in the Area of Interest as the reported value from the NG-RAN, as specified in clause D.2.

If the AMF has requested N2 Notification as specified in clause 4.8.3, the AMF determines the UE presence in Area Of Interest as follows, taking N2 Notification from NG-RAN into consideration:

-	IN:

-	if the UE is in CM-CONNECTED with RRC_CONNECTED state and if the last received User Location Information for the UE is inside the Area Of Interest service area; or

NOTE 1:	The above is valid e.g. under the condition that Area Of Interest border coincides with NG-RAN node service area border(s).

-	if the UE is in CM-CONNECTED and if the UE is inside a Registration Area which is completely contained within the Area Of Interest.

-	OUT:

-	if the UE is in CM-CONNECTED with RRC_CONNECTED state and if the last received User Location Information for the UE is outside the Area Of Interest; or

NOTE 2:	The above is valid e.g. under the condition that Area Of Interest border coincides with NG-RAN node service area border(s).

-	if the UE is in CM-CONNECTED and if UE is inside a Registration Area which does not contain any part of Area Of Interest.

-	UNKNOWN:

if none of above conditions for IN or OUT is met.

Otherwise, AMF determines the UE presence of Area Of Interest as follows:

-	IN:

-	if the UE is inside the Area Of Interest service area and if the UE is in CM-CONNECTED state; or

-	if the Parameter Type with value "Adjust AoI based on RA" is included, and the UE is inside a Registration Area which contains at least one Tracking Area that is contained within the Area Of Interest, or

-	if the UE is inside a Registration Area which is contained within the Area Of Interest.

-	OUT:

-	if the UE is outside the Area Of Interest in CM-CONNECTED and the Parameter Type with value "Adjust AoI based on RA" is not included; or

-	if UE is inside a Registration Area which does not contain any part of Area Of Interest.

-	UNKNOWN:

if none of above conditions for IN or OUT is met.

>>>> End of Changes <<<<