SA WG2 Meeting #S2-156E	S2-2303930

17 - 21 April, 2023, Electronic

3GPP TSG-CT WG1 Meeting #140	C1-231181

# Athens, Greece, 27 February – 3 March 2023

# Title:	LS on N3IWF selection for emergency services for UE not equipped with valid SNPN credentials

# Response to:	NA

# Release:	Release 18

# Work Item:	eNPN_Ph2

Source:	CT1

To:	SA2

Cc:	CT4

Contact Person:

Name:	ZHOU Xingyue (Joy)

E-mail Address:	zhou.xingyue@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	NA

1. Overall Description:

TS 23.501 clause 5.30.2.12 specifies:

"… …

UE initiates N3IWF selection for emergency services when the UE detects a user request for emergency session and determines that Untrusted non-3GPP access is to be used for the emergency access. The UE with SNPN subscription performs the following:

-	If the UE determines that it is located in the same country as the subscribed SNPN, the UE uses the configured N3IWF FQDN for N3IWF selection.

-	Otherwise, the UE follows the N3IWF selection procedure for Emergency services for UE not equipped with valid SNPN credentials, in the same way as defined in clause 6.3.6.4.2 for a UE not equipped with a UICC.",

where refers to following statement in clause 6.3.6.4.2 of TS 23.501:

"If the UE is not equipped with a UICC, the UE shall perform the emergency N3IWF selection procedure above as if always in a visited country and without using the Non-3GPP Access Node Selection Information, i.e. the UE may construct the Operator Identifier based N3IWF FQDN format based on a PLMN ID obtained via implementation specific means."

According to the statement above, for a UE attempting to access SNPN services via untrusted non-3GPP access for emergency services, if the UE:

a)	determines that it is located in a visited country; or

b)	is not equipped with valid SNPN credentials,

the UE will turn to select an N3IWF using an Operator-Identifier based N3IWF FQDN where the "Operator Identifier" is a PLMN ID obtained in an implementation way. Then it seems that the N3IWF selection has nothing to do with "accessing SNPN" and this seems confusing.

Hence, CT1 have following question to SA2 for clarification:

Is the "Operator Identifier" in Operator-Identifier based N3IWF FQDN used by a UE in SNPN access mode, without valid SNPN credentials or located in visited country, to selects an N3IWF for emergency services, associated with any SNPN identifier?

2. Actions:

To SA WG2 group.

ACTION: 	CT1 kindly asks SA2 to provide an answer to the question for clarification.

3. Date of Next CT1 Meetings:

CT1#141e	17th - 21st April 2023 		E-Meeting

CT1#142	22nd – 26th May 2023 		Bratislava, Slovakia