3GPP TSG-WG SA2 Meeting #156-e	S2-2303995

April 17 - 21, 2023, Electronic	(revision of S2-2302297)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3879 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Satellite Coverage Availability Function (SCAF) services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, THALES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of Satellite Coverage Availability Function (SCAF) services as proposed in S2-2302295. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Extends the NEF services with a new service for Satellite Coverage Availability provisioning via the NEF. 2) Defines SCAF services for: - Provisioning of satellite coverage availability (SCA) information from the AF (either directly or via the NEF) - Providing satellite coverage availability for specific geographical area (e.g. based on AMF request). - Subscription for SCA updates. - Notification of SCA updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The provisioning of satellite coverage availability information to AMF will remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.1; 5.2.6.x (new); 5.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR4123 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*********** Start change **************

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |
| Nnef_PDTQPolicyNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_SCAFProvisioning | Update | Request/Response | AF |

*********** Next change **************

#### 5.2.6.x	Nnef_SCAFProvisioning service

##### 5.2.6.x.1	General

This service is for allowing external party to provision the SCAF with satellite coverage availability information.

##### 5.2.6.x.2	Nnef_SCAFProvisioning_Update operation

Service operation name: Nnef_SCAFProvisioning_Update

Description: The consumer updates the satellite coverage availability information for a specific geographical area.

Inputs (required): Geographical area(s), Satellite coverage availability information.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

*********** Next change **************

### 5.2.x	SCAF Services

#### 5.2.x.1	General

The following table illustrates the SCAF Services.

Table 5.2.x.1-1: NF services provided by SCAF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Nscaf_Provisioning | Update | Request/Response | AF, NEF |
| Nscaf_SatelliteCoverageAvailability | Obtain | Request/Response | AMF |
|  | Subscribe | Subscribe/Notify | AMF |
|  | Unsubscribe | Subscribe/Notify | AMF |
|  | Notify | Subscribe/Notify | AMF |

#### 5.2.x.2	Nscaf_Provisioning service

##### 5.2.x.2.1	Nscaf_Provisioning_Update service operation

Service operation name: Nscaf_Provisioning_Update.

Description: The consumer updates the satellite coverage availability information for a specific geographical area.

Inputs (required): Geographical area(s), Satellite coverage availability information.

Inputs (optional): None.

Outputs (required): None.

Outputs (optional): None.

#### 5.2.x.3	Nscaf_SatelliteCoverageAvailability service

##### 5.2.x.3.1	Nscaf_SatelliteCoverageAvailability_Obtain service operation

Service Operation name: Nscaf_SatelliteCoverageAvailability_Obtain

Description: Consumer NF gets the satellite coverage availability information corresponding to a specific geographical area.

Inputs, Required: Geographical area.

Inputs, Optional: None.

Outputs, Required: Satellite coverage availability information.

Outputs, Optional: None.

##### 5.2.x.3.2	Nscaf_SatelliteCoverageAvailability _Subscribe service operation

Service operation name: Nscaf_SatelliteCoverageAvailability_Subscribe

Description: The NF consumer subscribes for updates to satellite coverage availability information. The SCAF shall check that the requested consumer is authorized to subscribe to requested updates.

Inputs, Required: None.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.x.3.3	Nscaf_SatelliteCoverageAvailability _Unsubscribe service operation

Service operation name: Nscaf_SatelliteCoverageAvailability_Unsubscribe

Description: The NF consumer unsubscribes from updates to satellite coverage availability information.

Inputs, Required: None.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.x.3.4	Nscaf_SatelliteCoverageAvailability _Notify service operation

Service Operation name: Nscaf_SatelliteCoverageAvailability_Notify

Description: Producer NF provides notifications about updates to satellite coverage availability information to subscribed consumer NF.

Inputs, Required: Geographical area(s), Satellite coverage availability information.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

*********** End change **************