3GPP TSG-WG SA2 Meeting #156-e	S2-2303977

April 17 - 21, 2023, Electronic	(revision of S2-2302877)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.682 | CR | 0491 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Satellite Coverage Availability Function (SCAF) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, THALES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  |  |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of Satellite Coverage Availability Function (SCAF) services as proposed in S2-2302295. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EPS architecture for provisioning of satellite coverage information in the SCAF is added. SCAF network element description is added. High level function description of SCAF provisioning is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The provisioning of satellite coverage availability information to SCAF will remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1; 3.2, 4.2, 4.3.2, 4.3.3.x (new), 4.4.x (new), 4.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.401 CR3725 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*********** Start change **************

# 1	Scope

The present document specifies architecture enhancements to facilitate communications with packet data networks and applications (e.g. Machine Type Communication (MTC) applications on the (external) network/MTC servers) according to the use cases and service requirements defined in TS 22.368 [2], TS 22.101 [3], and related 3GPP requirements specifications. Both roaming and non-roaming scenarios are covered.

The present document also specifies transmission of non-IP data via SCEF for the CIoT EPS Optimization.

The present document also specifies the interface between the SCEF and the SCS/AS.

The present document also specifies provisioning of UCMF with RACS information.

The present document also specifies provisioning of SCAF with satellite coverage availability information.

*********** Next change **************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AS	Application Server

CDR	Charging Data Record

CDF	Charging Data Function

CGF	Charging Gateway Function

CIoT	Cellular Internet of Things

CP	Communication Pattern

DDN	Downlink Data Notification

IWK-SCEF	Interworking SCEF

MTC	Machine Type Communications

MTC-IWF	Machine Type Communications-InterWorking Function

NIDD	Non-IP Data Delivery

PCRF	Policy and Charging Rules Function

P-GW	PDN Gateway

PFDF	Packet Flow Description Function

PSM	Power Saving Mode

SCAF	Satellite Coverage Availability Function

SCEF	Service Capability Exposure Function

SCS	Services Capability Server

SLF	Subscriber Location Function

SME	Short Message Entities

SMS-SC	Short Message Service-Service Centre

SRI	Send Routing Information

UCMF	UE radio Capability Management Function

WB-E-UTRAN	Wide Band E-UTRAN

*********** Next change **************

## 4.2	Architectural Reference Model

Figures 4.2-1a and 4.2-1b show the architecture for a UE used for MTC connecting to the 3GPP network (UTRAN, WB-E-UTRAN, NB-IoT, GERAN, etc.) via the Um/Uu/LTE-Uu interfaces or satellite access interface for NB-IoT, WB-E-UTRAN and LTE-M. They also show the 3GPP network service capability exposure to SCS and AS. The architecture covers the various architectural models described in clause 4.1.

Figure 4.2-1a: 3GPP Architecture for Machine-Type Communication (non-roaming)

# Figure 4.2-1b: 3GPP Architecture for Machine-Type Communication (Roaming)

Figure 4.2-2 shows the overall architecture for Service Capability Exposure which enables the 3GPP network to securely expose its services and capabilities provided by 3GPP network interfaces to external 3rd party service provider SCS/AS hosting an Application(s).

Figure 4.2-2: 3GPP Architecture for Service Capability Exposure

Figure 4.2-3: 3GPP roaming Architecture for Service Capability Exposure

Figure 4.2-4 shows the overall architecture for RACS which enables provisioning of RACS database information in the 3GPP network.

Figure 4.2-4: 3GPP Architecture for RACS

Figure 4.2-4 shows the overall EPS architecture for provisioning of satellite coverage availability information.

Figure 4.2-x: EPS architecture for provisioning of satellite coverage availability information

NOTE 1:	Refer to TS 23.002 [5], TS 23.060 [6], TS 23.401 [7], TS 23.272 [11] and TS 23.040 [12] for the details of 3GPP network-internal reference points not specifically shown or labelled in figure 4.2-1a, figure 4.2-1b, figure 4.2-2, or described in this specification.

NOTE 2:	The SCS is controlled by the operator of the HPLMN or by a MTC Service Provider.

NOTE 3:	In the non-roaming case, all 3GPP network entities providing functionality for MTC are in the same PLMN. In the roaming case, 3GPP architecture for MTC supports both the home routed (illustrated in Figures 4.2-1a and 4.2-1b) and the local-breakout roaming (not illustrated) scenarios. For the home routed scenario, the MTC Server/Application User Plane communication is routed through the HPLMN. In the local breakout scenario, the User Plane communication is routed directly through the serving PLMN/VPLMN over deployed GGSN/P-GW.

NOTE 4:	Figure 4.2-2 does not include all the interfaces and network elements that may be connected to SCEF.

NOTE 5:	Figure 4.2-3 does not include all the interfaces and network elements that may be connected to an Interworking SCEF (IWK-SCEF).

The SCS is an entity which connects to the 3GPP network to communicate with UEs used for MTC and the MTC-IWF and/or SCEF in the HPLMN. The SCS offers capabilities for use by one or multiple MTC Applications. A UE can host one or multiple MTC Applications. The corresponding MTC Applications in the external network are hosted on one or multiple ASs.

Tsms is the interface that encompasses all the various proprietary SMS-SC to SME interface standards (see TR 23.039 [14]) and is outside the scope of 3GPP specifications. Tsms can be used to send a trigger to a UE encapsulated in a MT-SMS as an over-the-top application by any network entity (e.g. SCS) acting as a SME. Tsp is a 3GPP standardized interface to facilitate value-added services motivated by MTC (e.g. control plane device triggering) and provided by a SCS.

T8 is the interface between the SCEF and the SCS/AS. SCEF exposed network services can be accessed by SCS/AS through APIs over T8 interface. In the indirect model, the SCS and the Application Server hosting Application(s) can be collocated.

For the roaming scenario, the MTC-IWF shall have the connection with HSS and SMS-SC within the home network only as shown in the figure 4.2-1b.

The Service Capability Exposure Function (SCEF) is the key entity within the 3GPP architecture for service capability exposure that provides a means to securely expose the services and capabilities provided by 3GPP network interfaces. In standalone MTC-IWF deployment, MTC-IWF functionality (e.g. T4 triggering) is made available to the SCS/AS via the Tsp interface. In certain deployments, the MTC-IWF may be co-located with the SCEF in which case MTC-IWF functionality is exposed to the SCS/AS via T8 interface (i.e. API). In deployments where MTC-IWF is not co-located with SCEF, interactions between MTC-IWF and SCEF are left up to the implementation.

The trust domain (see figure 4.2-2) cover entities that are protected by adequate network domain security. The entities and interfaces within the trust domain may all be within one operator's control, or some may be controlled by a trusted business partner which has a trust relationship with the operator e.g. another operator or a 3rd party. The security requirements for the trust domain are out of scope of this specification.

When the SCEF belongs to a trusted business partner of the HPLMN, it is still seen as an HPLMN entity by other HPLMN or VPLMN functional entities invoked by the SCEF (e.g. HSS, MME).

Applications operating in the trust domain may require only a subset of functionalities (e.g. authentication, authorization, etc.) provided by the SCEF. Applications operating in the trust domain can also access network entities (e.g. PCRF), wherever the required 3GPP interface(s) are made available, directly without the need to go through the SCEF.

The Interworking SCEF (IWK-SCEF) is optional. When deployed, the IWK-SCEF is located in the VPLMN as shown in the figure 4.2-1b.

*********** Next change **************

### 4.3.2	List of Reference Points

The description of the MTC and Service Capability Exposure related reference points:

Tsms:	Reference point used by an entity outside the 3GPP network to communicate with UEs used for MTC via SMS.

Tsp:	Reference point used by a SCS to communicate with the MTC-IWF related control plane signalling.

T4:	Reference point used between MTC-IWF and the SMS-SC in the HPLMN.

T6a:	Reference point used between SCEF and serving MME.

T6b:	Reference point used between SCEF and serving SGSN.

T6ai:	Reference point used between IWK-SCEF and serving MME.

T6bi:	Reference point used between IWK-SCEF and serving SGSN.

T7:	Reference point used between IWK-SCEF and SCEF.

T8:	Reference point used between the SCEF and the SCS/AS.

T9a:	Reference point used between UCMF and AS.

T9b:	Reference point used between UCMF and SCEF.

T9c:	Reference point used between SCAF and AS.

T9d:	Reference point used between SCAF and SCEF.

S6m:	Reference point used by MTC-IWF to interrogate HSS/HLR.

S6n:	Reference point used by MTC-AAA to interrogate HSS/HLR.

S6t:	Reference point used between SCEF and HSS.

Rx:	Reference point used by SCEF and PCRF. Functionality for Rx reference point is specified in TS 23.203 [27].

Ns:	Reference point used between SCEF and RCAF.

Nt:	Reference point used by SCEF and PCRF. Functionality for Nt reference point is specified in TS 23.203 [27].

Nu:	Reference point used by SCEF to interact with the PFDF.

NOTE 1:	Protocol assumption: User plane communication with SCS, for Indirect model, and AS, for Direct and Hybrid models, is achieved using protocols over Gi and SGi reference points. Control plane protocols over those reference points such as RADIUS/Diameter as specified in TS 29.061 [8] can also be supported towards the SCS.

NOTE 2:	It is assumed that interfaces on the T6ai/T6bi/T7 reference points use the same protocol(s) as interfaces on the T6a/T6b reference points.

*********** Next change **************

### 4.3.3	Reference Point Requirements

#### 4.3.3.x	T9c/T9d Reference Point Requirements

The T9c and T9d reference points shall fulfil the following requirements:

-	T9c connects the SCAF to an AS;

-	T9d connects the SCAF to the SCEF;

-	supports the following functionality:

-	provisioning of satellite coverage availability information to the SCAF.

*********** Next change **************

## 4.4	Network Elements

### 4.4.x	Satellite Coverage Availability Function

SCAF specific functionality to support provisioning of satellite coverage availability information includes the following:

-	SCAF terminates the T9c reference point;

-	SCAF terminates the T9d reference point.

The services and capabilities offered by SCAF to SCEF and SCS/AS include:

-	Provisioning of satellite coverage availability information.

*********** Next change **************

## 4.5	High Level Function

### 4.5.x	Provisioning of satellite coverage availability information

The SCAF (Satellite Coverage Availability Function) is used for storage of satellite coverage availability information, see TS 23.401 [7]. Provisioning of satellite coverage availability information in the SCAF is performed from an AS that interacts with the SCAF either directly or via the SCEF (or via OAM).

*********** End change **************