3GPP TSG-SA2 Meeting #156 (e-meeting)	S2-2304032

# Elbonia, April 17th – 21st, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0731 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the Procedure for Preparation of Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add sevice name for Server NWDAF sending Federated Learning preparation request to the Client NWDAF(s), i.e., Nnwdaf_MLModelTraining_Subscribe service operation with ML Preparation Flag. Solving the EN in clause 6.2C.2.1: "Other parameters in the preparation request and the definition of 'Available data requirement' and 'availability time requirement' are FFS". Available data requirement is a list of event IDs that should be used as the input data for the ML model training. The availability time requirement is the time needed for the whole FL process. Regarding other parameters, one could think computation requirement, framework requirement, security requirement (e.g., whether use secure multi-party computation), etc. The interoperability information can be used to indicate those information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: - Added "Nnwdaf_MLModelTraining_Subscribe service operation with ML Preparation Flag" to step 7 for Server NWDAF sending Federated Learning preparation request to the Client NWDAF(s). - To solve the EN, the following changes are made: - Added descriptions to the parameters (i.e., "Available data requirement" and "availability time requirement") to step 7. - Modified step 8 to enable Client NWDAF(s) making determination by checking the parameters in the request from Server NWDAF. - Added descriptions to step 9 accordingly to the changes in steps 7 and 8. - Added parameters “availability” and “computation and communication capability” as optional outputs to Nnwdaf_MLModelTraining_Subscribe service operation in clause 7.10.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for preparation of Federated Learning is incompleted, parameters are missing in the preparation request from Server NWDAF to Client NWDAF(s). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.1, 7.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 6.2C.2.1	Registration and Discovery procedure for Federated Learning

Figure 6.2C.2.1-1: Registration and Discovery procedure for Federated Learning

1-3.	NWDAF containing MTLF as Server NWDAF or Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability information including FL capability Type (i.e. FL server or FL client) and Time interval supporting FL as described in clause 5.2.

Editor's note:	Definition of FL capability and whether FL capability includes computation capability is FFS.

4-6.	NWDAF containing MTLF determines ML model requires FL based on Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as Server NWDAF, the MTLF first discovers and selects Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, Model filter information as defined in clause 6.2A.2, FL capability Type (i.e. FL server), check if the selected Server NWDAF is currently executing an Federated Learning process for the Analytics ID, Time Period of Interest, Service Area.

Once the Server NWDAF (the requested or the selected one) is determined, it discovers and selects other NWDAF(s) containing MTLF as Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, Data availability by the Client NWDAF, Time Period of Interest, Interoperability Indicator.

Editor's note:	Additional criteria for FL server to discover FL clients from the NRF (e.g. data availability) is FFS.

7.	Server NWDAF sends Federated Learning preparation request to the Client NWDAF(s) by invoking Nnwdaf_MLModelTraining_Subscribe service operation with ML Preparation Flag as defined in clause 7.10.2 and Interoperability information. In the preparation request, the following parameters may be added: Available data requirement (i.e. a list of Event IDs that the local data needs include for ML model training), availability time requirement (i.e. time period needed for the FL process).

8.	Client NWDAF(s) decides whether to join the Federated Learning process based on implementation. The Client NWDAF(s) make determination by checking the Interoperability information, its data and time availability, computation and communication capability.

NOTE:	Interoperability information can indicate the execution environment of the FL process. The execution environment can have computation and communication capability requirement, which need to be checked by the Client NWDAF(s).

9.	Client NWDAF(s) sends response to Server NWDAF indicating if it will join the FL process or not. If the Client NWDAF(s) will not join the FL process, it may contain the reason in the response message, e.g. it is not available or cannot fullfil the data, time, computation or communication requirement.

10.	Server NWDAF conducts selection of Client NWDAF(s).

* * * Next of Changes * * *

### 7.10.2	Nnwdaf_MLModelTraining_Subscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Subscribe

Description: Subscribes to NWDAF ML model training with specific parameters.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Interoperability information;

-	Notification Target Address (+ Notification Correlation ID);

Inputs, Optional:

-	ML Model ID: identifies the provided ML model.

-	ML Model Information (i.e. file address (e.g. URL or FQDN) of ML Model that needs to update);

-	Subscription Correlation ID (in the case of modification of the ML Model Training subscription);

-	ML Training Information, i.e. data availability requirement, time availability requirement.

-	ML Preparation Flag;

-	ML Model Accuracy Check Flag;

-	ML Correlation ID;

-	Training Filter Information;

-	Target of Training Reporting;

-	Training Reporting Information as defined in clause 6.2F.2;

-	Use case context;

-	Expiry time.

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the ML training requirements).

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: ML Correlation ID (e.g. confirm of the subscription for this FL process), cause code (e.g. Client NWDAF is not available or cannot fullfill the data, time, computation and communication reqirement).

* * * End of Changes * * *