3GPP TSG-SA WG2 Meeting #156 (e-meeting)	 S2-2304145

# Elbonia, April 17 – 21, 2023																		(rev of )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3971 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve EN on Parameter Provision data key for MBS Session Assistance Info in UDM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Regarding what data key should be used for MBS Session Assistance Information, there is following EN: Editor’s Note: Whether GPSI or MBS Session ID should be used as data key is to be decided. Per clause 5.2.3.6.1, “MBS Session Authorization information” in Parameter Provisiong service is indexed by UEId (External Group Id) but not by MBS Session Id. Per TS 29.503, Figure 6.5.3.1-1 (see below), UE ID is included in Resource URI structure of the Nudm_PP API, but not MBS Session ID. Figure 6.5.3.1-1: Resource URI structure of the Nudm_PP API Currently MBS Session ID is not part of UDM’s resource structure for PPdata. Introducing new Data Key “MBS Session ID” in PP data would impact the UDM’s resource structure. Besides, if the list of UEs is handled by multiple UDMs, it is not possible to discover the UDM using MBS Session ID as query parameter. Instead, GPSI(s) will be used to discover the UDM(s). Based on the above, to minimize the impact to the PP handling in the UDM and make the solution work regardless of list of UEs handled by a single UDM or multiple UDMs, it is proposed to reuse the existing mechanism and use GPSI as the data key for MBS Session Assistance Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the EN by using the GPSI as the data key for MBS Session Assistance Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN remains unsolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 5.2.3.6	Nudm_ParameterProvision service

##### 5.2.3.6.1	General

This service is for allowing NEF to provision of information which can be used for the UE in 5GS.

Parameter Provision data types used in the Nudm_ParameterProvision Service are defined in Table 5.2.3.6.1-1 below.

Table 5.2.3.6.1-1: Parameter Provision data types

| Parameter Provision data type | Description |
| --- | --- |
| Expected UE Behaviour parameters | See clause 4.15.6.3 |
| Network Configuration parameters | See clause 4.15.6.3a |
| 5G VN group configuration data | 5G VN Group Data and 5G VN Group membership management parameters for the 5G VN Group. See clause 4.15.6.3b and clause 4.15.6.3c. |
| Location Privacy Indication parameters | Location Privacy Indication parameters of the "LCS privacy" Data Subset of the Subscription Data. See clause 5.2.3.3.1 and clause 7.1 of TS 23.273 [51]. |
| Enhanced Coverage Restriction Information | See clause 4.27.1 and clause 5.31.12 of TS 23.501 [2]. |
| ECS Address Configuration Information | See clause 4.15.6.3d. |
| Multicast MBS group membership management parameters | See clause 7.2.9 of TS 23.247 [78]. |
| MBS Session Authorization information | See clause 7.2.9 of TS 23.247 [78]. |
| MBS Session Assistance Information | See clause 7.2.9a of TS 23.247 [78]. |
| DNN and S-NSSAI specific Group Parameters | See clause 4.15.6.3.e. |

At least a mandatory key is required for each Parameter Provision Data Type to identify the corresponding data as defined in Table 5.2.3.6.1-2.

Table 5.2.3.6.1-2: Parameter Provision data types keys

| Parameter Provision Data Types | Data Key | Data Sub Key |
| --- | --- | --- |
| Expected UE Behaviour parameters | GPSI or External Group ID | - |
| Network Configuration parameters | GPSI | - |
| 5G VN group data | External Group Identifier | - |
| 5G VN group membership management parameters | External Group Identifier | - |
| Location Privacy Indication parameters. | GPSI | - |
| Enhanced Coverage Restriction Information | GPSI | - |
| ECS Address Configuration Information | GPSI or External Group ID or any UE | - |
| Multicast MBS group membership management parameters | External Group Identifier | - |
| MBS Session Authorization information | External Group ID | - |
| MBS Session Assistance Information | GPSI |  |
| DNN and S-NSSAI specific Group Parameters | External Group ID |  |

* * * End of Changes * * *