3GPP TSG-SA2 Meeting #156E	S2-2304373

# 17th – 21th April, 2023, Elbonia													   revision of S2-xxxxxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4287 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN identifiers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in TR 23.700-88, PIN ID needs to be supported by 5G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe identifers for PIN. The 5G System already supports group identification and group membership management through External Group ID and Internal Group ID. A PIN also has exactly the same requirements that can be already fullfilled using this group management feature. It is proposed that a PIN is identified externally by a PIN ID. It is also proposed that External Group ID and Internal Group ID is used to identify and manage the PEGC UEs that are part of a PIN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the PIN is identified and how the PEGCs that are part of a PIN are managed in 5GC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 5.44.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change (all text are new) * * * *

## 5.44	Support for Personal IoT Networks

### 5.44.x	PIN Identifiers

A PIN, pre-configured or created by an AF, is identified by a PIN ID. There may be multiple PEGC UEs part of a PIN and the PEGC UE's association with the PIN may be managed dynamically using the 5GC group management feature. The AF for PIN uses the External Group ID to identiy the list of PEGC UEs that are part of a PIN. The 5GC determines the Internal Group ID from the External Group ID.

The AF for PIN may use the External Group ID to provide URSP guidance and/or QoS flow parameters to the 5GC, that is applicable for all the PEGC UEs associated with a PIN.

# * * * * End of change * * * *