3GPP WG-SA2 Meeting #156E	S2-2304426

# April 17th – 21th, 2023                                     		     	                   (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4006 | rev |  | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Member-UE selection without the NEF assistance in TS 23.502 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2022-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the result of Member Selection Terminology Discussions after SA2#155, the CR proposes to make text adjustment and use the term “Member-UE selection” for the Member-UE selection without the NEF assistance at the AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use the term “Member-UE selection” for the Member-UE selection without the NEF assistance at the A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not unified use for Member Selection Terminology. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13, Annex I (informative) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First change ***

4.15.13	5GS assistance for Member-UE selection

4.15.13.0	General

AF provides a UE list and at least one UE member filtering criterion as part of the service operation parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve the information from 5GC NFs. Before sending the list(s) of candidate UE to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the UE member filtering criteria requested by the AF.

The Member-UE selection assistance capability can be used to assist the AF to select the group of UEs to support application service (e.g. FL operation) and it is further defined in clause 5.46.2 of TS 23.501 [2].

AF leverages the 5GC network exposure for Member-UE selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how AF to leverage the 5GC network exposure for Member-UE selection can be referred to informative Annex I.

# *** Second change ***

Annex I (informative):
Member-UE selection without the NEF assistance at the AF

This informative Annex describes an example of the procedure that AF selects the FL members by collecting network exposure information in case that no NEF is present in the 5GS. In this example, QoS Monitoring is used for FL member selection.

Network exposure information as described in clause 4.15 of TS 23.502 [4], e.g. UE location reporting from the AMF, user plane information from the UPF and data analytics from NWDAF may be collected and used to assist the AF in application layer member selection e.g. assist in the selection of members participating in a federating learning operation.

Figure I-1: Example of Procedure for Member-UE selection without the NEF assistance

1.	[Optional] The AF requests the location reporting of the UEs from the AMF by invoking existing Namf_EventExposure_Subscribe (Location Reporting).

2.	[Optional] The AF initiates direct notification of QoS Monitoring procedure for delay information for the UEs in the candidate list, as defined in steps 1a-5 of clause 6.4.2.1 of TS 23.548 [10].

3.	[Optional] The AF requests user plane information, e.g. Throughput UL/DL, Packet transmission, Packet retransmission, for the UEs in the candidate list from UPF.

4. [Optional] The AF requests analytics from NWDAF by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation, such as UE Communication, User Data Congestion Analytics, WLAN performance analytics per UE, etc. as defined in TS 23.288 [50].

NOTE 1:	In Steps 1-4, the AF maintains the candidate list itself and collects the Member-UEselection information directly from the 5GC, and includes the Target UE Identifier(s) for the UEs in the candidate list in the request.

5.	The AF selects members, e.g. for application layer member selection for FL, based on the information collected in steps 1-4.

NOTE 2:	What information needs to be collected by the AF to perform Member-UEselection is decided by the AF itself.

NOTE 3:	AF may keep monitoring the information from AMF/UPF/SMF/NWDAF to update the members.

# *** End of Changes ***