3GPP TSG-WG SA2 Meeting #156e	S2-2304440

Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

Title:	[DRAFT] ReplyLS on API enhancement for GMEC services and AIMLSys services

Response to:	LS (S2-2303934/C3-230783) on API enhancement for GMEC services and AIMLSys services from CT3

Release:	Rel-18

Work Item:	GMEC, AIMLSys

Source:	[Huawei to be] SA2

To:	CT3

Cc:

Contact Person:

Name:	Qianghua Zhu

Tel. Number:

E-mail Address:	zhuqianghua AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank 5G-ACIA for their LS (S2-2303934/C3-230783) and would like to provide the following reply:

CT3 Questions:

- Whether SA2 is planning to merge these two APIs into a single one?

- What is the common part and/or difference between the Nnef_AFRequestForQoS service API and the Nnef_MultiMemberAFsessionWithQoS service API?

SA2 reply: Nnef_AFRequestForQoS service aims to allow the AF to use the External Group ID or GPSI for provisioning of the QoS parameters and QoS monitoring, the 5GC NFs, e.g. TSCTSF will manage the AF request QoS for the group, such as

-	translates the External Group ID to a list of SUPIs, and maps the SUPI to UE address for the UE member that have active PDU sessions to the group, and

-	maintains the request status for each UE member in the group, and

-	aggregates the reporting for each UE member in the group, and

-	updates the request to PCF in the case that the UE member releases or establishes the PDU Session to the group, and

-	handles the case that the UE member joins or leaves the group.

Nnef_AFRequestForQoS service aims to allow the AF to use the list of UE addresses for provisioning of the QoS parameters and QoS monitoring, the 5GC NFs do not need to support management of the AF request QoS, it rather indicates immediately to the AF whether the UE address in the list is successful or not.

The table below provides the summary for the differences between the Nnef_AFRequestForQoS service API and the Nnef_MultiMemberAFsessionWithQoS service API:

|  | GMEC: Nnef_AFRequestForQoS | AIML: Nnef_MultiMemberAFsessionWithQoS |
| --- | --- | --- |
| Target UE | - Identified by External Group ID or GPSI | - Identified by list of UE IP addresses |
| Member description | - UE identifier | - UE IP address |
| Multiple memberships | - Yes (one per PDU Session) | - Yes (multiple per PDU Session) |
| UE connectivity status | - Impact connected UEs and unconnected UEs | - Impact only connected UEs |
| Area/scope | - Regional area/factory | - Whole PLMN |
| Deployment dependency | - TSCTSF required | - None |
| NF under control of group | - TSCTSF | - NEF |
| QoS | - Both GBR and non-GBR QoS - Maximum group data rate (i.e. for all active UEs) | - Primarily Individual GBR QoS (i.e. per UE) |
| Flow description | - Communication between UEs or between UE and DN | - Communication only between UE and DN |
| Configuration update | - rare | - Frequent (wrt. current members, QoS, monitoring) |
| Specifics | - Traffic characteristics - Temporal invalidity condition (start-time, end-time) | - Consolidated bitrate monitoring |

Given the above differences, SA2 doesn’t plan to merge two APIs into a single one.

2. Actions:

To CT3 group.

ACTION: 	SA2 asks CT3 group to take the above responses into account

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #157		May 22 – 26, 2023				Berlin, DE

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE