3GPP TSG-WG SA2 Meeting #156-e	S2-2304489

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4328 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solve the EN about PINs sharing a PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As analysed in the Discussion Paper (S2-2304928), if PIN traffic uses local switching, privacy issues will be caused when multiple PINs share a single PDU session. Moreover, it is difficult to perform charging per PIN if multiple PINs share a single PDU session. In addition, the PDU session dedicated for PIN shall not be used for transmitting traffic not related to PIN. Therefore, this CR aims to clarify the PDU session for PIN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the EN and add the text to clarify that the PEGC shall establish separate PDU Session for each PIN if the PIN traffic is via PEGC/5GC (i.e. multiple PINs cannot share a PDU session). Additional description is added to clarify that traffic not related to PIN shall not be transmitted via the PDU session dedicated for PIN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Whether multiple PINs can share a single PDU session is not clear and if the multiple PINs shares the same PDU session, issues may be caused in terms of traffic transmission and charging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.44.3.1	PDU Session Establishment for PIN

When a PDU Session associated with a PIN is established by UE with PEGC, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The UE with PEGC may use IP address allocation methods as specified in clause 5.8.2.2 (e.g. IPv6 Prefix Delegation feature).

One PEGC may serve more than one PINs and in this case the PEGC shall establish separate PDU Session for each PIN if the PIN traffic is via PEGC/5GC (i.e. multiple PINs cannot share a PDU session). One PIN may be served by more than one PDU sessions in the PEGC. The PDU session dedicated for PIN traffic shall not be shared for traffic not related to PIN, for the traffic related to application residing in the PEGC which are considered not part of PIN or from device behind the PEGC which are not part of PIN (for example if the WLAN of UE is shared with other devices that are not added in any of the PIN supported by PEGC this traffic shall be transmitted via a different PDU session).

Editor's note:	The handling of the PEMC in 5GC in relation with PIN is FFS.

# * * * * End of changes * * * *