3GPP TSG-WG SA2 Meeting #156-e	S2-2304732

# Elbonia, April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4389 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on IAB Authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IABARC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the LS on IAB Authorization from RAN3 (R3-231010), there are different options of AMF behaviour when the IAB’s operation is not authorized: - Option 1: the AMF rejects the IAB-UE’s registration or de-registers the IAB-UE. - Option 2: the AMF initiates UE Context setup procedure or UE Context Modification procedure by providing IAB authorized indication with the value set to “not authorized” to the NG-RAN, but the IAB-UE is still registered. R3-230943 and R3-230944 clarify IAB Authorized IE in UE Context Modification procedure in TS 38.413 as the following: 8.3.4.2 Successful Operation Figure 8.3.4.2-1: UE context modification: successful operation Upon receipt of the UE CONTEXT MODIFICATION REQUEST message the NG-RAN node shall - if supported, store the received IAB Authorization information in the UE context. If the IAB Authorized IE is set to "not authorized" for an IAB-MT, the NG-RAN node shall, if supported, initiate actions to ensure that the IAB node will not serve any UE(s). Therefore, it is proposed to update clause 5.35.2 of TS 23.501 to align with RAN’s agreements. In addition, if the IAB operation is not authorized but the IAB-UE is still registered with the network, the PDU session for the OAM system access shall be rejected or released by the 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on AMF behaviour when the IAB’s operation is not authorized. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current descriptions of TS 23.501 are not aligned with RAN3’s agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.401(CR3728, CR3729, CR3730), TS 23.501 (CR4388, CR4390) |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.35.2	5G System enhancements to support IAB

In IAB operation, the IAB-UE interacts with the 5GC using procedures defined for UE. The IAB-node gNB-DU only interacts with the IAB-donor-CU and follows the CU/DU design defined in TS 38.401 [42].

For the IAB-UE operation, the existing UE authentication methods as defined in TS 33.501 [29] applies. Both USIM based methods and EAP based methods are allowed, and NAI based SUPIs can be used.

The following aspects are enhanced to support the IAB operation:

-	the Registration procedure as defined in clause 4.2.2.2 of TS 23.502 [3] is enhanced to indicate IAB-node's capability to the AMF;

-	The IAB-node provides an IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB and includes the IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform IAB authorization.

-	the UE Subscription data as defined in clause 5.2.3 of TS 23.502 [3] is enhanced to include the authorization information for the IAB operation;

-	Authorization procedure during the UE Registration procedure is enhanced to perform verification of IAB subscription information;

-	If the IAB operation is not authorized, the AMF may reject the IAB-UE’s registration or de-register the IAB-UE. Or the AMF may initiate UE Context setup/modification procedure by providing IAB authorized indication with the value set to “not authorized” to the NG-RAN, but the IAB-UE is still registered.

-	If the IAB operation is authorized, UE Context setup/modification procedure is enhanced to provide IAB authorized indication with the value set to “authorized” to NG-RAN.

-	When the IAB operation is changed from “authorized” to “not authorized” and the IAB-UE is still registered, the AMF notifies the SMF which serves the PDU session for OAM system access based on subscription, the SMF shall release this PDU Session.

After registered to the 5G system, the IAB-node remains in CM-CONNECTED state if the IAB operation is authorized. In the case of radio link failure, the IAB-UE uses existing UE procedure to restore the connection with the network. The IAB-UE uses Deregistration Procedure as defined in clause 4.2.2.3 of TS 23.502 [3] to disconnect from the network.

# * * * * End of changes * * * *