3GPP TSG SA-WG2 Meeting #156-e 	S2-2304745

# Online, 17-21 April 2023	(revision of)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4397 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the support for Network Slices with Network Slice Area of Service not matching deployed Tracking Areas |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | this CR clarifies some aspects of the feature: - We propose no per UE differentiation shall be considered as the network slice AoS is a slice level information and there is no requirement in the WID to restrict the availability of a slice per UE in rel-18. If this is desirable a rel-19 study should be opened on the topic and this should also apply to the per TA availability. - We propose that much in the same way as we have Registration allowed for Rejected S-NSSAI in RA within the TA if the UE knows the S-NSSAI is available in a certain cell of the TA, the UE shall perform a Mobility Registration when exiting the availability area cells for a cell in the Allowed NSSAI. Basically the availability information defines a new “logical contour “of the RA. - We assume the RAN takes care to steer the UE inside the availability area of a slice as far as possible based on the Radio Quality (based on the incoming LS from RAN3 in R3-230899 . There is no clear need of any SA2 additional effort. - The handling of the PDU sessions in the RAN should be uniform across supporting and non supporting UEs - The CN may perform special handling for non supporting UEs by removing the S-NSSAI from allowed, Partially allowed and configured NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provides the necessary normative text |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.18, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

### 5.15.18	Support for Network Slices with Network Slice Area of Service not matching deployed Tracking Areas

#### 5.15.18.1	General

The network support for a Network Slice is defined on a per Tracking Area granularity. It may be beneficial to deploy some Network Slices such that the Network Slice have a limited geographical availability that is not matching existing Tracking Area boundaries.

The operator can in this case decide to change the topology of the Tracking Areas so they match the boundaries of the Network Slice, or the operator may configure resources for the Network Slices in the cells of TAs where the Network Slices are to be available, and in areas of the TAs where the network slice is defined to be not available the cells are configured with zero resources.

The AMF receives from the OAM the information on availability of a network slice when the granularity is smaller than TA, i.e. if the NS-AoS includes TAs where the network slice is not available in some cells of the TA.

In order to optimize the end-to-end behaviour, the AMF can, based on NS-AoS information received from OAM, configure supporting UEs with S-NSSAI location availability information, and the network may need to monitor the S-NSSAI usage and take decisions on whether to release or deactivate PDU sessions and enforce the NS-AoS e.g. if the UE does not support the S-NSSAI location availability information.

#### 5.15.18.2	S-NSSAI location availability information

S-NSSAI location availability information defines additional restrictions to the usage of an S-NSSAI in TAs where the Network Slice availability does not match the TA boundaries. The AMF is configured per S-NSSAI whether to send the S-NSSAI location availability information to supporting UEs.

The S-NSSAI location availability information sent to the UE includes, for each applicable S-NSSAI of the Configured NSSAI, Location information indicating the cells of TAs in the RA where the related S-NSSAI is available if the S-NSSAI is not available in all the cells of the TA.

If the UE has indicated that the UE supports S-NSSAI location availability information in the 5GMM Core Network Capability (see clause 5.4.4a), the AMF may, based on OAM configuration, configure the UE with S-NSSAI location availability information for one or more S-NSSAIs when the AMF allocates an RA where the Network Slice availability does not match whole TAs, by including the S-NSSAI location availability information in the Registration Accept message or the UE Configuration Command message. A UE that receives S-NSSAI location availability information applies the information as follows.

1.	If the S-NSSAI is rejected in the RA or rejected partially in the RA or rejected with a cause code that allows attempting to register the S-NSSAI again, the UE can request the S-NSSAI only if the S-NSSAI location availability information indicates that the S-NSSAI is available at the cell where the UE is camping.

2.	If the S-NSSAI is in the Partially Allowed NSSAI, the UE shall not activate User Plane for any already established PDU Session with that S-NSSAI if the UE is in a cell within the RA but outside the Location information of the S-NSSAI.

3.	If the S-NSSAI is in the Partially Allowed NSSAI, and the UE moves to a cell inside the RA but outside the Location information of the S-NSSAI, the UE needs not perform a Mobility Registration update and, if the UE has an established PDU Session with that S-NSSAI, the PDU Session is kept as locally deactivated.

4.	If the S-NSSAI is in the Allowed NSSAI, and the UE moves to a cell inside the RA but outside the Location information of the S-NSSAI, the UE shall perform a Mobility Registration update and, if the UE has an established PDU Session with that S-NSSAI, the PDU Session is dropped locally.

#### 5.15.18.3	Network side handling of Network Slice Area of Service not matching deployed Tracking Areas

OAM may configure RRM policies for S-NSSAIs on a per cell basis as defined in TS 28.541 [149], i.e. cells outside the Network Slice Area of Service while in a TA supporting the S-NSSAI are allocated with no RRM resources for the S-NSSAI. The NG-RAN

The network handles the NS-AoS for an S-NSSAI as follows:

1.	The network monitors the validity of the S-NSSAI for UE in CM-CONNECTED state, i.e. as per the location. The NG-RAN can detect when the UE moves to cell where a network slice is not available and release the related PDU sessions resources while keeping the PDU session context for network slices which have S-NSSAI in the Partially Allowed NSSAI. The NG-RAN reports the updated PDU session resources to the AMF in N2 messages. 

When the User Plane Resources are activated for a PDU Session to of an S-NSSAI part of the Partially Allowed NSSAI and the UE moves to a cell outside the NS-AoS, the User Plane Resources for the PDU Session shall be deactivated as per clause 4.3.7 of TS 23.502[3], but the PDU Session context in UE, NG-RAN and SMF is not released unless requested by the AMF. The User Plane Resources for the PDU Session shall not be activated as long as the UE is located in a cell which is not part of the NS-AoS associated with the S-NSSAI of the Partially Allowed NSSAI.

When the User Plane Resources are activated for a PDU Session to of an S-NSSAI part of the Allowed NSSAI and the UE moves to a cell outside the NS-AoS, the User Plane Resources for the PDU Session shall be released by the NG-RAN and then the AMF requests the SMF to locally release in the network the related PDU Sessions with that S-NSSAI as per step 1f in clause 4.2.3.4 in TS 23.502 [3].

When the User Plane Resources are deactivated for a PDU Session of an S-NSSAI in the Partially Allowed NSSAI and the UE moves to a cell which is part of the NS-AoS associated with the S-NSSAI, the NG-RAN shall re-activate the User Plane Resources for the PDU Session and request the AMF to reactivate the PDU session.

Editor's note:	The mechanism  described above needs to be confirmed by RAN3

2.	If a non-supporting UE makes a PDU Session establishment request with an S-NSSAI that is not valid as per the S-NSSAI location availability information, the AMF may reject the NAS Transport message with a back-off timer using S-NSSAI based congestion control as described in clause 5.19.7.4.

3.	If the AMF determines that the UE in CM-CONNECTED has moved outside the NS-AoS when it receives indication from the RAN that the PDU session resources have been updated in the RAN, the AMF performs the following logic:

a)	If the non-supporting UE has other S-NSSAI(s) in the Allowed NSSAI, then the AMF may update the UE with a UE Configuration Update by removing the S-NSSAI from the Allowed NSSAI (which causes the UE to locally release the PDU Sessions) and removing the S-NSSAI from the Configured NSSAI, and then the AMF requests the SMF to locally release in the network the related PDU Sessions for this S-NSSAI as per step 1f in clause 4.2.3.4 in TS 23.502 [3].

b)	If the non-supporting UE does not have any other S-NSSAI in the Allowed NSSAI, then the AMF may update the UE with a UE Configuration Update by removing the S-NSSAI from the Allowed NSSAI (which causes the UE to locally release the PDU Sessions) and removing the S-NSSAI from the Configured NSSAI, and adding a default S-NSSAI to the Allowed NSSAI, and then the AMF requests the SMF to locally release in the network the related PDU Session for the S-NSSAI as per step 1f in clause 4.2.3.4 in TS 23.502 [3].

c)	For a non-supporting UE that does not have any other S-NSSAI in the Allowed NSSAI nor in the Configured NSSAI, then the AMF indicates to the SMF to release the PDU Session.

4.	If the AMF determines that the S-NSSAI becomes valid e.g. the UE has moved into the NS-AoS, the AMF updates the UE with a UCU e.g. including the S-NSSAI in the Configured NSSAI.

## END of CHANGES