3GPP TSG-SA WG2 Meeting #156e 	S2-2304815

April 17 – 21, 2023, e-meeting	(was)

Source:	vivo

Title:	TS 23.586: QoS handling clarification

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper provides some clarifications.

1. Introduction/Discussion

Some following aspects are clarified:

- Clarify that the current Policy/parameters for Ranging/SL positioning is on top of the V2X or 5G ProSe

- Clarify that the QoS requirement for Ranging/SL positioning (not RSPP transport) can be from the SL positioning client UE or LMF as well.

- Clarify that the RSPP layer determines the QoS parameters for Ranging/SL positioning (not RSPP transport) either based on the mapping configuration if receiving no QoS requirement or based on its derivation if receiving the QoS requirement.

- Add reference clause about the QoS handlings for RSPP transport defined in TS23.287 and TS23.304

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

## 5.1	Authorization and Provisioning for Ranging/SL positioning service

In 5GS, the parameters for Ranging/SL positioning service may be made available to the UE in following ways:

-	provisioned in the ME; or

-	configured in the UICC; or

-	provisioned in the ME and configured in the UICC; or

-	provided or updated by the Ranging/SL positioning Application Server via PCF and/or PC1 reference point; or

-	provided or updated by the PCF to the UE.

The UE shall apply any given parameter in the following priority order:

-	provided or updated by the PCF (regardless whether the parameter is determined by the PCF itself or provided by the Ranging/SL positioning Application Server to the PCF), if available, otherwise;

-	provided or updated by the Ranging/SL positioning Application Server via SR1 reference point, if available, otherwise;

-	configured in the UICC, if available, otherwise;

-	provisioned in the ME, if available.

The parameters provided or updated by the Ranging/SL positioning Application Server via SR1 reference point may need to be complemented with configuration data from other sources listed above.

NOTE:	The Ranging/SL positioning Application Server can provision the same Ranging/SL positioning parameters via 5GC or directly to the UE via SR1 reference point, and can revoke (e.g. delete) the Ranging/SL positioning parameters via 5GC in order for the provisioning via SR1 reference point to take effect.

The basic principles of service authorization and provisioning for Ranging/SL positioning service are as follows:

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorized to use Ranging/SL positioning.

-	The UE is authorized to use Ranging/SL positioning only in the PLMN(s) indicated by the PCF, and the equivalent PLMNs thereof.

-	The PCF in the HPLMN merges authorization information from home and other PLMNs and provides the UE with the final authorization information.

-	The PCF in the VPLMN or HPLMN may revoke the authorization (via H-PCF when roaming) at any time by using the UE Configuration Update procedure for transparent UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [3].

-	The Ranging/SL positioning Policy/parameters provisioning to UE is controlled by the PCF and may be triggered by UE. In addition to the Policy/parameters for V2X as defined in TS 23.287 [6] and 5G ProSe as defined in TS 23.304 [7], the PCF also provisions one or more of the following Policy/parameters:

-	When the UE is served by NG-RAN, the PLMNs in which the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	When the UE is not served by NG-RAN, indicates whether the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and Ranging/SL positioning QoS parameters (Quality of the Ranging/SL Positioning).

-	The authorized Ranging/SL positioning role per PLMN, e.g. target UE, reference UE, assistant UE, and located UE.

-	When the UE is not served by NG-RAN, indicates the authorized Ranging/SL positioning role, e.g. target UE, reference UE, assistant UE, and located UE.

Editor's note:	FFS if authorized roles for Assistant UE is needed.

## 5.7	QoS Handling

### 5.7.1	General

QoS of Ranging/SL Positioning includes 2 aspects:

-	Ranging/SL Positioning QoS, which refers to the quality of the Ranging/SL Positioning result

-	RSPP transport QoS, which refers to the quality of the PC5-U communication for transmitting RSPP traffic.

### 5.7.2	Handling of Ranging/SL Positioning QoS

Ranging/SL Positioning QoS requirement may be provided in the Ranging/SL Positioning service request, which is generated at the application layer and is provided from the application layer to the Ranging/SL Positioning layer, provided by the SL Positioning Client UE or provided by LMF.

The Ranging/SL Positioning layer maps the Ranging/SL Positioning QoS requirement to the Ranging/SL Positioning QoS parameters and provides the Ranging/SL Positioning QoS parameters to the AS layer. If there is not received Ranging/SL Positioning QoS requirement from the application layer, the Ranging/SL Positioning layer determines the Ranging/SL Positioning QoS parameters based on the Ranging/SL positioning Policy/parameters as configured in the clause of 5.1.

Ranging/SL Positioning QoS information contains attributes defined in clause 4.1b of TS 23.273 [8] with the following additions:

-	The accuracy attribute also includes the accuracy of horizontal/vertical distance and the accuracy of horizontal/vertical direction;

-	Range, which indicates the applicability of the QoS attributes in the Ranging/SL Positioning operation over PC5.

Editor's note:	Other QoS attributes are FFS, and will be aligned with RAN WGs.

5.7.3	Handling of RSPP transport QoS

The V2X/ProSe layer handles the RSPP traffic as the V2X/ProSe application data packets for the QoS treatment. QoS handling mechanism of V2X as defined in clause 5.4 of TS 23.287 [6] or QoS handling mechanism of 5G ProSe as defined in clause 5.6.1 of TS 23.304 [7] is reused.

Editor's note:	PQI values for RSPP transport is FFS.

# * * * * End of changes * * * *