3GPP SA WG2 Meeting #156E	S2-2304940

April 17 - 21, 2023, Elbonia                  	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1296 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Bootstrap Data Channel Set up Signalling Procedure Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | After terminating network negotiation (Step. 14) in bootstrap data channel set up signalling procedure, the SDP answer for terminating UE’s bootstrap DC may cause the updating of media resource in DCMF for terminating UE. In Steps. 21~24, several scenarios (Stream ID: 0, 10, 100, 110) are mixed together in the step description; it is proposed to describe Step. 21~24 separately for different scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update bootstrap data channel set up signalling procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for bootstrap data channel set up signalling is not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AC.7.1	Bootstrap Setup Signalling procedure

Figure AC.7.1-1depicts a signalling flow diagram for establishing a Bootstrap Data Chanel in a person-to-person use case. The DCMF anchors the Bootstrap Data Channel, and the originating network is offering a Bootstrap Data Channel to the remote peer as well for application download.

In the call flow the two UEs have already established an IMS audio session, and the originating UE is updating the IMS audio/video session to an IMS Data Channel session.

NOTE 1:	Some SIP signalling not relevant for the procedure, is not shown in the call flow below.

Figure AC.7.1-1: Bootstrap Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

-	Step 1: UE#1 sends the SIP INVITE request with an initial SDP to AS, through originating network P-CSCF and S-CSCF. The initial SDP contains offers for the Bootstrap Data Channel establishment requirement. In this example procedure, SDP contains both Bootstrap Data Channel offers for originating side and terminating side.

NOTE 2:	This SIP INVITE can also be a SIP re-INVITE performed after the initial IMS audio session is setup.

-	Step 2: IMS AS validates user subscription data to determine whether the Data Channel call request should be conveyed to DCSF.

If the IMS AS determined, based on the user profile, that Data Channel call request needs to be sent to DCSF, the IMS AS selects a DCSF for this user based on local configuration or discovery and selection of a DCSF instance via NRF.

If the IMS AS determined, based on the user profile, that the Data Channel call request need not sent to DCSF, or DCSF decides that DC request is not allowed, the IMS AS proceeds with normal IMS procedures to setup the MMTel session without performing Data Channel bootstrapping, by deleting DC related media information and sending the updated SIP INVITE to originating S-CSCF.

-	Step 3: IMS AS notifies the DCSF of the call event sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, CallingID, CalledID, SessionCase, SessionID, MediaInfoList) Notify request to the DCSF.

-	Step 4: After receiving the DC control request, the DCSF determines the policy about how to process the Bootstrap Data Channel establishment requirement based on the related parameters in the Data Channel control request and/or operator strategy.

-	Step 5: Since the SessionEstablishmentRequestEvent indicates that served user is offered local bootstrap media, DCSF, based on its policies prepares originating side MDC1 media information, as well as the terminating side MDC1 remote bootstrap media (targeting remote UE).

-	Step 6: DCSF invokes the Nimsas_MediaControl_MediaInstruction (SessionID, MediaInstructionSet) operation based on its policies instructing the IMS AS how to set up bootstrap data channel with DCMF both for originating and terminating side.

-	Step 7: The IMS AS selects a DCMF based on local configuration or discovers and selects a DCMF instance or MRF supporting DC media function via NRF.

-	Step 8:	IMS AS invokes DCMF service to instruct DCMF to allocate a Mb resource to terminate the media descriptor specified by the DCSF for the originating side. For IMS AS to stand alone DCMF, DC1 is used to reserve data channel media resources reserved based on DCSF media information created in step 5. If MRF is used, IMS AS uses Mr'/Cr to the MRF to reserve data channel media resources.

-	Step 9: IMS AS invokes DCMF service to instruct DCMF to allocate a Mb resource to terminate the media descriptor specified by the DCSF for the terminating side. For IMS AS to stand alone DCMF, DC2 is used to reserve data channel media resources based on DCSF received in step 5. If MRF is used, IMS AS uses Mr'/Cr to the MRF to reserve data channel media resources.

NOTE 3:	The DCMF media resource allocation could be done with one or multiple service invocations.

-	Step 10: IMS AS responds to the MediaInstruction request received in step 6. The response may include the atomic success of operation received by the IMS AS and includes also media resource information.

-	Step 11: The DCSF stores the media resource information and responds to the Notify Request received in step 3.

-	Steps 12-14: IMS AS sends the INVITE which includes the updated SDP offer adding media information of DCMF or MRF via the originating S-CSCF to remote network side and UE#2. The SDP offer for bootstrap data channel to UE#2 will in this case be included.

-	Steps 15:	UE#2 and terminating network returns an 18X response with SDP answer to bootstrap DC to originating network. If the terminating network decides to establish another bootstrap DC to UE#1 and UE#2, the corresponding data channel media description is also included in SDP answer.

According to the received SDP answer, DCMF or MRF may update data channel media resource for UE#2.

-	Step 16-17: UE#2 and terminating network returns a 200 OK response.

-	Step 18: IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, SessionID, MediaInfoList) to DCSF.

-	Step 19: The DCSF responds to the notification.

-	Steps 20: 200 OK forwarded to UE#1.

-	Step 21: For UE#1’s bootstrap data channel SDP with Stream ID 0 or 10, the bootstrap data channel has been established between originating DCMF or MRF and UE#1. The UE#1sends application request messages to DCMF or MRF via bootstrap data channel with its data channel capabilities. The originating DCMF or MRF forwards the message to received media point of DCSF. The DCSF provides the proper data channel applications to UE#1 based on their data channel capabilities through DCMF or MRF.

-	Step 22: For UE#2’s bootstrap data channel SDP with Stream ID 100 or 110, the bootstrap data channel has been established between originating DCMF or MRF and UE#2. The data channel application is requested and downloaded to UE#2 from originating DCSF.

-	Step 23: For UE#1’s bootstrap data channel SDP with Stream ID 100 or 110, the bootstrap data channel has been established between terminating DCMF or MRF and UE#1. The data channel application is requested and downloaded to UE#1 from terminating DCSF.

-	Step 24: For UE#2’s bootstrap data channel SDP with Stream ID 0 or 10, the bootstrap data channel has been established between terminating DCMF or MRF and UE#2. The data channel application is requested and downloaded to UE#2 from terminating DCSF.

NOTE 4: If multiple DC applications are available, UE may request a DC applications list after bootstrap data channels establishment.

END OF CHANGES