3GPP TSG-SA2 Meeting # 156E	S2-2305266

# Elbonia, April 17-21, 2023															 (revision of S2-XXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2099 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Differentiated service for NAUN3 devices connected behind a 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, Differentiated QoS and charging for NUAN3 devices behind 5G-RG is not supported. This CR proposes QoS and charging differentatiation for UE(s) behind 5G-RG using the following - Grouping NUAN3 known as “connectivity group” and providing QoS and charging to the group based on URSP rules. - URSP enhancement to provide “connectivity group” rules to the 5G-RG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Providing QoS and charging differentiation for NAUN3 devices behind 5G-RG and URSP enhancements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This feature as defined in the conclusion is not currently supported in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.316 CR 2099. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 4.10b	Differentiated services for NAUN3 devices behind 5G-RG

NAUN3 devices are locally authenticated by the 5G-RG using pre-shared secret etc., These devices are not visible to the 5G network though 5G-RG is subscribed to the 5G-Core. Differentiated QoS servivces for NAUN3 devices are provided using local configuration in the 5G-RG and based on DNN/S-NSSAI for the 5G RG’s PDU sessions.

5G-RG can set up distinct PDU session for each NAUN3 device or a group of NAUN3 devices based on the local configuration in the 5G-RG. For eg., consider admin settings on the 5G-RG where admin can group NAUN3 devices based on mac addresses and set different QoS levels/priority levels for a group of NAUN3 devices. Based on these local configuration in the 5G-RG can provide distinct PDU session for each NAUN3 device. When these devices have common QoS sessions and DNN/S-NSSAI to be served then, these devices are classifed into “connectivity groups” where each group connects to the 5G-RG using a separate physical or virtual port. These ports could - for eg., refer to a separate physical ethernet ports and/or a separate WLAN SSIDs and the groups may also be separted on the LAN behind the RG using VLANs.

Each connectivty group is then mapped to a separate PDU sesssion that is established by 5G-RG based on existing procedures defined in TS 23.316 [5]. The overall architecture is illustrated in Figure-4.10b-1.

Figure 4.10b-1: Example scenario for NAUN3 devices behind 5G-RG based on connectivity groups

The 5G-RG is configured with the (virtual) port information (e.g. VLANs and SSIDs) based on TR-69, TR-360 and TR-181. URSP rules can be provided to the RG to indicate what PDU Sessions to establish (DNN, S-NSSAI etc). Alternatively, the 5GRG can be manually configured by the admin to form the connectivity groups.

The mapping between virtual ports and DNN/S-NSSAI can be configured using URSP and TR-69/TR-181. In case of URSP, VLAN IDs and Connection Capabilities can be used as Traffic Descriptors, provided that the 5G-RG has a corresponding configuration of virtual ports with the same VLAN IDs or Connection Capabilities.

In addition, it is assumed that the mapping between a "virtual port" and DNN/S-NSSAI can be configured via TR-69/TR-181.

The NAUN3 devices are manually configured to use a specific SSID or connect to a certain Ethernet port on the 5G-RG. No new 5GS mechanism is added to enforce or authorize a specific non-3GPP device to access a certain SSID or port on the RG. It is assumed that the end-users of the local network provisions only authorized devices with the required Wi-Fi credentials and ensures that only authorized devices connect to physical ports. MAC address filters may be installed on the 5G-RG for each logical port to ensure that only devices with certain MAC addresses can connect to each logical port on the 5G-RG.

The connectivity groups are fully separated from each other and differentiation of charging and QoS is provided via existing mechanisms. The solution also allows isolation of the connectivity groups into separate network slices, i.e. with separate N-SSAIs for each PDU Session. Differentiated charging for sectors/departments can be enabled by assigning separate DNNs/N-SSAIs to them.

The procedures for 5G-RG connecting via W-5GAN and/or NG-RAN are captured in TS 23.316 [5].

* * * End of Changes * * *