3GPP TSG-SA2 Meeting #156E	S2-2305346

# Elbonia, April 17 – 21, 2023												 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4556 | rev | 0 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.501 – General Clause Update for AIMLsys |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The intent of this CR is to update the general description clause for AIMLsys. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating the general descriptions to reflect the overall functionalities of AIMLsys. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of general descriptions to describe the AIMLsys overall functionalities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.46.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y | X | Other core specifications |  |  |  | TS/TR. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5.46	Assistance to AI/ML Operations in the Application Layer

### 5.46.1	General

This clause describes the list of 5GC enablers to support three types of AI/ML operations:

- AI/ML operation splitting between AI/ML endpoints

-	AI/ML model/data distribution

-	Distributed/Federated Learning

NOTE:	Requirements on AI/ML operations are specified in presents an overview on how the 5G System provides assistance to support AI/ML operations in the application layer based on service requirements specified in clause 6.40 of TS 22.261 [2].

The AF may subscribe to NEF event as described in clause 4.15.3.2.3 in TS 23.502 [3] to navigate its application AI/ML operation. For example, for non-real time AI/ML operation (e.g. Distributed/Federated Learning operation) the AF may subscribe to session inactivity time monitoring event to assist the AI/ML application server in scheduling UE(s) to participate in the AI/ML operation.

The AF that aims to initiate the AI/ML operation, e.g. Distributed/Federated Learning, may request the 5GC  (e.g. via NWDAF, SMF, UPF etc.) to determine the network and/or resource performance statistic and/or prediction corresponding to one or more UEs at the target AOI.  The AF may subscribe to the NEF events to be notified about the subset list of UEs that fulfil certain filtering criteria (e.g. QoS, access type, historical location etc.) as described in clause 5.46.2 and clause 4.15.13 of TS 23.502 [3].  Such list of UEs may become the list of candidate UEs to be considered for participating in the AI/ML operation depending on the AF internal policies. Alternatively, the AF may select the list of candidate UEs for the AI/ML operation e.g. Distributed/Federated Learning, without NEF involvement as described in Annex I of TS 23.502 [3].   The AF may also determine a list of UEs without any assistance from the 5GC and proceed directly to initiate the AI/ML operation.

The AF that selects a list of UEs may request the network to negotiate and to provide the preferred time window(s) for the AI/ML operation using the Planned Data Transfer with QoS (PDTQ) requirements as described in TS 23.502 [3] and TS 23.503 [45].

At the time and during the AI/ML Operation, e.g. Distributed/Federated Learning, the AF may request 5GC to provision the QoS resource allocation for the list of candidate UEs identified by their respective UE IP addresses. The AF may also subscribe to individual or aggregated QoS flows Monitoring as described in clause 5.45 and in clause 4.15.6.13 of TS 23.502 [3] for those AF requests with successful QoS resource allocation. The AF specifies the QoS parameters according to the performance requirements listed in clause 7.10 of TS 22.261 [2].

The AF may be notified about changes in the subset list of candidate UEs, as such the AF may request a new preferred time window for the AI/ML operation via PDTQ as described in clause 4.16.15 of TS 23.502 [3]. The AF may request the NEF to provide QoS for the updated list of candidate UEs, each identified by UE IP addresses, that may result in QoS resources previously allocated to some UE IP addresses to be released, while other UE IP addresses may require allocation of QoS resource and QoS monitoring as described in clause 5.45 and in clause 4.15.6.13 of TS 23.502 [3].

In addition, the following fundamental principles apply when 5GS assists the AI/ML operation at the application layer:

-	AF requests to the 5G System in the context of 5GS assistance to AI/ML operations in the application layer shall be authorized by the 5GC using existing mechanisms.

NOTE:	In this Release, assistance to AI/ML operations in the application layer is not supported to roaming UEs.

-	UPF can perform traffic detection for AI/ML traffic as defined in clause 5.8.2.4 and the network needs to support charging for AI/ML traffic.

Editor's note:	How the network and/or AF can differentiate AI/ML traffic is FFS.

-	Application AI/ML decisions and their internal operation logic reside at the AF and UE application client and is out of scope of 3GPP.

-	Based on application logic, it is the application decision whether to request assistance from 5GC e.g. for the purpose of selection of UEs that participate in certain AI/ML operation.

-	In this Release, 5GS assistance to support AI/ML operations in the application layer, is applicable to AI/ML operations in the application layer conducted within a single slice.

* * * End of Changes * * *