3GPP TSG-SA WG2 Meeting #156E	S2-2305445

# Elbonia, April 17 – 21, 2023,													   (revision of S2-2304616r02)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4040 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys: KI#7 – additional information provided by NEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As the text agreed below in Clase 5.46.2 of TS 23.501, NEF optionally provides additional information in addition to the one or more lists of candidate UE(s) - Providing the AF with the UE member selection assistance information, including one or more lists of candidate UE(s), and optionally other additional information (e.g., one or more recommended time window(s) for performing the application operation, QoS of each target UE, UE(s) location, Access/RAT type). The AF may use the additional information to make the final selection of UEs among the one or more lists of candidate UE(s) for the application operation (e.g., FL). The additional information could be the values that the candidate UEs that fulfill the criteria received from the AF. The recommended time window(s) could be the time window(s) provided by the AF that the candidate UEs can fulfil the criteria received from the AF. The NEF may provide the different lists of candidate UE(s) according to the different time windows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Optional Input parameters are added to the UEMembmerSelectionAssistance_Notify service operation to provide additional information for member selection functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide additional information to support the member selection functionality to assist the Application AI/ML operation as described in TS 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.1, 5.2.6.32.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.15.13.1	UE member selection general information flow

This clause describes the procedures that are generally applicable independently of the UE member filtering criteria sent by the AF.

Figure 4.15.13.1-1: 5GC assistane to UE Member selection

1.	AF subscribes the member selection assistance functionality by sending Nnef_UEMemberSelectionAssistance_subscribe request including a list of target UE(s), one or more UE member filtering criteria listed in the Table 4.15.13.2-1, and optionally, time window(s).

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executes the corresponding service operation based on the UE member filtering criteria provided by the AF, e.g. events, analytics and/or notifications.

3.	NEF interacts with different 5GC network functions to collect the required information. The set of interactions between NEF and among 5GC NFs are dependent on the UE member filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, NEF consolidates all the information collected from other 5GC NFs to derive the list(s) of candidate UE(s) which fulfill the UE member filtering criteria in the AF request.

5.	NEF sends a Nnef_UEMemberSelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and possibly additional information. See Clause 5.2.6.32.4 for details.

* * * * Start of 2nd Change * * * *

5.2.6.32.4	Nnef_UEMemberSelectionAssistance_Notify service operation

Service operation name: Nnef_UEMemberSelectionAssistance_Notify

Description: NEF reports the UE member selection assistance information to the consumer that has previously subscribed.

Inputs, Required: Notification Correlation Information, one or more list(s) of candidate UE(s).

Inputs, Optional: time window received from the AF per list of candidate UE(s), specific value of the parameters that NEF gathered for the member UE filtering criteria per candidate.

Outputs, Required: Operation execution result indication.

* * * * End of Changes * * * *