3GPP TSG-SA2 Meeting #156E(e-meeting)	S2-2305598

# April 17 - 21, 2023, Electronic Meeting										 (revision of S2-2305070)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0357 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the Editor’s Notes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the discussion in S2-2305064, the following Editor’s Note can be removed simply without any update to TS 23.273: - Clause 4.3.8: Editor's note: Whether and how AF provides PRU information (e.g. known location) on behalf of a PRU to network is FFS. - Clause 4.3.11: the static/mobile type of PRU in the EN. Editor's note: It is FFS whether PRU static/mobile type and ON/OFF state is needed in PRU information. - Clause 6.17.4: Editor's note: The procedure may be further updated based on RAN WG feedback regarding the support of simultaneous measurements of PRU(s) and target UE. This CR is proposed to remove the Editor’s Notes above simply. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Editor’s Note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not Solved Editor’s Note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.8, 4.3.11, 6.17.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type and indication of requiring reliable UE location information.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

-	Determine whether to use user plane or control plane for positioning.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF.

Editor's note:	LMF functionality of user plan connection handling will be updated according to the protocol stack decision made by CT WG1 and SA WG3 groups.

-	Support collection of GNSS assistance data from AFs.

-	Support service level PRU Association, PRU Association update or PRU Disassociation.

-	LMF supports verification of a PRU initiated Association or Disassociation by checking whether there is an PRU verified indication from AMF.

-	LMF stores the received PRU information contained in service level PRU Association message.

-	LMF may indicate support of PRU function to NRF via NF profile and may further send the PRU information to NRF via NF profile update.

-	LMF may request a PRU to associate to a new LMF by returning a Routing ID of the new LMF.

-	Support selection of a PRU based on stored PRU information if the LMF needs to obtain the location measurements from the PRU to assist positioning of a target UE.

-	Support to obtain PRU location measurements as described in clause 5.4.5 of TS 38.305 [9] by triggering the procedure in clause 6.11.

-	Support to obtain PRU location measurements from other PRU serving LMF(s).

-	As a serving LMF of target UE(s), support discovery and selection of other PRU serving LMF(s) by querying the NRF and support to request PRU location measurements from the selected LMF(s).

-	As a serving LMF of PRU(s), support to provide PRU location measurements to other LMF(s) after receiving a request from other LMF(s).

-	Support to determine UE location by considering obtained PRU location measurements.

NOTE 2:	Country, area within a country, or an international area can be supported as different types of geographical area.

-	Support a request for user plane reporting from a UE to an LCS Client or AF for a periodic or triggered 5GC-MT-LR. Subsequently, support the transfer of cumulative event reports from the target UE via control plane back to the H-GMLC and LCS Client or AF. Also support any request for assistance data received in a cumulative event report.

-	Determine UE location for a UE connecting to a MBSR based on location and velocity of the MBSR and the timing of the location estimations for the target UE and MBSR.

* * * Second Change * * *

4.3.11	Positioning Reference Unit, PRU

A UE may support the functions of a PRU. The PRU supports the following functions including functions defined in TS 38.305 [9]:

-	Support service level association, association update and disassociation with a serving LMF.

-	The PRU sends service level association, association update or disassociation to LMF via LCS supplementary service message.

-	Support association with multiple LMFs. e.g. for the case a PRU is in multiple LMF overlapped serving areas.

-	The PRU information included in a PRU association or PRU association update contains one or more than one of the following aspects:

-	PRU Positioning Capabilities.

-	Location information if known.

Editor's note:	It is FFS whether PRU ON/OFF state is needed in PRU information.

* * * Third Change * * *

6.17.4	Positioning of a target UE

Figure 6.17.4-1 shows a procedure used by a serving LMF for a target UE to obtain a location of the target UE using location information provided by one or more PRUs.

Figure 6.17.4-1: Location of a target UE using PRUs

1.	The serving LMF for the target UE and other PRU serving LMFs may use the procedures defined in clause 6.11 to obtain location information from one or more PRUs associated in the serving LMF and in the other PRU serving LMFs that is not related to the target UE. For example, the location information may include location information for the PRU(s) or for the NG-RAN or both.

2.	The serving LMF for the target UE receives a location request from the serving AMF for the target UE. The location request may be included in an Nlmf_Location_DetermineLocation Request service operation for a 5GC-MO-LR, 5GC-MT-LR or 5GC-NI-LR for the target UE. Alternatively, the location request may be implied by receipt of an Namf_Communication_N1MessageNotify service operation carrying a supplementary services event report from the target UE for a periodic or triggered 5GC-MT-LR.

3.	The serving LMF uses the procedures defined in clause 6.11 to obtain location information for the target UE from the target UE and/or from the NG-RAN. During the procedures, the LMF decides to use PRUs to improve the positioning result.

4.	The serving LMF selects one or more PRUs associated with the serving LMF to assist in locating the target UE. The selected PRU(s) may be nearby to an initial location estimate for the target UE obtained at step 3 or indicated by a serving cell identifier for the target UE received at step 2.

NOTE 1:	The PRU selection criteria are implementation specific and may be based on operator policies.

5.	The serving LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF. The service operation includes a PRU indication and an area which could be a list of cells or TAs decided by the serving LMF of the target UE based on the serving cell of the target UE.

6.	If step 5 is performed, the NRF selects one or more other PRU serving LMFs based on the PRU indication and the area received in step 5 and sends an Nnrf_NFDiscovery Response to the serving LMF of target UE. The service operation includes the profiles of the other PRU serving LMFs selected by the NRF. Each profile may include PRU information, e.g. a PRU identifier, PRU location.

7.	If steps 5 and 6 are performed, the serving LMF of the target UE may send an Nlmf_Location_MeasurementData Request service operation to one or more of the other PRU serving LMFs indicated at step 6. The service operation for each of the other PRU serving LMFs includes target UE cell ID or one or more PRU identifiers received at step 6 for this LMF, and an indication of location measurements requested from each PRU.

8.	The serving LMF uses the procedure defined in clause 6.11.1 to obtain location information related to the target UE from the PRU(s) selected at step 4.

Editor's note:	Location information for a target UE obtained from a PRU needs to be verified by RAN.

9.	If steps 5-7 are performed and if PRU information is included in the PRU serving LMF profile sent by the NRF to the target UE serving LMF, each of the other PRU serving LMFs for step 7 uses the procedure defined in clause 6.11.1 to obtain the location measurements requested at step 7 from each of the PRUs identified at step 7 for this LMF.

If steps 5-7 are performed and if PRU information is not included in the PRU serving LMF profile sent by the NRF to the target UE serving LMF, each of the other PRU serving LMFs for step 7 selects one or more PRUs based on the locally associated PRU information and information in the location measurements requested (e.g. the target UE cell ID), and uses the procedure defined in clause 6.11.1 to obtain the location measurements requested at step 7 from each of the selected PRUs.

NOTE 2:	Steps 3, 8 and 9 can be performed in any order, including simultaneously. A common scheduled location time may be used if the LMF determines that simultaneous measurements for UE and PRU(s) are desirable.

10.	If step 9 is performed, each of the other PRU serving LMFs for step 9 returns the location measurements obtained from PRUs at step 9 to the serving LMF for the target UE.

11.	The serving LMF for the target UE determines the location of the target UE based on the location information obtained at step 1 (if step 1 is performed), step 3, step 8 and step 10.

12a.	If an Nlmf_Location_DetermineLocation Request service operation for a 5GC-MO-LR, 5GC-MT-LR or 5GC-NI-LR was received at step 2, the serving LMF returns the location estimate of the target UE to the serving AMF.

12b.	If an Namf_Communication_N1MessageNotify service operation carrying a supplementary services event report from the target UE for a periodic or triggered 5GC-MT-LR was received at step 2, the serving LMF sends an event report for the target UE to a GMLC with the location estimate obtained at step 11 as described in clause 6.3.1.

* * * End of Changes * * *