3GPP TSG-SA2 Meeting # 156E (e-meeting)	S2-2306071

# Elbonia, April 17th -21st , 2023	(Revision of S2-2304553r02)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0767 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New parameter introduced in Contents of ML Model Provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the agreed CRs from SA2#155 meeting S2-2303885 and S2-2303546, the Nnwdaf_MLModelProvision service needs further enchancement to support MTLF-based ML model accuracy monitoring procedure and the general procedure for Federated Learning among Multiple NWDAFs. Changes proposed in this CR updates to Contents of ML Model Provisioning to include the ML model Monitoring information. The corresponding changes required to align the terminology for ML model Monitoring and step 0 in general procedure for Federated Learning among Multiple NWDAFs is covered in CR S2-2304551. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to Contents of ML Model Provisioning clause to include the ML model Monitoring information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2, 7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 6.2A.2	Contents of ML Model Provisioning

The consumers of the ML model provisioning services (i.e. an NWDAF containing AnLF) as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	Information of the analytics for which the requested ML model is to be used, including:

-	A list of Analytics ID(s): identifies the analytics for which the ML model is used.

-	NF consumer information: identifies the vendor of NWDAF containing AnLF.

NOTE 1:	NF consumer information such as Vendor ID is defined in Stage 3.

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML model ML model.

NOTE 2:	The NWDAF containing MTLF can use the parameter "Use case context" to select the most relevant ML model, when several ML models are available for the requested Analytics ID(s). The values of this parameter are not standardized.

-	[OPTIONAL] ML Model Interoperability Information. This is vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.

-	[OPTIONAL] ML Model Filter Information: enables to select which ML model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML model is requested, e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	[OPTIONAL] Requested representative ratio: a minimum percentage of UEs in the group whose data is a non-empty set and can be used in the model training when the Target of ML Model Reporting is a group of UEs.

-	ML Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	[OPTIONAL] Inference Input Data information: contains information about various settings that are expected to be used by AnLF during inferences such as:

-	the "Input Data" that are expected be used, each of them optionally accompanied by metrics that show the granularity with which this data will be used (i.e., a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data).

NOTE 3:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that are expected to be used as a list of NF instance (or NF set) identifiers.

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

Editor's note:	It is FFS how to convey degradation information for an ML model.

-	[OPTIONAL] Indication of supporting multiple ML models.

-	[OPTIONAL] Accuracy level(s) of Interest.

-	[OPTIONAL] Number of ML model(s), indicating the maxmium number of Multiple ML models the ML Model provider e.g. NWDAF(MTLF) could provide to the consumers of the ML model(s).

NOTE 4:	Multiple ML models Filter Information are composed by Indication of supporting multiple ML models, Accuracy level(s) of Interest, Number of ML model(s).

- [OPTIONAL] ML Model Monitoring Information: This is information provided to the NWDAF containing MTLF which may include ML Model metric (e.g., ML Model Accuracy, Accuracy reporting interval), pre-determined status (ML Model Accuracy threshold, Total training time).

The NWDAF containing MTLF provides to the consumer of the ML model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	For each Analytics ID requested by the service consumer, a set of pair (s) of unique ML Model identifier and the ML model Information.

ML Model Information, which includes:

-	the ML model file address (e.g. URL or FQDN).

-	[OPTIONAL] Validity period: indicates time period when the provided ML Model Information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

-	[OPTIONAL] ML model representative ratio: indicating the percentage of UEs in the group whose data is used in the ML model training when the Target of ML Model Reporting is a group of UEs.

-	[OPTIONAL] Training Input Data Information: contains information about various settings that have been used by MTLF during training, such as:

-	the "Input Data" that have been used, each of them optionally accompanied by metrics that show the granularity with which this data has been used (i.e., a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data).

NOTE 5:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that have been used as a list of NF instance (or NF set) identifiers.

NOTE 6:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

* * * Next Changes * * *

### 7.5.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe.

Description: Subscribes to NWDAF ML model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model subscription), ML Model Filter Information to indicate the conditions for which ML model for the analytics is requested and Target of ML Model Reporting to indicate the object(s) for which ML model is requested (e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs)), Requested representative ratio, ML Model Reporting Information (including e.g. ML Model Target Period), Expiry time, Use case context, Inference Input Data information, indication of support for multiple ML models, multiple ML models Filter Information to indicate the conditions for which multiple ML models are requested, ML Model Interoperability Information, ML Model Monitoring Information.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

* * * End of Changes * * *