3GPP TSG-SA2 Meeting #156-e	S2-2306253

# Online, , 17th Apr 2023 – 21st Apr 2023												     	(was S2-2304070)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0947 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Forwarding of UE reporting of URSP rule enforcement between SM-PCF and UE-PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom, InterDigital Inc., Samsung, Huawei, NEC, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#155, the support of UE reporting of URSP rule enforcement for deployments where the PCF serving the PDU session is not the same as the PCF serving the UE was left FFS. Furthermore, concerns were raised regarding using parameter provisioning, with some views expressed that reusing the procedure already present for application detection would be preferred Unclear whether this information reaches the H-PCF in roaming cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of the PCF event exposure with the same characteristics as application detection to PCF Event exposire to 6.1.3.8. Added a NOTE for clarification stating that although the same subscription is used, the events are different. For app start/stop, it is currently realized with APP_START and APP_STARTin the enumeration AppDetectionNotifType. The NOTE case already exists for the associated PCRT. - Added mention to sections 5.3.12, 6.6.2.4, as well as reference to TS 23.502 procedure - Merged content from S2-2304528, S2-2305007, S2-2305207, S2-2305316, S2-2305291 - Use of a separate event for PCF event reporting - NOTE mentioning that this feature does involve additional signaling for the associated Ues and URSP rules. - Enable the support of reporting URSP rule enforcement based on a matched URSP rule that includes Connection Capabilities in the TD and an indication to report URSP rule enforcement. - Policy control decisions based on awareness of URSP rule enforcement - LBO Roaming case where the PCF for Session is in VPLMN and the PCF for the UE is in HPLMN - Mention that other PCC input can also be used - Added example of usage in Annex A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality in deployments where the PCF serving the PDU session is not the same as the PCF serving the UE. Incomplete functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.12, 6.1.3.18, 6.1.6.1, 6.1.6.2, 6.1.6.3, 6.6.2.1, 6.6.2.4, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 23.502 CR 3959 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R01: - Revision of S2-2304070r17 - +reinstate EN ' Editor's note: the description on PCC rule generation based on pre-configured URSP rules is FFS’ |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

### 5.3.12	Interactions between PCF for a UE and PCF for a PDU Session

Npcf services enable reporting of PDU Session related events detected by the PCF for a PDU Session to the PCF for a UE. Such events are reporting the start and stop of application traffic detection and UE reporting of URSP rule enforcement.

* * * Next Change * * *

#### 6.1.3.18	Event reporting from the PCF

The AF may subscribe/unsubscribe to notifications of events from the PCF for the PDU Session to which the AF session is bound. The AF can either subscribe/unsubscribe directly at the PCF or indirectly via an NEF or a TSCTSF.

The PCF for the UE may subscribe/unsubscribe to notifications of events from the PCF for the PDU Session of a UE. Other NFs may subscribe/unsubscribe to notifications of events from the PCF for a PDU Session or for a UE.

The events that can be subscribed by the AF and by other NFs are listed in Table 6.1.3.18-1.

# Table 6.1.3.18-1: Events relevant for reporting from the PCF

| Event | Description | NF that can subscribe for reporting | Availability for Rx PDU Session (NOTE 2) | Availability for N5 per PDU Session | Availability for Bulk Subscription (NOTE 1) | Availability for N43 per SUPI, DNN, S-NSSAI | Availability for N5 per UE (NOTE 6) | Availability for N24 per UE (NOTE 6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PLMN Identifier Notification (NOTE 5) | The PLMN identifier or SNPN identifier where the UE is currently located. | AF, PCF | Yes | Yes | Yes | No | No | Yes |
| Change of Access Type | The Access Type and, if applicable, the RAT Type of the PDU Session has changed. | AF | Yes | Yes | Yes | No | No | No |
| EPS fallback | EPS fallback is initiated | AF | Yes | Yes | No | No | No | No |
| Signalling path status | The status of the resources related to the signalling traffic of the AF session. | AF | Yes | Yes | No | No | No | No |
| Access Network Charging Correlation Information | The Access Network Charging Correlation Information of the resources allocated for the AF session. | AF | Yes | Yes | No | No | No | No |
| Access Network Information Notification | The user location and/or timezone when the PDU Session has changed in relation to the AF session. | AF | Yes | Yes | No | No | No | No |
| Reporting Usage for Sponsored Data Connectivity | The usage threshold provided by the AF has been reached; or the AF session is terminated. | AF | Yes | Yes | No | No | No | No |
| Service Data Flow deactivation | The resources related to the AF session are released. | AF | Yes | Yes | No | No | No | No |
| Resource allocation outcome | The outcome of the resource allocation related to the AF session. | AF | Yes | Yes | No | No | No | No |
| QoS targets can no longer (or can again) be fulfilled | The QoS targets can no longer (or can again) be fulfilled by the network for (a part of) the AF session. | AF | No | Yes | No | No | No | No |
| QoS Monitoring parameters | The QoS Monitoring parameter(s) (as defined in clause 5.45 of TS 23.501 [2], e.g. UL packet delay, DL packet delay or round trip packet delay) are reported to the AF according to the QoS Monitoring reports received from the SMF. | AF | No | Yes | No | No | No | No |
| Packet Delay Variation monitoring parameters | The Packet Delay Variation monitoring parameter(s) (the variation of UL/DL packet delay between UE and PSA UPF) reported to the AF. | AF | No | Yes |  |  |  |  |
| Out of credit | Credit is no longer available. | AF | Yes | Yes | No | No | No | No |
| Reallocation of credit | Credit has been reallocated after the former Out of credit indication. | AF | Yes | Yes | No | No | No | No |
| 5GS Bridge/Router information Notification (NOTE 3) | 5GS Bridge/Router information that the PCF has received from the SMF. | TSN AF, TSCTSF | No | Yes | No | No | No | No |
| Notification on outcome of service area coverage change | The outcome of the request of service area coverage change. | AF | No | No | Yes | No | Yes | No |
| Notification on outcome of UE Policies delivery | The outcome of the request for UE policies delivery due to service specific parameter provisioning procedure. | AF | No | No | No | No | No | Yes |
| Start of application traffic detection and, Stop of application traffic detection | The start or the stop of application traffic has been detected. | PCF, AF | No | No | Yes | Yes (NOTE 4) | No | No |
| UE reporting Connection Capabilities from associated URSP rule | The Connection Capability received from the UE during PDU Session Establishment or Modification, see clause 6.6.2.4. | PCF | No | No | No | Yes | No | No |
| Satellite backhaul category change | The backhaul has changed between different satellite backhaul categories (e.g. GEO, MEO, LEO, OTHERSAT), or the backhaul has changed between satellite backhaul and non-satellite backhaul. | AF | No | Yes | Yes | No | No | No |
| Change of PDUID | The PDUID assigned to a UE has changed. | 5G DDNMF | No | No | No | No | Yes | No |
| SM Policy Association established or terminated | The establishment or termination of a SM Policy Association is reported | PCF | No | No | No | Yes (NOTE 7) | No | No |
| Reporting of extra UE addresses | Reporting of the extra IP addresses or address ranges allocated for the given PDU Session resulting from framed routes or Ipv6 prefix delegation. | TSCTSF | No | Yes | No | No | No |  |
| Notification on BAT offset | The PCF reports the BAT offset and optionally the adjusted periodicity that has been received from the SMF | TSCTSF | No | Yes | No | No | No |  |
| NOTE 1: Additional parameters for the subscription as well as reporting related to these events are described in TS 23.502 [3]. NOTE 2: Applicability of Rx is described in Annex C. NOTE 3: 5GS Bridge/Router information is described in clause 6.1.3.5. NOTE 4: Bulk subscription is implicit. NOTE 1 does not apply. NOTE 5: For a PDU Session established over a SNPN, the combination of the PLMN id and the NID identifies the SNPN. NOTE 6: This column contains also UE context related events that are reported to other consumers such as 5G DDNMF via other reference points than N5. The Conditions for reporting column indicates the respective consumer. NOTE 7: This PCF for the UE subscribes to this Event via AMF and SMF. |  |  |  |  |  |  |  |  |

If an AF requests the PCF to report the PLMN identifier where the UE is currently located, then the PCF shall provide the PLMN identifier or the SNPN identifier to the AF if available. Otherwise, the PCF shall provision the corresponding PCC rules, and the Policy Control Request Trigger to report PLMN change to the SMF. The PCF shall, upon receiving the PLMN identifier or the SNPN identifier from the SMF forward this information to the AF, including the PLMN Id and if available the NID. If the H-PCF requests to report the PLMN identifier where the UE is currently located, the V-PCF provisions the PCRT on "PLMN change" to the AMF as described in clause 6.1.2.5 and then forwards the PLMN ID received from the AMF to the H-PCF.

If an AF requests the PCF to report on the change of Access Type, the PCF shall provide the corresponding Policy Control Request Trigger to the SMF to enable the report of the Change in Access Type to the PCF. The PCF shall, upon reception of information about the Access Type the user is currently using and upon indication of change of Access Type, notify the AF on changes of the Access Type and forward the information received from the SMF to the AF. The change of the RAT Type shall also be reported to the AF, even if the Access Type is unchanged. For MA PDU Session the Access Type information may include two Access Type information that the user is currently using.

If an AF requests the PCF to report on the signalling path status, for the AF session, the PCF shall, upon indication of removal of PCC Rules identifying signalling traffic from the SMF report it to the AF.

If an AF requests the PCF to report Access Network Charging Correlation Information, the PCF shall provide to the AF the Access Network Charging Correlation Information, which allows to identify the usage reports that include measurements for the Service Data Flow(s), once the Access Network Charging Correlation Information is known at the PCF.

If an AF requests the PCF to report Access Network Information (i.e. the User Location Report and/or the UE Timezone Report) at AF session establishment, modification or termination, the PCF shall set the Access Network Information report parameters in the corresponding PCC rule(s) and provision them together with the corresponding Policy Control Request Trigger to the SMF. For those PCC rule(s) based on preliminary service information the PCF may assign the 5QI and ARP of the QoS Flow associated with the default QoS rule to avoid signalling to the UE.

NOTE 1:	The PCF can also use the dynamic or pre-defined PCC Rules related to the IMS signalling to request Access Network Information reporting. This can be used to support e.g., regulatory requirements for SMS over IP, where the IMS network (i.e. P-CSCF) needs to retrieve the user location and/or UE Time Zone information. Note that due to regulatory requirements, the Access Network Information can be requested for SMS over IP, impacting a large number of PDU Sessions, that can lead to significant increase in signalling load when the Access Network Information is requested from AMF.

The PCF shall, upon receiving an Access Network Information report corresponding to the AF session from the SMF, forward the Access Network Information as requested by the AF (if the SMF only reported the serving PLMN identifier or the SNPN identifier to the PCF, as described in clause 6.1.3.5, the PCF shall forward it to the AF). For AF session termination the communication between the AF and the PCF shall be kept alive until the PCF report is received.

If an AF requests the PCF to report the Usage for Sponsored Data Connectivity, the PCF shall provision the corresponding PCC rules, and the Policy Control Request Trigger to the SMF. If the usage threshold provided by the AF has been reached or the AF session is terminated, the PCF forwards such information to the AF.

If an AF requests the PCF to report the Service Data Flow deactivation, the PCF shall report the release of resources corresponding to the AF session. The PCF shall, upon being notified of the removal of PCC Rules corresponding to the AF session from the SMF, forward this information to the AF. The PCF shall also forward, if available, the reason why the resources are released, the user location information and the UE Timezone.

If an AF requests the PCF to report the Resource allocation outcome, the PCF shall report the outcome of the resource allocation of the Service Data Flow(s) related to the AF session. The AF may request to be notified about successful or failed resource allocation. In this case, the PCF shall instruct the SMF to report the successful resource allocation trigger (see clause 6.1.3.5). If the SMF has notified the PCF that the resource allocation of a Service Data Flow is successful and the currently fulfilled QoS matches an Alternative QoS parameter set (as described in clause 6.2.2.1), the PCF shall also provide to the AF the QoS Reference parameter or the Requested Alternative QoS Parameter Set which corresponds to the Alternative QoS parameter set referenced by the SMF.

If an AF requests the PCF to report when the QoS targets can no longer (or can again) be fulfilled for a particular media flow, the PCF shall set the QNC indication in the corresponding PCC rule(s) that includes a GBR or delay critical GBR 5QI value and provision them together with the corresponding Policy Control Request Trigger to the SMF. At the time, the SMF notifies that GFBR can no longer (or can again) be guaranteed for a QoS Flow to which those PCC Rule(s) are bound, the PCF shall report to the AF the affected media flow and provides the indication that QoS targets can no longer (or can again) be fulfilled. If additional information is received with the notification from SMF (see clause 5.7.2.4 of TS 23.501 [2]), the PCF shall also provide to the AF the QoS Reference parameter or the Requested Alternative QoS Parameter Set which corresponds to the Alternative QoS parameter set referenced by the SMF. If the SMF has indicated that the lowest priority Alternative QoS parameter set cannot be fulfilled, the PCF shall indicate to the AF that the lowest priority QoS Reference or the lowest priority set of Requested Alternative QoS Parameters of the Alternative Service Requirements cannot be fulfilled.

If the AF subscribes to be notified of the QoS Monitoring reports, the PCF decides about the path for the QoS Monitoring reports and sets the QoS Monitoring Policy Control Request Trigger accordingly, as described in clause 6.1.3.21. The PCF shall further send the QoS Monitoring reports it receives from the SMF to the AF, unless the AF has provided an indication of direct event notification (i.e. in this case, the AF will receive the QoS Monitoring reports directly from the UPF).

NOTE 2:	This event can only be subscribed as part of an AF session with required QoS (described in clause 6.1.3.22) and as part of AF requested QoS for a UE or group of Ues not identified by a UE address (described in clause 6.1.3.28).

If the AF requests the PCF to report Packet Delay Variation Monitoring parameters (e.g. the variation of UL/DL packet delay between UE and PSA UPF), the PCF will derive the Packet Delay Variation based on the QoS Monitoring reports received from the SMF and send the value to the AF.

If an AF requests the PCF to report on the Out of credit event for the associated service data flow(s), the PCF shall inform the AF (when it gets informed by the SMF) that credit is no longer available for the services data flow(s) related to the AF session together with the applied termination action.

If an AF requests the PCF to report on the Reallocation of credit event for the associated service data flow(s), the PCF shall inform the AF (when it gets informed by the SMF) that credit has been reallocated after credit was no longer available and the termination action was applied for the service data flow(s) related to the AF session.

The PCF can arm the trigger of 5GS Bridge/Router information available to SMF based on local policy (i.e. without an AF request) or based on subscription request from TSCTSF. The PCF shall, upon reception of the 5GS Bridge/Router information (refer to clauses 6.1.3.23, 6.1.3.23a, 6.1.3.23b) from the SMF, forward this information to the TSN AF or the TSCTSF. When the PCF has received the User plane node Management Information Container or Port Management Information Container and related port number from SMF, the PCF also provides User plane node Management Information Container or Port Management Information Container and related port number to the TSN AF or TSCTSF. When SMF has reported the 5GS Bridge/Router information and no AF session exists, the PCF forward this information to a pre-configured TSN AF, or to a pre-configured TSCTSF or a TSCTSF discovered and selected via NRF. In the case of private Ipv4 address being used for IP type PDU Session, the PCF shall additionally report DNN and S-NSSAI of the PDU Session to TSCTSF.

If the AF requests the PCF to report on the outcome of the service area coverage change, the PCF reports the outcome of the service area coverage change to the AF and notifies the current service area coverage to the AF. The outcome is the result of the execution of the request of service coverage change at the PCF; the outcome is successful if the request was executed, and includes the current service area coverage that may be the same or different from the service area coverage provided by the AF. The subscription may also be implicit. In this case there may be bulk subscription, either for an Internal-Group-Id or for any UE. In order to prevent massive notifications to the AF, the request for any UE is associated to a specific Application Identifier or DNN, S-NSSAI. For bulk subscription, when the AF request includes an expiration time, the PCF stops reporting to the AF when the expiration time is reached.

If the AF requests the (H-)PCF, via V-PCF when roaming, to report on the outcome of the UE Policies delivery due to service specific parameter provisioning procedure targeting a single UE, the (H-)PCF reports the outcome of the related UE Policies provisioning procedure for the related traffic descriptor for the UE to the AF, via V-PCF when roaming. The outcome of the UE Policies provisioning procedure includes the success, the failure with an appropriate cause or the interim status report such as UE is temporarily unreachable. (See clauses 4.15.6.7 and 5.2.5.7 of TS 23.502 [3])

A request to report Start of application traffic detection and Stop of application traffic detection triggers the reporting when the PCF receives start of application traffic detection event or stop of application traffic detection event from SMF. The reception of a subscription to this event triggers the setting of the corresponding Policy Control Request Trigger to SMF, if not already subscribed.

A request to forward UE reporting Connection Capabilities from an associated URSP rule triggers the reporting when the PCF receives UE reporting of URSP rule enforcement information from the SMF matching specific Connection Capabilities (see clause 6.6.2.4). The request may include SUPI(s), DNN(s) and/or S-NSSAI(s) to which the request applies. The PCF includes the received Connection Capabilities and PDU session information including PDU session ID, the allocated UE address/prefix, SUPI, DNN, S-NSSAI, SSC Mode, Access Type, RSN, PDU Session Pair ID. The reception of a subscription to this event triggers the setting of the corresponding Policy Control Request Trigger to SMF, if not already subscribed.

If an AF requests the PCF to report Start of application traffic detection and Stop of application traffic detection via bulk subscription, the AF shall provide the application identifier together with the S-NSSAI and DNN. The PCF provides a PCC rule for the application identifier together with the corresponding Policy Control Request Trigger to the SMF for every PDU Session to this S-NSSAI and DNN. When the PCF receives start of application traffic detection event or stop of application traffic detection event for the PCC rule in a PDU Session, the PCF forwards the event to the AF together with the UE identifier and optionally the IP address of the PDU Session corresponding to this PCC rule. When the AF removes bulk subscription for this application identifier, then the PCF removes the Policy Control Request Trigger from the SMF for every PDU Session to this S-NSSAN and DNN, if it is not used for other purpose.

NOTE 3:	The restriction of the bulk subscription to a specific combination of S-NSSAI and DNN avoids excessive signalling load.

If an AF requests the PCF to report on the change between different satellite backhaul categories (e.g. GEO, MEO, LEO, OTHERSAT) or the change between satellite backhaul and non-satellite backhaul, the PCF shall provide the corresponding Policy Control Request Trigger to the SMF to enable the report of satellite backhaul category change (see clause 6.1.3.5) to the PCF. The PCF shall, upon reception of information about the change between satellite backhaul categories or change between satellite backhaul and non-satellite backhaul, notify the AF on the satellite backhaul category change event was met and forward the current satellite backhaul category information received from the SMF to the AF, or indicate that a satellite backhaul is no longer used.

If 5G DDNMF requests the PCF to report on the Change of PDUID, the PCF shall notify whenever a new PDUID is allocated. Further details on how the 5G DDNMF retrieves and subscribes to notifications on Change of PDUID are defined in TS 23.304 [34].

A request to report SM Policy Association established or terminated triggers the reporting when the PCF receives the request for notification on the SM Policy Association from SMF. The PCF notifies on the EventID "SM Policy Association established/terminated", includes the PCF binding information of the PCF for the PDU Session of the UE, as described in clause 6.1.1.2.2.

If the TSCTSF requests the PCF notifications for reporting of extra UE addresses, the PCF shall provide the extra UE addresses allocated to the PDU Session due to Framed Routes or IPv6 prefix delegation. The report shall include the actual list of IPv4 address masks or a list of IPv6 prefixes as currently allocated.

If the AF provides the Capability for BAT adaptation or BAT Window and subscribes to PCF for Notification on BAT offset, the PCF will trigger the subscription to SMF for Notification on BAT offset defined in clause 6.1.3.5. When the Notification on BAT offset trigger is set and the PCF receives a BAT offset and optionally an adjusted periodicity from the SMF, the PCF identifies the affected AF session (based on the PCC rule indicated by the SMF) and forwards the BAT offset and optionally the adjusted periodicity for this AF session to the TSCTSF.

* * * Next Change * * *

### 6.1.6	Policy control decisions based on awareness of URSP rule enforcement

#### 6.1.6.1	General

Based on operator policy, the PCF may, together with any other input for PCC decisions (see clause 6.2.1.2), make policy control decisions based on awareness of URSP rule enforcement for an application by using the following mechanisms:

-	Policy control decisions based on awareness of URSP rule enforcement with UE assistance: PCF may make policy control decisions based on UE reported Connection Capabilities as illustrated in clause 6.1.6.2.

-	Policy control decisions based on awareness of URSP rule enforcement without UE assistance: PCF may make policy control decisions without involving UE based on application detection as illustrated in clause 6.1.6.3.

Editor’s note: the description on PCC rule generation based on pre-configured URSP rules is FFS.

The PCF for the UE may adjust the URSP rules when needed, based on the notified URSP rule enforcement information, which may include UE reported Connection Capabilities if available, PDU Session parameters if available, and detected application event if applicable.

#### 6.1.6.2	Policy control decisions based on awareness of URSP rule enforcement with UE assistance

The PCF may obtain UE reporting of URSP rule enforcement as described in clause 6.6.2.4.

Based on the received URSP rule enforcement information, the PCF for the PDU Session may interact with the PCF for the UE as described in clasue 4.16.X of TS 23.502 [3]. The PCF for the UE may adjust the URSP rules based on the notification from the PCF for a PDU Session e.g. when the PCF for the UE determines that the UE does not have up-to-date URSP rules.

NOTE:	The PCF can combine UE reporting of URSP rule enforcement with analytics information (see clause 6.1.6.3).

#### 6.1.6.3	Policy control decisions based on awareness of URSP rule enforcement without UE assistance

The PCF for the UE may subscribe to Statistics for traffic monitoring of known traffic according to provisioned URSP rule(s). If the PCF for a UE is notified with traffic which is not expected according to a URSP rule as described in TS 23.288 [24], the PCF for a UE may adjust the URSP rules when unexpected application traffic is detected.

In detail, the PCF for a UE generate the input as described in clause 6.20.2 of TS 23.288 [24] and the NWDAF notifies the unexpected traffic as described in clause 6.30.3 of TS 23.288 [24].

* * * Next Change * * *

#### 6.6.2.1	Structure Description

The UE Route Selection Policy (URSP) includes a prioritized list of URSP rules.

Table 6.6.2.1-1: UE Route Selection Policy

| Information name | Description | Category | PCF permitted to modify in a URSP | Scope |
| --- | --- | --- | --- | --- |
| URSP rules | 1 or more URSP rules as specified in table 6.6.2.1-2 | Mandatory | Yes | UE context |

The structure of the URSP rules is described in Table 6.6.2.1-2 and Table 6.6.2.1-3.

Table 6.6.2.1-2: UE Route Selection Policy Rule

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |
| --- | --- | --- | --- | --- |
| Rule Precedence | Determines the order the URSP rule is enforced in the UE. | Mandatory (NOTE 1) | Yes | UE context |
| Traffic descriptor | This part defines the Traffic descriptor components for the URSP rule. | Mandatory (NOTE 3) |  |  |
| Application descriptors | It consists of OSId and OSAppId(s) (NOTE 2, NOTE 8). | Optional | Yes | UE context |
| IP descriptors (NOTE 6) | Destination IP 3 tuple(s) (IP address or IPv6 network prefix, port number, protocol ID of the protocol above IP) (NOTE 8). | Optional | Yes | UE context |
| Domain descriptors | FQDN(s) or a regular expression which are used as a domain name matching criteria (NOTE 7, NOTE 8). | Optional | Yes | UE context |
| Non-IP descriptors (NOTE 6) | Descriptor(s) for destination information of non-IP traffic (NOTE 8). | Optional | Yes | UE context |
| DNN | This is matched against the DNN information provided by the application (NOTE 8). | Optional | Yes | UE context |
| Connection Capabilities | This is matched against the information provided by a UE application when it requests a network connection with certain capabilities (NOTE 4, NOTE 8) or traffic categories (NOTE 5). | Optional | Yes | UE context |
| List of Route Selection Descriptors | A list of Route Selection Descriptors. The components of a Route Selection Descriptor are described in table 6.6.2.1-3 (NOTE 9). | Mandatory |  |  |
| NOTE 1: Rules in a URSP shall have different precedence values. NOTE 2: The information is used to identify the Application(s) that is(are) running on the UE's OS. The OSId does not include an OS version number. The OSAppId does not include a version number for the application. NOTE 3: At least one of the Traffic descriptor components shall be present. NOTE 4: The format and some values of Connection Capabilities, e.g. "ims", "mms", "internet", etc., are defined in TS 24.526 [19]. More than one Connection Capabilities value can be provided. NOTE 5: The format and values of Connection Capabilities Traffic Descriptor to match against standardized traffic categories are defined in TS 24.526 [19] according to the requirements in GSMA PRD NG.135 [39]. The reserved values of Connection Capabilities to match operator-specific traffic categories are specified in TS 24.526 [19]. Traffic categories requested by the UE application are independent from the UE's Operating System. Operator-specific traffic categories values are out of scope of 3GPP specifications. Details on how UE applications indicate traffic categories to the UE's Operating System are out of scope of 3GPP specifications. NOTE 6: A URSP rule cannot contain the combination of the Traffic descriptor components IP descriptors and Non-IP descriptors. NOTE 7: The match of this traffic descriptor does not require successful DNS resolution of the FQDN provided by the UE Application. NOTE 8: Not applicable for PINE traffic. NOTE 9: Only applies to traffic to/from PINEs. PIN ID and other traffic descriptor components are mutually exclusive, i.e. if PIN ID is included in a URSP rule, then no other traffic descriptor components are supported in the same URSP rule. |  |  |  |  |

Table 6.6.2.1-3: Route Selection Descriptor

| Information name | Description | Category | PCF permitted to modify in URSP | Scope |
| --- | --- | --- | --- | --- |
| Route Selection Descriptor Precedence | Determines the order in which the Route Selection Descriptors are to be applied. | Mandatory (NOTE 1) | Yes | UE context |
| Route selection components | This part defines the route selection components | Mandatory (NOTE 2) |  |  |
| SSC Mode Selection | One single value of SSC mode. (NOTE 5) | Optional | Yes | UE context |
| Network Slice Selection | Either a single value or a list of values of S-NSSAI(s). | Optional (NOTE 3) | Yes | UE context |
| DNN Selection | Either a single value or a list of values of DNN(s). | Optional | Yes | UE context |
| PDU Session Type Selection | One single value of PDU Session Type | Conditional (NOTE 8) | Yes | UE context |
| Non-Seamless Offload indication | Indicates if the traffic of the matching application is to be offloaded to non-3GPP access outside of a PDU Session. | Optional (NOTE 4) | Yes | UE context |
| ProSe Layer-3 UE-to-Network Relay Offload indication | Indicates if the traffic of the matching application is to be sent via a ProSe Layer-3 UE-to-Network Relay outside of a PDU session. | Optional (NOTE 4) | Yes | UE context |
| ProSe Multi-path Preference | Indicates if the traffic of the matching application is preferred to be sent via a PDU Session over the Uu reference point and a ProSe Layer-3 UE-to-Network Relay outside of a PDU session. | Optional (NOTE 9) | Yes | UE context |
| Access Type preference | Indicates the preferred Access Type (3GPP or non-3GPP or Multi-Access) when the UE establishes a PDU Session for the matching application. | Optional | Yes | UE context |
| PDU Session Pair ID | An indication shared by redundant PDU Sessions as described in clause 5.33.2.1 of TS 23.501 [2]. | Optional | Yes | UE context |
| RSN | The RSN as described in clause 5.33.2.1 of TS 23.501 [2]. | Optional | Yes | UE context |
| Route Selection Validation Criteria (NOTE 6) | This part defines the Route Validation Criteria components | Optional |  |  |
| Time Window | The time window when the matching traffic is allowed. The RSD is not considered to be valid if the current time is not in the time window. | Optional | Yes | UE context |
| Location Criteria | The UE location where the matching traffic is allowed. The RSD rule is not considered to be valid if the UE location does not match the location criteria. | Optional | Yes | UE context |
| NOTE 1: Every Route Selection Descriptor in the list shall have a different precedence value. NOTE 2: At least one of the route selection components shall be present. NOTE 3: When the Subscription Information contains only one S-NSSAI in UDR, the PCF needs not provision the UE with S-NSSAI in the Network Slice Selection information. The "match all" URSP rule has one S-NSSAI at most. NOTE 4: If this indication is present in a Route Selection Descriptor, no other components shall be included in the Route Selection Descriptor. NOTE 5: The SSC Mode 3 shall only be used when the PDU Session Type is IP. NOTE 6: The Route Selection Descriptor is not considered valid unless all the provided Validation Criteria are met. NOTE 7: In this Release of specification, inclusion of the Validation Criteria in Roaming scenarios is not considered. NOTE 8: This component shall be present when the Route Selection Component does neither include the "Non-Seamless Offload indication" nor "ProSe Layer-3 UE-to-Network Relay Offload indication". NOTE 9: If this indication is present in a Route Selection Descriptor, ProSe Layer-3 UE-to-Network Relay Offload indication shall not be included in the Route Selection Descriptor. |  |  |  |  |

Each URSP rule contains a Traffic descriptor (containing one or more components described in Table 6.6.2.1-2) that determines when the rule is applicable. A URSP rule is determined to be applicable when every component in the Traffic descriptor (for traffic descriptors other than the PIN ID) matches the corresponding information from the application or matches the information configured for a PIN (if the URSP rule contains a PIN ID traffic descriptor). A URSP rule is determined not to be applicable when for any given component in the Traffic descriptor:

-	No corresponding information from the application/for a PIN is available; or

-	The corresponding information from the application/for a PIN does not match any of the values in the Traffic descriptor component.

NOTE 1:	It is recommended to avoid listing more than two components in the Traffic descriptor of a URSP rule.

If a URSP rule is provided that contains a Traffic descriptor with two or more components, it is recommended to also provide URSP rule(s) with lower precedence and a Traffic descriptor with less components, in order to increase the likelihood of URSP rule matching for a particular application.

Each URSP rule contains a list of Route Selection Descriptors containing one or multiple Route Selection Descriptors each having a different Route Selection Descriptor Precedence value. A Route Selection Descriptor contains one or more of the following components:

-	Session and Service Continuity (SSC) Mode: Indicates that the traffic of the matching application shall be routed via a PDU Session supporting the included SSC Mode.

-	Network Slice Selection: Indicates that the traffic of the matching application shall be routed via a PDU Session supporting any of the included S-NSSAIs, see clause 5.15.4 in TS 23.501 [2]. It includes one or more S-NSSAI(s).

-	DNN Selection: Indicates that the traffic of the matching application shall be routed via a PDU Session supporting any of the included DNNs. It includes one or more DNN(s). If a DNN Selection component is provided in the Route Selection Descriptor then the UE shall use any of the DNNs of the DNN Selection component, instead of the DNN requested by the application for the PDU Session that is used to route the traffic of the matching application. If there is no DNN Selection component in the Route Selection Descriptor, then the UE shall use the DNN requested by the application for the PDU Session that is used to route the traffic of the matching application.

NOTE 2:	To provide uniform service experience for UEs from earlier Releases, when a USRP rule with a DNN in both, Traffic descriptor and Route Selection Descriptor, is provided to the UEs, the DNN(s) used in the Traffic descriptor would also need to be included in the policy for DNN replacement in the network. In addition, a lower priority Route Selection Descriptor without a DNN would also need to be provided to the UEs.

-	PDU Session Type Selection: Indicates that the traffic of matching application shall be routed via a PDU Session supporting the included PDU Session Type. The possible PDU Session Types are defined in clause 5.6.10 in TS 23.501 [2].

-	Non-Seamless Offload indication: Indicates that traffic of the matching application is to be offloaded to non-3GPP access outside of a PDU Session when the rule is applied. If this component is present in a Route Selection Descriptor, no other components shall be included in the Route Selection Descriptor.

-	ProSe Layer-3 UE-to-Network Relay Offload indication: Indicates that the traffic of the matching application is to be sent via a ProSe Layer-3 UE-to-Network Relay outside of a PDU Session when the rule is applied. If this indication is absent and the ProSe Multipath Preference indication is absent then the traffic matching the URSP rule shall not be sent via a ProSe Layer-3 UE-to-Network Relay outside of a PDU Session. If this component is present in a Route Selection Descriptor, no other components shall be included in the Route Selection Descriptor.

-	ProSe Multipath Preference indication: Indicates that the traffic of the matching application is preferred to be sent via a PDU Session over the Uu reference point and a ProSe Layer-3 UE-to-Network Relay without N3IWF outside of a PDU Session. The traffic of the matching application may be sent via a PDU session over Uu reference point or via ProSe Layer-3 UE-to-Network Relay outside of a PDU Session when e.g. one of the paths is not available. If this indication is absent and the ProSe Layer-3 UE-to-Network Relay Offload indication is absent then the traffic matching of the URSP rule shall not be sent via a ProSe Layer-3 UE-to-Network Relay outside of a PDU Session. If this component is present in a Route Selection Descriptor, other components can be included in the Route Selection Descriptor to determine the PDU Session over the Uu reference point.

-	Access Type Preference: If the UE needs to establish a PDU Session when the rule is applied, this indicates the Access Type (3GPP or non-3GPP or multi-access) on which the PDU Session should be established. The type "Multi-Access" indicates that the PDU Session should be established as a MA PDU Session, using both 3GPP access and non-3GPP access.

NOTE 3:	The Access Type of 3GPP also includes the use of 5G ProSe Layer-2 UE-to-Network Relay access as defined in TS 23.304 [34]. The Access Type of non-3GPP also includes the use of 5G ProSe Layer-3 UE-to-Network Relay with N3IWF as defined in TS 23.304 [34].

-	PDU Session Pair ID: An indication shared by redundant PDU Sessions as described in clause 5.33.2.1 of TS 23.501 [2].

-	RSN: The RSN for redundant PDU Sessions as described in clause 5.33.2.1 of TS 23.501 [2].

NOTE 4:	For backward compatibility, PCF may provide a RSD with PDU Session Pair ID and RSN and a RSD without PDU Session Pair ID and RSN in the URSP rule. In this case, the RSD with PDU Session Pair ID and RSN has a lower precedence value (i.e. higher prioritised) than the one without PDU Session Pair ID. If a non-supporting UE receives the RSD containing PDU Session Pair ID, it ignores this RSD.

NOTE 5:	The UE may also set the PDU Session Pair ID and RSN parameters based on UE implementation as described in clause 5.33.2.1 of TS 23.501 [2].

-	Time Window: The Route Selection Descriptor is not be considered valid unless the UE is in the time window.

-	Location Criteria: The Route Selection Descriptor is not be considered valid unless the UE's location matches the Location Criteria.

NOTE 6:	The structure of the URSP does not define how the PCF splits the URSP when URSP cannot be delivered to the UE in a single NAS message.

NOTE 7:	It is expected that UE applications will not be able to change or override the PDU Session parameters in the URSP rules. A UE application can express preferences when it requests a network connection (e.g. certain Connection Capabilities), which can be mapped into specific PDU Session parameters by the URSP rules.

NOTE 8:	A Route Selection Descriptor can include a Time Window and/or a Location Criteria or neither a Time Window nor a Location Criteria. A URSP rule can include RSDs with or without validation criteria at the same time.

In the case of network rejection of the PDU Session Establishment Request, the UE may trigger a new PDU Session establishment based on the rejection cause and the URSP policy.

When the PCF provisions URSP rules to the UE, one URSP rule with a "match all" Traffic descriptor may be included.

NOTE 9:	When URSP rules containing NSSP are available to the UE and the URSP rule with the "match all" Traffic descriptor is not part of them, a UE application that has no matching URSP rule and no UE Local Configuration cannot request a network connection.

The URSP rule with the "match all" Traffic descriptor is used to route the traffic of applications which do not match any other URSP rules and shall therefore be evaluated as the last URSP rule, i.e. with lowest priority. There shall be only one Route Selection Descriptor in this URSP rule. The Route Selection Descriptor in this URSP rule includes at most one value for each Route Selection Component.

NOTE 10:	How to set the URSP rule with the "match all" Traffic descriptor as the URSP rule with lowest priority is defined in TS 24.526 [19].

NOTE 11:	The URSP rule with the "match all" Traffic descriptor is not applicable to PINE traffic.

* * * Next Change * * *

#### 6.6.2.4	UE reporting of URSP rule enforcement

In order to determine the URSP rule enforcement, for a UE indicating the capability of reporting URSP rule enforcement to network (see clause 4.2.2.2.2 of TS 23.502 [3]), the PCF may indicate the UE to send reporting of URSP rule enforcement.

If the PCF indicates to the UE to send reporting of URSP rule enforcement, and if a UE URSP rule includes Connection Capabilities in the TD (see clause 6.6.2.1):

-	when a UE associates a newly detected application to a new PDU Session based on URSP evaluation result (see clause 6.6.2.3), the UE shall include in the PDU Session Establishment Request (see clause 4.3.2.2.1 of TS 23.502 [3]) the Connection Capabilities contained in the Traffic descriptor of the associated URSP rule.

-	when a UE associates a newly detected application to an existing PDU Session based on URSP evaluation result (see clause 6.6.2.3), the UE shall send a PDU Session Modification Request (see clause 4.3.3.2 of TS 23.502 [3]) including the Connection Capabilities contained in the Traffic descriptor of the associated URSP rule.

NOTE 1:	UE reporting of URSP rule enforcement can increase the amount of signalling in the network. Use of this feature is recommended to be restricted for specific UEs and URSP rules based on the deployment choices of the operator.

If the UE enforces several URSP rules for multiple applications, and these multiple applications' traffic are all associated to this PDU session, in order reduce the number of uplink NAS messages, the UE may include more than one URSP rule enforcement report in one PDU Session Modification Request to 5GC (see clause 4.3.3.2 of TS 23.502 [3]).

NOTE 2:	A rule with the "match-all" traffic descriptor cannot contain Connection Capabilities in the Traffic descriptor. The format and values of the Traffic descriptor component type identifier are defined in clause 5.2 of TS 24.526 [19].

The PCF receives reporting from URSP rule enforcement for a given UE via Policy Control Request Triggers (see clause 6.1.3.5).

When the PCF serving the PDU session is not the same as the PCF serving the UE, the PCF serving the UE subscribes to the PCF serving the PDU session to receive the reporting of URSP rule enforcement for a given UE via PCF event reporting (see clause 6.1.3.18 and related procedure in clause 4.16.X of TS 23.502 [3]).

For LBO roaming session case, the H-PCF for the UE sends the PCRT for the UE reporting of URSP rule enforcement to the V-PCF for the UE during the UE Policy Association Establishment or Modification.

The PCF for the UE may check whether the value of URSP rule enforcement and its PDU Session parameters (e.g. DNN/S-NSSAI) are compliant to the URSP rule of the UE. If the PCF for the UE found an inconsistency, the PCF for the UE may perform approprieate actions (e.g. initiating slice replacement procedure).

Policy control decisions based on awareness of URSP rule enforcement are described in clause 6.1.6.

* * * Next Change * * *

Annex A (informative):
URSP rules example

As an example, the URSP rules provisioned in the UE may include the following rules:

Table A-1: Example of URSP rules

| Example URSP rules |  | Comments |
| --- | --- | --- |
| Rule Precedence =1 Traffic Descriptor: Application descriptor=App1 | Route Selection Descriptor Precedence=1 Network Slice Selection: S-NSSAI-a SSC Mode Selection: SSC Mode 3 DNN Selection: internet Access Type preference: 3GPP access | This URSP rule associates the traffic of application "App1" with S-NSSAI-a, SSC Mode 3, 3GPP access and the "internet" DNN. It enforces the following routing policy: The traffic of App1 should be transferred on a PDU Session supporting S-NSSAI-a, SSC Mode 3 and DNN=internet over 3GPP access. If this PDU Session is not established, the UE shall attempt to establish a PDU Session with S-NSSAI-a, SSC Mode 3 and the "internet" DNN over 3GPP access. |
| Rule Precedence =2 Traffic Descriptor: Application descriptor=App2 | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-a Access Type preference: Non-3GPP access | This URSP rule associates the traffic of application "App2" with S-NSSAI-a and Non-3GPP access. It enforces the following routing policy: The traffic of application App2 should be transferred on. a PDU Session supporting S-NSSAI-a using a Non-3GPP access. If this PDU Session is not established, the UE shall attempt to establish a PDU Session with S-NSSAI-a over Access Type=non-3GPP access. |
|  | Route Selection Descriptor Precedence =2 Non-seamless Offload indication: Permitted (WLAN SSID-a) | If the PDU Session cannot be established, the traffic of App2 shall be directly offloaded to WLAN, if the UE is connected to a WLAN with SSID-a (based on the 2nd RSD) |
| Rule Precedence =3 Traffic Descriptor: DNN=DNN_1 | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-a Access Type preference: Non-3GPP access | This URSP rule associates the traffic of applications that are configured to use DNN_1 with DNN_1, S-NSSAI-a over Non-3GPP access. It enforces the following routing policy: The traffic of application(s) that are configured to use DNN_1 should be transferred on a PDU Session supporting S-NSSAI-a over Non-3GPP access. If this PDU Session is not established, the UE shall attempt to establish the PDU Session with S-NSSAI-a over Non-3GPP access. |
| Rule Precedence =4 Traffic Descriptor: Application descriptor=App1 Connection Capabilities="internet", "supl" | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-a DNN Selection: DNN_1 Access Type preference: Non-3GPP access | This URSP rule associates the application "App1" and the Connection Capabilities "internet" and "supl" with DNN_1, S-NSSAI-a over Non-3GPP access. It enforces the following routing policy without UE reporting the URSP rule enforcement as specified in clause 6.6.2.4: When the "App1" requests a network connection with Connection Capability "internet" or "supl", the UE establishes (if not already established) a PDU Session with DNN_1 and S-NSSAI-a over Non-3GPP access. After that, the UE routes the traffic of "App1" over this PDU Session. |
| Rule Precedence =5 Traffic Descriptor: Application descriptor=App3 Connection Capabilities="ims" | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-c DNN Selection: DNN_1 Access Type preference: Multi-Access | This URSP rule associates the application "App3" and the Connection Capability "ims" with DNN_1, S-NSSAI-c and multi-access connectivity. It enforces the following routing policy without UE reporting the URSP rule enforcement as specified in clause 6.6.2.4: When the "App3" requests a network connection with Connection Capability "ims", the UE establishes (if not already established) a MA PDU Session with DNN_1 and S-NSSAI-c. After that, the UE routes the traffic of "App3" over this MA PDU Session by using the received ATSSS rules. |
| Rule Precedence =6 Traffic Descriptor: Application descriptor=App1 | Route Selection Descriptor Precedence =1 DNN Selection: DNN_1 Network Slice Selection: S-NSSAI-a Access Type preference: Multi Access | This URSP rule associates App 1 with DNN_1, S-NSSAI-a with Multi Access connectivity. It enforces the following routing policy: The traffic of Application 1 should be transferred on a PDU Session supporting S-NSSAI-a and DNN_1 according to the received ATSSS rules. After that the UE routes the traffic of any other application according to the ATSSS rule with match all packet filters if available. |
| Rule Precedence =7 Traffic Descriptor: Connection Capabilities="Real time interactive" | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-d Access Type preference: 3GPP-Access | This URSP rule associates the Connection Capability "Real time interactive" with S-NSSAI-d and 3GPP-access connectivity. It enforces the following routing policy and: When any application requests a network connection with Connection Capability "Real time interactive", the UE establishes (if not already established) a PDU Session with S-NSSAI-d over 3GPP access. After that, the UE routes the traffic from the requesting application over this PDU Session. |
| Rule Precedence =8 Traffic Descriptor: Connection Capabilities="Real time interactive" UE supports reporting of URSP rule enforcement conditions (see clause 6.6.2.4) | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-d Access Type preference: 3GPP-Access | If UE reporting of URSP rule enforcement conditions are met (see clause 6.6.2.4), same as in example with Rule Precedence=7, with the difference that the UE enforces the routing policy and additionally, the UE reports the URSP rule enforcement as specified in clause 6.6.2.4. Otherwise, same as in example with Rule Precedence=7. |
| Rule Precedence = lowest priority Traffic Descriptor: * | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-b SSC Mode Selection: SSC Mode 3 DNN Selection: internet | This URSP rule associates all traffic not matching any prior rule a PDU Session with S-NSSAI-b, SSC Mode 3 and the "internet" DNN. It enforces the following routing policy: All traffic not matching any prior rule should be transferred on a PDU Session supporting S-NSSAI-b, SSC Mode 3 and DNN=internet with no access network preference. |

Editor's note:	The example URSP rules with precedence =7 and 8 may need to be updated based on the agreed set of standardized traffic categories and the format defined for operator-specific traffic category.

* * * End of Change* * *