3GPP TSG-SA2 Meeting #156-e	S2-2304069r09

# Online, , 17th Apr 2023 - 21st Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3959 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Forwarding of UE reporting of URSP rule enforcement between SM-PCF and UE-PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#155, the support of UE reporting of URSP rule enforcement for deployments where the PCF serving the PDU session is not the same as the PCF serving the UE was left FFS. Furthermore, concerns were raised regarding using parameter provisioning, with some views expressed that reusing the procedure already present for application detection would be preferred |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to reuse the existing procedure in 4.16.14.2.1 for forwarding the information between SM-PCF and UE-PCF. For clarity reasons, it was decided to write a new clause. Adding the required information to the existing clause or writing a new clause based on describing changes to the existing clause would result in convoluted text that would not be clear to the reader, worst case breaking existing text |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality in deployments where the PCF serving the PDU session is not the same as the PCF serving the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 23.503 CR 0947 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 4.16.X |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change (all new text)* * *

### 4.16.X	Awareness of URSP Rule Enforcement

#### 4.16.X.1	General

Awareness of URSP rule enforcement is specified in clause 6.6.2.4 of TS 23.503 [20].

The content of this clause describes the PCF procedures necessary to realize this functionality.

#### 4.16.X.1	Forwarding of URSP Rule Enforcement Information

This procedure applies when the PCF serving the PDU session receives URSP rule enforcement information from the SMF and forwards this information to the PCF serving the UE (see clause 6.1.3.18 of TS 23.503 [20]).

Figure 4.16.X.1-1: Forwarding of URSP Rule Enforcement Information

1.	The UE Policy Association is established, as described in clause 4.16.11.

2.	If the PCF indicates the UE to send reporting of URSP rule enforcement as described in clause 6.6.2.4 of TS 23.503 [20], then depending on operator policies in the PCF, the PCF may subscribe to the BSF, then step 3 follows, or provides its PCF binding information to the AMF in step 1 with the indication to be notified about the PCF for the PDU Session for a UE, then step 4 follows.

3.	The PCF for the UE determines that URSP rules depend on the UE reporting of URSP rule enforcement, it then subscribes to the BSF to be notified when a PCF for the PDU Session for this SUPI is registered in the BSF, by invoking Nbsf_Management_Subscribe (SUPI; DNN,). Steps 4 and 5 are repeated for each PCF registered for a PDU Session to a SUPI included in the Nbsf_Management.

4.	The SMF establishes a SM Policy Association as described in clause 4.16.4. The allocated UE address/prefix, SUPI, DNN, S-NSSAI, and the PCF address is registered in the BSF, as described in clause 6.1.1.2.2 in TS 23.503 [20].

5a.	If the PCF for the UE subscribed to the BSF in step 3, then the BSF notifies that a PCF for the PDU Session is registered in the BSF, by invoking Nbsf_Management_Notify (UE address(es), PCF address, PCF instance id, PCF Set ID, level of binding). When there are multiple PDU Sessions to the same UE the BSF provides multiple notification to the PCF.

5b.	If the PCF for the UE sent the request to notify that a PCF for the PDU Session is available to the AMF in step 1, then the PCF for the PDU Sessions sends Npcf_PolicyAuthorization_Notify (EventID set to SM Policy Association established, UE address, PCF address, PCF instance is, PCF Set ID) to the PCF indicated in the PCF binding information provided by the SMF.

6.	The PCF for the UE subscribes to notifications of event “UE reporting Connection Capabilities from associated URSP rule” as defined in clause 6.1.3.18 in TS 23.503 [20], using Npcf_PolicyAuthorization_Subscribe (EventId set to “UE reporting Connection Capabilities from associated URSP rule”, EventFilter set to at least “list of Connection Capabilities”) and immediate reporting flag set to the PCF for the PDU Session. The response includes the NotificationCorrelationId and any Connection Capabilities if already available at the PCF for the PDU Session.

7.	If not already installed, the PCF installs the Policy Control Request Trigger to detect "UE reporting Connection Capabilities from associated URSP rule" in the SMF.

8.	When the SMF receives a UE report of URSP rule enforcement via PDU session establishment/modification as descriped in clauses 4.3.2, 4.3.3 (step 8a) and the Policy Control Request Trigger is met, it then reports the received information traffic to the PCF serving the PDU Session, by invoking Npcf_SMPolicyControl_Update as defined in clause 6.1.3.5 of TS 23.503 [20] (step 8b).

9.	The PCF for the UE is notified on the “UE reporting Connection Capabilities from associated URSP rule” by Npcf_PolicyAuthorization_Notify (NotificationCorrelationId, EventId set to “UE reporting Connection Capabilities from associated URSP rule”, EventInformation including the Connection Capabilities) as defined in clause 6.1.3.18 of TS 23.503 [20].

10.	The PCF for the UE checks operator policies and then may make policy control decisions based on awareness of URSP rule enforcement as described in clause 6.1.6 of TS 23.503 [20].

11.	The SM Policy Association is terminated as described in clause 4.16.6. The allocated UE address/prefix, SUPI, DNN, S-NSSAI, and the PCF address are deregistered in the BSF.

12a.	If the PCF for the UE subscribed to the BSF, then the BSF notifies that the PCF serving a PDU Session is deregistered in the BSF, by invoking Nbsf_Management_Notify (Binding Identifier for the PDU Session).

12b.	If the PCF for the UE sent the request to notify that a PCF for the PDU Session is available to the AMF in step 1, then the PCF for the PDU Session sends Npcf_PolicyAuthoritation_Notify (EventID set to SM Policy Association termination, Notification Correlation Id).

Editor's note:	whether Nbsf_Management_Subscribe/ Nbsf_Management_Register need to be enhanced is FFS.

Editor's note:	whether “and PDU session information” should be mentioned in step 9 as part of Npcf_PolicyAuthorization_Notify is FFS

* * * End of Change* * *