3GPP TSG SA WG 2 Meeting# 156-e		S2-2304164r01

17 – 21 April 2023, E-meeting

Title:	[Draft] LS on introduction of support for L4S in 5GS

Response to:

Release:	Rel-18

Work Item:	XRM

Source:	3GPP TSG SA WG2

To:	3GPP TSG RAN WG3

Cc:

Contact person:	Paul Schliwa-Bertling

Paul.Schliwa-Bertling(at)Ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	23.501 CR4219; 23.501 CR4503; 23.501 CR4294

1	Overall description

Based on the Rel-18 FS_XRM study conclusions documented in chapter 8 of TR 23.700-60, SA2 has agreed to support the following:

- Congestion Information monitoring in clause 5.45.3 of TS 23.501 (see 23.501 CR4219).

- Congestion information for exposure in clause 5.45.4 of TS 23.501 (see 23.501 CR4503).

- support for ECN marking for L4S in 5GS in clause 5.37.3 of TS 23.501 (see 23.501 CR4294). This functionality is supported in two ways:

  - By PSA UPF using information provided by NG-RAN.

  - By the NG-RAN alone.

To enable congestion information exposure and/or for ECN marking for L4S in PSA UPF, the SMF should request NG-RAN to measure and report the congestion information for a QoS flow.

- In case of congestion information exposure, SMF indicates to NG-RAN to report the congestion information (i.e. a percentage of congestion level for exposure) of the QoS Flow on UL and/or DL directions via GTP-U header extension to PSA UPF.

- In case of ECN marking for L4S in PSA, SMF indicates to NG-RAN to report the congestion information (i.e. a percentage of packets that UPF uses for ECN marking for L4S) of the QoS Flow on UL and/or DL directions via GTP-U header extension to PSA UPF.

SA2 would like to notified that the “percentage of congestion level for exposure” in clause 5.45.3 and “percentage of packets that UPF uses for ECN marking for L4S” in clause 5.37.3.3 share same value. RAN only needs to report one percentage which is common for the both cases and need not to differentiate whether UPF use this percentage for congestion information exposure and/or for ECN marking for L4S in PSA UPF.

To introduce system wide support for this functionality, SA2 kindly asks RAN3 to introduce necessary changes in the specifications under RAN3 responsibility reflecting SA2´s requirements agreed in the CR.

2	Actions

To RAN WG3

ACTION: 	SA2 kindly ask RAN3 to take the above into account and introduce the necessary support in the specifications under RAN3 responsibility reflecting stage 2 requirements.

3	Dates of next TSG SA WG2 meetings

TSG-SA2 Meeting #157	22-26 May 2023	                     Berlin, DE

TSG-SA2 Meeting #158	21-25 August 2023	                     Goteborg, SE