3GPP TSG-SA2 Meeting #156 (e-meeting)	S2-2304030

Elbonia, April 17th – 21st, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0729 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to ML model storage and retrieval via ADRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#155, it is agreed that ML model ID is always provisioned by MTLF no matter if it is for single model provision of multiple models provision. When MTLF stores ML models in ADRF, there is no need to distinguish if MTLF stores a single model or multiple models. MTLF always provides ML model ID to ADRF. When ML model consumer retrieves an ML model with Nadrf_MLModelManagement_Retrieve service, it will also use ML model ID to identify the model. In clause 6.2B.7, there is an EN: Editor's note: Whether the ADRF sends ML model(s) when requested the NF Consumer is FFS. Since ADRF already can provide model address to the consumer, there is no need to send model to the consumer. Thus, the EN can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add ADRF (Set) ID in the model information in clause 6.2A.2 - Update clause 6.2B.7 that MTLF always provides ML model ID to ADRF. When ML model consumer retrieves an ML model with Nadrf_MLModelManagement_Retrieve service, it will also use ML model ID to identify the model. Remove the EN shown above. - Update clause 10.3.2, 10.3.3 and 10.3.4 about the input and output parameters of ADRF_MLModelManagement service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ADRF_MLModelManagement service definition is incomplete and confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2, 6.2B.7, 10.3.2, 10.3.3, 10.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** Start of Changes ***************

6.2A.2	Contents of ML Model Provisioning

The consumers of the ML model provisioning services (i.e. an NWDAF containing AnLF) as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	Information of the analytics for which the requested ML model is to be used, including:

-	A list of Analytics ID(s): identifies the analytics for which the ML model is used.

-	NF consumer information: identifies the vendor of NWDAF containing AnLF.

NOTE 1:	NF consumer information such as Vendor ID is defined in Stage 3.

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML model ML model.

NOTE 2:	The NWDAF containing MTLF can use the parameter "Use case context" to select the most relevant ML model, when several ML models are available for the requested Analytics ID(s). The values of this parameter are not standardized.

-	[OPTIONAL] ML Model Interoperability Information. This is vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.

-	[OPTIONAL] ML Model Filter Information: enables to select which ML model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML model is requested, e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	[OPTIONAL] Requested representative ratio: a minimum percentage of UEs in the group whose data is a non-empty set and can be used in the model training when the Target of ML Model Reporting is a group of UEs.

-	ML Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	[OPTIONAL] Inference Input Data information: contains information about various settings that are expected to be used by AnLF during inferences such as:

-	the "Input Data" that are expected be used, each of them optionally accompanied by metrics that show the granularity with which this data will be used (i.e., a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data).

NOTE 3:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that are expected to be used as a list of NF instance (or NF set) identifiers.

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

Editor's note:	It is FFS how to convey degradation information for an ML model.

-	[OPTIONAL] Indication of supporting multiple ML models.

-	[OPTIONAL] Accuracy level(s) of Interest.

-	[OPTIONAL] Number of ML model(s), indicating the maxmium number of Multiple ML models the ML Model provider e.g. NWDAF(MTLF) could provide to the consumers of the ML model(s).

NOTE 4:	Multiple ML models Filter Information are composed by Indication of supporting multiple ML models, Accuracy level(s) of Interest, Number of ML model(s).

The NWDAF containing MTLF provides to the consumer of the ML model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	For each Analytics ID requested by the service consumer, a set of pair (s) of unique ML Model identifier and the ML model Information.

ML Model Information, which includes:

-	the ML model file address (e.g. URL or FQDN), or

-	ADRF (Set) ID.

-	[OPTIONAL] Validity period: indicates time period when the provided ML Model Information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

-	[OPTIONAL] ML model representative ratio: indicating the percentage of UEs in the group whose data is used in the ML model training when the Target of ML Model Reporting is a group of UEs.

-	[OPTIONAL] Training Input Data Information: contains information about various settings that have been used by MTLF during training, such as:

-	the "Input Data" that have been used, each of them optionally accompanied by metrics that show the granularity with which this data has been used (i.e., a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data).

NOTE 5:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that have been used as a list of NF instance (or NF set) identifiers.

NOTE 6:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

********** Next of Changes ***************

6.2B.7	ML model retrieval from ADRF

The procedure depicted in Figure 6.2B.7-1 is used by consumers (NWDAF containing MTLF and NWDAF containing AnLF) to retrieve ML models from an ADRF.

Figure 6.2B.7-1: Procedure for ML model(s) retrieval from ADRF

1.	The NWDAF service consumer (NWDAF containing AnLF or NWDAF containing MTLF) subscribes/requests for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Request service (if consumer is NWDAF containing AnLF) or by invoking the Nnwdaf_MLModelTraining_Subscribe service (if consumer is NWDAF containing) operation.

2.	The NWDAF containing MTLF determines whether the set of ML Model(s) associated with a/an (set of) Analytics ID(s) should be retrieved from the ADRF.

When NWDAF containing MTLF authorizes the NF consumer to retrieve the ML model(s) stored in the ADRF directly, steps 3 and 4 is skipped.

If NWDAF containing MTLF determines that the set of ML Model(s) corresponding Analytics ID(s) requested in step 1 needs to be retrieved from ADRF and the NF consumer is agnostic to where the ML model(s) is stored, then Steps 3 and 4 is performed.

Note:	How NWDAF containing MTLF and ADRF authorizes the NF Consumer is in scope of SA WG3.

3.	The ADRF service consumer (NWDAF containing MTLF) requests for the ML model stored in ADRF by invoking the Nadrf_MLModelManagement_Retrieve request (one or more  unique ML model Identifier(s)) service operation.

4.	The ADRF sends Nadrf_MLModelManagement_Retrieve response (address(es) e.g., URL or FQDN of Model file(s) stored in ADRF) service operation.

5.	The NWDAF containing MTLF notifies/ response to the NWDAF service consumer with the tuple Analytics ID, one or more tuples of unique ML Model identifier and ML Model Information. The ML model information may contain the ML model file address (e.g. URL or FQDN) or ADRF (Set) ID. The ADRF(Set) ID in included only when the NWDAF containing MTLF authorizes the NF consumer to retrieve the ML model(s) stored in the ADRF in step 2.

6.	If in step 5, the NWDAF service consumer (NWDAF containing AnLF or NWDAF containing MTLF) received ADRF (Set) ID (where the ML model(s) requested in step 1 is stored), then the NWDAF service consumer may invoke the Nadrf_MLModelManagement_Retrieve Subscribe/Request (one or more unique ML Model identifier(s)) service operation to get the ML model stored in ADRF.

7.	The ADRF sends Nadrf_MLModelManagement_Retrieve notify/ response (ML Model identifier(s) and address(es)of Model file(s) stored in ADRF to the NWDAF service consumer.

********** Next of Changes ***************

10.3.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store ML model(s). ML model(s) or ML model address(es) stored in NWDAF containing MTLF are provided to the ADRF in the request message.

Inputs, Required: NF instance ID of the NWDAF containing MTLF and a set of:

-	the tuple ( unique ML Model identifier and address (e.g. URL or FQDN) of Model file).

NOTE:	The ADRF can download ML model files based on the ML model addresses provided by the NWDAF containing MTLF. How the ADRF downloads the ML model and locally stores the ML model is left for implementation.

Inputs, Optional: None.

Outputs Required: Result Indication, one of the following:

-	one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: Storage Transaction Identifier.

10.3.3	Nadrf_MLModelManagement_Delete service operation

Service operation name: Nadrf_MLModelManagement_Delete

Description: This service operation instructs the ADRF to delete stored ML model file(s) or ML model address(es).

Inputs, Required:

-	Storage Transaction Identifier; or

-	one or more unique ML Model identifier(s).

When a Storage Transaction Identifier is given, ADRF shall delete all the models that stored in the corresponding storage transaction.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

10.3.4	Nadrf_MLModelManagement_Retrieval Request service operation

Service operation name: Nadrf_MLModelManagement_Retrieval Request

Description: The consumer NF uses this service operation to request the ADRF to get the ML model(s) stored in ADRF.

Inputs, Required:

-	unique ML Model identifier(s).

Inputs, Optional: None.

Outputs Required:

-	one or more tuples of unique ML Model identifiers and address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: None.

********** End of Changes ***************