3GPP TSG-SA2 Meeting #156e	S2-2304035r01

Elbonia, April 2023, 17th-21st

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3953 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys - QoS filtering criteria corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Fixing incorrect or incomplete information in the QoS filter criteria section for UE member selection for AIML operation. In particular, this CR focuses on providing real-time QoS filter criteria UE member selection. Additionally, the Specific procedure for the 5GC assistance to member selection based on the UE’s current location, historical location and direction got implemented in the middle of the Specific procedure for QoS UE member filtering criteria, mixing both procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed references to time windows - Added QoS filter criteria parameters - Clarified that Application ID or Flow descriptions must be included - Moved former clause 4.15.13.3.2 to a new clause 4.15.13.X. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect or incomplement information leads to inability to provide a QoS filtering criteria assistance to UE member selection and confuses ther reader with the procedure for member selection base don UE’s current location, historical location anddirecton. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.2, 4.15.13.3.1, 4.15.13.3.2, 4.15.13.3.3, 4.15.13.X (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.15.13.2	UE Member Filtering Criteria for 5GS assistance to UE member selection

Table 4.15.13.2-1 provides a summary of the UE member filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of UE Member filtering criteria

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the selected UEs match or exceed the QoS of the filtering criteria | Service operation: Nsmf_EventExposure or Nudm_EvenExposure, Filter: target=SUPI, Event=QoS Monitoring, Application ID or Flow Descriptions | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the selected UE for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI Event ID: Change of Access Type and/or Change of RAT Type | This can be provided by existing service operations, events, and procedures. |
| End-to-end data volume transfer time | Indicate the end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE to AF | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = End-to-end data volume transfer time | Clause 6.18 of TS 23.288 [50]. |
| UE current location | Indicate the certain area that the selected UE currently located in. | Service operation: Namf_EventExposure Filter: a list of GPSI(s) or SUPI(s) | 4.15.13.X |
| UE historical location | Indicate the certain area that the selected UE appeared in a historical period of time | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period | 4.15.13.X |
| UE direction | Indicate the selected UEs should include different moving direction | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Mobility, Direction | 4.15.13.X |

Editor's note:	The list of UE Member filtering criteria and their corresponding actions are FFS.

* * * Next Change * * *

#### 4.15.13.3	Specific procedure for QoS UE member filtering criteria

##### 4.15.13.3.1	General

An AF may invoke Nnef_UEMemberSelectionAssistance_Subscribe service operation with a QoS filtering criteria for receiving a list of UEs that match or exceed such criteria.

In addition to the mandatory parameters, the AF also includes in the request:

-	QoS filtering criteria.

-	Optionally, an Area of Interest: location area of the candidate UEs.

The QoS filtering criteria includes:

- Flow descriptions or Application ID.

And one or more of:

- Uplink packet delay.

- Downlink packet delay.

- Round trip packet delay.

The current list of UEs that match or exceed the QoS filtering criteria and are optionally located in the AoI in real time, for the duration of the subscription, is further described in clause 4.15.13.3.3.

* * * Next Change * * *

##### 4.15.13.3.2	VoidSpecific procedure for the 5GC assistance to member selection based on the UE's current location, historical location and direction.

Figure 4.15.13.3.2-1: 5GC assistane to Member selection based on the UE's travel history, direction and separation distance

1.	AF requests 5GS assistance to support the UE selection by considering the UE's historical location, UE's current location and direction. AF includes the UE lists and the following criteria as part of the FL UE selection request:

-	UE historical location: The Target AoI where the UEs have been roving over the historical nomadic period before moving into the FL coverage area.

-	UE current location: The current AoI which is the coverage area of the FL training server where the selected UEs located in to participate in the FL operation.

-	Direction: Select the UE with the different direction in the FL coverage Area.

When providing a target area for FL, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area.

2.	NEF translates the GPSIs to SUPIs and maps the filtering criteria into the corresponding UE filtering information.

3-4. The NEF invokes Namf_EventExposure_Subscribe service operation with the UE list, the current AoI and event ID = Location report. AMF will provide a list of UEs that are within the current AoI to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible targets UEs from AMF within the current AoI.

5.	In order to identify the appropriate NWDAF which can provide analytic output to derive the visited AOI info and to get the UE direction for the possible target UEs above, the NEF needs to select the suitable NWDAF by initiating the NWDAF discovery request (Analytics ID = UE mobility, AOI = Target AOI) with UE list received from step 4.

6-7. The NEF invokes NWDAF Analytics Info request (UE list received from step 4, Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period"). NWDAF will then provide a list of UEs that were ever roving within the target AOI, at the minimum, over the historical nomadic period NWDAF and also provide a list of UEs location in the order of which the UE passes through to the NEF. Then the NEF gets the corresponding statistics of UE mobility and the UE's direction.

8-9. Based on the information provided by the AMF and NWDAF, the NEF can determine the FL candidate UEs which are now within the FL coverage area but were roving within the target AOI over the historical nomadic period and UE with the different direction as requested by AF. The NEF notifies AF for such UE candidate list.

10.	The NEF also needs to consider the list of UEs which are now within the FL coverage area, but may move out of the FL coverage area. Therefore, for each UE in the candidate list in steps 8-9, the NEF invokes the Namf_EventExposure_Notify service with UE ID = SUPI, Event ID=UE moving in/out of AOI to keep track the movement of the UE(s) in/out of the AOI.

11-12.	If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify AF of the given UE which is moving out the coverage.

* * * Next Change * * *

##### 4.15.13.3.3	UE Member Assistance with QoS filtering criteria for real-time QoS Monitoring

At the reception of Nnef_UEMemberSelectionAsistance_Subscribe request of step 1, NEF determines to do real-time QoS monitoring for determining the list of UEs that match or exceeds the QoS filtering criteria in real time. QoS Monitoring must have been activated in SMF for that QoS Flow before (e.g. by AF Session with required QoS). Nsmf_EventExposure consumer becomes an additional receiver of the QoS Monitoring information.

Editor's note:	For QoS Flows other than QoS Flow associated to the default QoS rule, whether and how QoS monitoring is performed is FFS.

Figure 4.15.13.3.3-1: 5GC assistance to UE member selection for real-time QoS monitoring

1.	AF subscribes the member selection assistance functionality by sending Nnef_UEMemberSelectionAssistance_subscribe request including the Application ID (or S-NSSAI/DNN), Flow Description(s) (or Application ID) and the QoS filtering criteria that includes one or more of UL packet delay, UL packet delay and round-trip packet delay.

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executed based on the QoS filtering criteria provided by the AF.

3.	If an Application ID is present in the request from the AF, the NEF derives the S-NSSAI and DNN to which this Application ID have access. NEF discovers querying UDM and NRF the SMFs that are deployed in the Area of Interest. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination (this S-NSSAI and DNN combination has been derived from the Application ID).

4.	For each target SMF, NEF sends an Nsmf_EventExposure_Subscribe request (Event: 'QoS monitoring', target UEs, Application ID or Flow Description(s), S-NSSAI, DNN, etc.). Alternatively, NEF may send an Nudm_EventExposure_Subscribe request to the UDM (not shown in Figure 4.15.13.3.3-1).

5.	The SMF many need to interact with UPF for requesting the QoS monitoring for certain flows.

6.	SMF sends an Nsmf_EventExposure_Notify request including an Event Exposure notification.

7.	Based on the Event Exposure reports received from the SMFs, NEF consolidates the results received from the SMFs and derives the list(s) of candidate UE(s) and additional information which fulfil the QoS filtering criteria requested by the AF request.

8.	NEF sends a Nnef_ UEMemberSelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

* * * Next Change * * *

##### 4.15.13.X	Specific procedure for the 5GC assistance to member selection based on the UE's current location, historical location and direction.

Figure 4.15.13.X-1: 5GC assistane to Member selection based on the UE's travel history, direction and separation distance

1.	AF requests 5GS assistance to support the UE selection by considering the UE's historical location, UE's current location and direction. AF includes the UE lists and the following criteria as part of the FL UE selection request:

-	UE historical location: The Target AoI where the UEs have been roving over the historical nomadic period before moving into the FL coverage area.

-	UE current location: The current AoI which is the coverage area of the FL training server where the selected UEs located in to participate in the FL operation.

-	Direction: Select the UE with the different direction in the FL coverage Area.

When providing a target area for FL, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area.

2.	NEF translates the GPSIs to SUPIs and maps the filtering criteria into the corresponding UE filtering information.

3-4. The NEF invokes Namf_EventExposure_Subscribe service operation with the UE list, the current AoI and event ID = Location report. AMF will provide a list of UEs that are within the current AoI to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible targets UEs from AMF within the current AoI.

5.	In order to identify the appropriate NWDAF which can provide analytic output to derive the visited AOI info and to get the UE direction for the possible target UEs above, the NEF needs to select the suitable NWDAF by initiating the NWDAF discovery request (Analytics ID = UE mobility, AOI = Target AOI) with UE list received from step 4.

6-7. The NEF invokes NWDAF Analytics Info request (UE list received from step 4, Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period"). NWDAF will then provide a list of UEs that were ever roving within the target AOI, at the minimum, over the historical nomadic period NWDAF and also provide a list of UEs location in the order of which the UE passes through to the NEF. Then the NEF gets the corresponding statistics of UE mobility and the UE's direction.

8-9. Based on the information provided by the AMF and NWDAF, the NEF can determine the FL candidate UEs which are now within the FL coverage area but were roving within the target AOI over the historical nomadic period and UE with the different direction as requested by AF. The NEF notifies AF for such UE candidate list.

10.	The NEF also needs to consider the list of UEs which are now within the FL coverage area, but may move out of the FL coverage area. Therefore, for each UE in the candidate list in steps 8-9, the NEF invokes the Namf_EventExposure_Notify service with UE ID = SUPI, Event ID=UE moving in/out of AOI to keep track the movement of the UE(s) in/out of the AOI.

11-12.	If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify AF of the given UE which is moving out the coverage.

* * * End of Changes * * *