3GPP TSG-SA WG2 Meeting #156E	S2-2304052r01

# Electronic, 17 – 21 April 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4196 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving (Removing) ENs for TSN TN integration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are still some ENs in TS 23.501 and wrong definitions. It is proposed to perform following changes: - Resolve the Editor's Note: “It is FFS if the interface between the SMF/CUC and TN CNC could allow the SMF/CUC to indicate the TEID and QFI of the given QoS Flow to the TN CNC.” TN CNC is a control entity for a Layer 2 Network. The handling of TEID and QFI requires changes in various IEEE 802.1 standards (i.e. IEEE Std 802.1Q-2022, P802.1Qdj, IEEE Std 802.1CB-2017, IEEE Std 802.1CBcv-2021. There is no active or planned project in IEEE 802.1 that asks for such an extension. The probability that such an extension is accepted by IEEE 802.1 is very low, because: - TEID and QFI is only used in 3GPP 5GS networks. - While the used MAC address of the StreamID identifies the End Station in TN, TEID contains the IP address of the GTP-U Tunnel End Point. This requires at least additional configuration at TN CNC to map Topology information exchanged via LLDP to the respective IP addresses of the End Station. This is not covered by the current YANG data model. - QFI is even more critical, because it doesn’t contain any reference to identify a Talker End Station. Additionally, the limitation to 63 QFI values in 3GPP requires further actions to support enough Talker End Stations in TN. - At least two IEEE 802.1 TSN projects need to be started. Most critical point: Even if the projects are initiated in IEEE 802.1 to support TEID and QFI, there is no way that the respective IEEE 802.1 standards are stable or finalized before 2025. This is too late for 3GPP Rel.18. - Resolve the Editor's Note: “It is FFS if and how the Service Data Flows can be used to identify streams in the TN.” There are multiple methods how a Base Station (AN-TL) and UPF (CN-TL) can identify the packet of a stream (e.g. stream identification configuration information, QFI, stream header information (e.g. Service Data Flow Filter information), …). A method shall guarantee that a stream uses in TN an individual DMAC and VLAN for the packets, which is not shared with other traffic. This can be achieved when the Stream Transformation information contains an identification information that identifies if a packet belongs to the stream addressed in the Stream Transformation information. Further there are situations where a single QoS Flow may carry multiple streams. This happens when for example, the outer CNC doesn’t configure PSFP for streams. As a conclusion, it should be up to implementation, which identification information is used in AN-TL or CN-TL acting as Talker End Station. The resulting specification work is to allow the transfer of the different identification information together with the Stream Transformation information in the TL-Container. The remaining work is covered already in the first Editor’s Note. - Resolve the Editor's Note: “How SMF/CUC derives the input information based on the TimeAwareOffset and where this input information is mapped to Gate Control configuration information is FFS.”. Due to complexity IEEE 802.1 decided that Gate Control Information for each egress port of a bridge is provided by the CNC (see IEEE Std 802.1Qcc-2018 now integrated in IEEE Std 802.1Q-2022). The complexity is reasoned in the fact that the Gate Control Configuration considers all active stream information and information related to the respective Traffic Classes that are used for the streams. Based on this, there are two options possible where the Gate Control Information is calculated in 5GS, Option 1 at AN-TL/CN-TL or Option 2 at SMF/CUC. An important aspect for both options is that Jitter values cannot be pre-configured. Instead, it is only possible to pre-configure if Jitter occurs or not based on the selected implementation. Respective examples are described in IEEE Std 802.1Q-2022 (former Qcc-2018), Annex U1.1, U1.2, and U1.3. When stream interference with other traffic cannot be avoided (i.e. Jitter occurs), the Jitter needs to be calculated dynamically by SMF/CUC based on arrival times of the streams, supported port bandwidth, and the message sizes. Option 1: foresees that AN-TL/CN-TL determines the Gate Control Information. This means that AN-TL/CN-TL acting as Talker needs following additional information from SMF/CUC: - Stream Identification + InterfaceConfiguration + Jitter (conditional) + List for all Streams for selected Traffic Class (TC) with [Stream Identification, TimeAwareOffset, MaxFrameSize, MaxFramesPerInterval, Interval] This information needs to be provided to AN-TL/CN-TL when a stream is established and also when a stream is terminated, because the information is not stored in the user plane. Option 2: To avoid the permanent data exchange via TL containers, the Gate Control Information calculation is performed in the SMF/CUC. The information exchange is then as follows: - AN-TL/CN-TL provides a list of ports that may act as Talker with TickGranularity and SupportedListMax. If this information is not exchanged via a TL container, it is possible to pre-configure the information at SMF/CUC. - SMF/CUC provides the Stream Identification + InterfaceConfiguration + Gate Control Information to AN-TL/CN-TL (same parameters that are already defined for Gate Control in PMIC). While the complexity to derive Gate Control Information is identical for both options, the information in the TL containers differs between the two options. The proposed changes on Jitter and Input Information to derive Gate Control Information still support both options. The decision whether Option 1 or Option 2 is preferred can be selected based on the TL container content preferences. This is again covered in the first Editor’s Note and allows to resolve this Editor’s Note. - When StreamID is generated, Talker MAC address, PDU Session ID and QFI are not unique over multiple UEs in the same transport network. - The assumption of the study of no connected mode mobility is not yet documented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal is to solve the 3 Editor’s Notes related to TEID and QFI at UNI, stream packet identification, and mapping of Gate Control information. Clarify that SMF/CUC may use the TEID and QFI information to construct IEEE mask-and-match configuration that may be provided to the TN CNC in case mask-and-match stream identification is supported in the TSN TN. TN CNC provides the configuration for mask-and-match stream identification to the intermediate bridges and/or the AN-TL or CN-TL. Document the assumption that no connected mode mobility is guaranteed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solutions for KI#5 TSN enabled TN are incomplete in Rel.18. Risk of non-unique StreamID leading to unpredictable behaviour. Basic assumptions about the solution remain undocumented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.28a.1, Annex M.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * Start of Changes * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[X]	IEEE Std 802.1CBdb-2021: "Amendment 2: Extend Stream Identification Functions".

* * * Next Change * * *

## 5.28a	Support for TSN enabled Transport Network

### 5.28a.1	General

When the 5GS supports interworking with IEEE TSN deployed in the transport network, the CUC that is collocated with SMF interworks with the CNC in the transport network (TN CNC) as specified in clause 46.2 of IEEE Std 802.1Qcc [95]. The SMF/CUC provides the stream requirements on QoS Flow basis (i.e. translated Talker group and Listener group information) via the User/Network-Interface (UNI) to the TN CNC. The TN CNC uses the stream requirements as input to configure respective path(s) and schedules in TN. Based on the results, the TN CNC provides a Status group that contains the end station communication-configuration back to the SMF/CUC.

When interworking with TSN deployed in the transport network is applied, the dynamic value for the CN PDB of a Delay-critical GBR 5QI shall be configured in the SMF as described in clause 5.7.3.4. When the SMF setups a new QoS Flow, the SMF signals the dynamic value for the CN PDB and TSCAI for the QoS Flow to NG-RAN on QoS Flow basis. Upon receiving the TSCAI for a QoS Flow from the SMF, if the TSCAI includes a BAT in UL direction, the RAN may determine a dynamic value of 5G-AN PDB in UL direction for the QoS Flow. The NG-RAN provides the dynamic value of 5G-AN PDB to the SMF in a response to the QoS Flow request. The dynamic value of 5G-AN PDB is used to generate EarliestTransmitOffset as described in Annex M.

The details of providing End Station related information to generate the stream requirements for the QoS Flow by the SMF/CUC are described in Annex M.

If the NG-RAN and UPF support the TSN Talker and Listener functionality (i.e. implement the AN-TL and CN-TL, respectively), the SMF/CUC can communicate with the AN-TL and CN-TL via TL-Container. The TL-Container conveys the data sets defined in IEEE P802.1Qdj [146] between the SMF/CUC and AN-TL and CN-TL.

The AN-TL and CN-TL enable the following functions:

a)	hold and buffer functionality in a case when the TSCAI contains a BAT in UL and/or DL direction.

b)	support of stream transformation functionality with respective information exchange with SMF/CUC.

c)	for SMF/CUC to retrieve the InterfaceCapabilities and/or EndStationInterfaces from the AN-TL or CN-TL.

d)	topology information exchange functionality via LLDP in the TN as described in clause 5.28a.3.

NOTE 1:	How to realize AN-TL in the base station and CN-TL in UPF is up to implementation.

NOTE 2:	In this Release of the specification, it is assumed that connected mode mobility is not used in deployments with a TSN enabled TN.

* * * Next Change * * *

# M.1	Mapping of the parameters between 5GS and TSN UNI

Editor's note:	The information provided here is a guidance for better understanding of the function. Content of this Annex will be updated to a functional level detail once stage 3 work is stable.

The details of the parameters in the TSN UNI are specified in IEEE Std 802.1Q [98]) and IEEE P802.1Qdj [146].

The SMF/CUC derives the End Station related information for the stream requirements towards the TN CNC for the QoS Flow as follows:

a)	For the Talker group:

‐	StreamID: can be generated by the SMF/CUC based on the End Station MAC address acting as Talker, and PDU Session ID and QFI. The MAC address is either pre-configured at the SMF/CUC or provided by the AN-TL or CN-TL to the SMF/CUC (e.g., as part of the EndStationInterfaces information).

-	StreamRank: set to zero for ARP priority values 1-8; set to one for other ARP values.

‐	EndStationInterfaces: If the AN-TL and CN-TL are supported, the SMF/CUC receives the EndStationInterfaces (MacAddress, InterfaceName) from the AN-TL and CN-TL via TL-Container. If the AN-TL and CN-TL are not supported the SMF/CUC sets the information based on pre-configuration.

-	DataFrameSpecification (optional): When it is present it specifies how the TN can identify packets of the TN stream using Ethernet, IP and transport protocol header fields in order to apply the required TSN configuration.

The SMF/CUC may derive the DataFrameSpecification based on:

  - N3 tunnel end point addresses that are used for the QoS Flow. The SMF/CUC may instruct the UPF and NG-RAN to assign a separate N3 tunnel end point address for each QoS Flow that may carry TSC streams so that the TN can distinguish the QoS Flows based on the N3 tunnel destination IP addresses.

NOTE 1:	IPv6 can be used in the N3 tunnel end point addresses to provide sufficient address space in case separate N3 tunnel end point addresses are used for each QoS flow that can carry time sensitive streams.

  - Mask-and-match stream identification parameters (IEEE 802.1CBdb [X] section 9.1.6). The SMF/CUC may indicate mask-and-match configuration based on the TEID and QFI of the given QoS flow to the TN CNC when the deployment supports mask-and-match stream identification function as defined in section 6.8 in IEEE 802.1CBdb [X]. This functionality can be used to check for the TEID and QFI in the GTP header and distinguish the QoS Flows based on the N3 destination TEID and QFI in addition to the header fields that can be filtered using the parameters in the DataFrameSpecification. This enables the TN CNC to configure the mask-and-match stream identification function in the transport network, which may be combined with other stream identification methods. This is an option that allows to use a single GTP-U tunnel as defined for non-TSN Transport networks.

The DataFrameSpecification and mask-and-match stream identification parameters may be provided to the AN-TL and CN-TL to configure stream identification. In that case, the AN-TL and CN-TL can perform the stream identification without relying on additional information from the upper layers of the AN or CN node.

When AN-TL and CN-TL are not supported the TN CNC configures the edge bridge to perform the stream transformation based on the provided the DataFrameSpecification and mask-and-match parameters when applicable.

If the DataFrameSpecification is provided to the TN CNC, the TN CNC configures the edge bridge to perform the stream transformation based on the provided the DataFrameSpecification

‐	TrafficSpecification elements:

‐	Interval: derived from the Periodicity of the traffic as indicated in the TSCAI.

‐	MaxFramesPerInterval: specifies the maximum number of frames that the Talker transmits in one Interval.

‐	MaxFrameSize: derived from the Burst Size of the QoS Flow. The PCF transfers the Burst Size to the SMF/CUC. The SMF/CUC sets MaxFrameSize based on the following formula: Burst Size of the QoS Flow - the framing bits which is not used for transferring in 5GS, (e.g. CRC + the GTP-U tunnel overhead).

-	TransmissionSelection: specifies the algorithm that the Talker uses to transmit the Stream's traffic class. If no algorithm is known, the value zero (strict priority) is used.

‐	TSpecTimeAware group (optional, present only if the traffic in the QoS Flow is time-synchronized):

‐	EarliestTransmitOffset: the earliest offset within the Interval.

For uplink, EarliestTransmitOffset should be set based on the following formula:

TSCAC BAT in UL direction (presented in TAI time and corrected for clock drifting as specified in TS 23.501) + UE-DS-TT Residence Time + 5G-AN PDB - M x Interval, where M is the largest integer for which the relation:

TSCAC BAT in UL direction + UE-DS-TT Residence Time + 5G-AN PDB > M x Interval duration.

would be true.

For downlink, EarliestTransmitOffset should be set based on the following formula:

TSCAC BAT in DL direction (presented in TAI time and corrected for clock drifting as specified in TS 23.501) + UPF Residence Time - M x Interval, where M is the largest integer for which the relation:

TSCAC BAT in DL direction + UPF Residence Time > M x Interval duration.

would be true.

‐	LatestTransmitOffset: the last chance within an interval should leave enough time to transfer a packet with MaxFrameSize. Derived from the end of the interval, the time to transfer a packet with MaxFrameSize.

‐	Jitter: derived in SMF/CUC based on local information on Jitter in AN-TL and CN-TL and respective stream and traffic interference. IEEE Std 802.1Q [98], Annex U1.1, U1.2, and U1.3 provide some examples.

‐	UserToNetworkRequirements:

‐	NumSeamlessTrees: set to one (no redundancy) or other value (if redundancy is required).

‐	MaxLatency: set to CN PDB subtracted by maximum possible buffer duration in Talker. Maximum possible buffer duration is set to LatestTransmitOffset subtracted by EarliestTransmitOffset up to Buffer capability sent by the Talker.

-	InterfaceCapabilities (optional): If the AN-TL and CN-TL are supported, the SMF/CUC collects InterfaceCapabilities from AN-TL and CN-TL via TL-Container. If the AN-TL and CN-TL are not supported, the SMF/CUC leaves the InterfaceCapabilities empty.

b)	For the Listener group:

-	Stream ID and Stream Rank: that were generated for the Talker of the TN stream are also used by the SMF/CUC for the Listener.

-	EndStationInterfaces: derived as with the corresponding information for the Talker group.

-	UserToNetworkRequirements:

‐	NumSeamlessTrees: set to one.

‐	MaxLatency: derived as with the corresponding information for the Talker group.

-	InterfaceCapabilities: derived as with the corresponding information for the Talker group.

c)	For the Status group: The Status group contains the end station communication-configuration provided by TN CNC to the SMF/CUC:

‐	Stream ID.

‐	StatusInfo.

‐	AccumulatedLatency: If the AccumulatedLatency is included from TN CNC to SMF/CUC for a stream in DL direction, the SMF/CUC may use the AccumulatedLatency to update the TSCAI BAT to the NG-RAN; the SMF sets the TSCAI Burst Arrival Time in downlink direction as the sum of the TSCAC BAT value in downlink direction and AccumulatedLatency and the buffer duration in Talker in CN-TL. The buffer duration in CN-TL is zero if TimeAwareOffset for the Talker group is not present, and TimeAwareOffset - EarliestTransmitOffset if the TimeAwareOffset is present for the Talker group.

‐	InterfaceConfiguration (optional):

‐	MAC Address (optional, present only if the respective InterfaceCapability contains a value for Active Destination MAC and VLAN Stream identification in CB-StreamIdenTypeList, and stream transformation is performed in AN-TL and CN-TL).

‐	VLAN Tag (optional, present only if the respective InterfaceCapability contains that it is VlanTagCapable and a value for Active Destination MAC and VLAN Stream identification in CB-StreamIdenTypeList, and the stream transformation is performed in AN-TL and CN-TL).

‐	IPv4/IPv6 Tuples (optional, but not supported in this release of the specification).

‐	TimeAwareOffset (optional, present only if the traffic is time-synchronized, AN-TL and CN-TL is supported, and TSpecTimeAware elements were provided in the stream requirements).

If the InterfaceConfiguration is included, , and AL-TL/CN-TL acting as Talker End Station support the Stream Transformation as described in IEEE Std 802.1Q [98], the SMF/CUC can instruct the UPF and NG-RAN to assign for each stream an individual TSN Transport address by providing the InterfaceConfiguration to the AN-TL/CN-TL via TL-Container. The Talker in AN-TL/CN-TL shall use the indicated InterfaceConfiguration, e.g. source MAC address, multicast destination MAC address, VLAN ID, as assigned by the TN CNC for the data stream in a QoS Flow. The TN can identify the streams based on the Stream Transformation that is applied in the AN-TL/CN-TL acting as Taker End Station. This allows to use a single GTP-U tunnel as defined for non-TSN Transport networks.

If the TimeAwareOffset is included, the SMF/CUC should derive input information based on the TimeAwareOffset, which allows the configuration of Gate Control information as defined in IEEE Std 802.1Q [98] at the AN-TL (for streams in UL direction) and the CN-TL port (for streams in the DL direction), i.e., AdminBaseTime, AdminCycleTime, AdminControlListLength, and AdminControlList). The AN-TL or CN-TL acting as Talker buffers the data burst until the time indicated in the TimeAwareOffset is reached.

If the SMF/CUC receives a TimeAwareOffset from TN CNC for a downlink stream (i.e. for a Talker in the UPF/CN-TL), the SMF/CUC adds the received TimeAwareOffset value to the TSCAI BAT in the DL direction in the TSCAI and updates the NG-RAN for the new TSCAI.

‐	FailedInterfaces (optional) provides a list of one or more physical ports of failed end stations or bridges to locate the interfaces in the physical topology that caused the failure. It is up to implementation how the SMF reacts when it receives FailedInterfaces.

NOTE 2:	It is assumed that the end station communication-configuration will contain at least the same information as defined for the status.

NOTE 3:	If Jitter value needs to be considered; EarliestTransmitOffset for UL and DL and LatestTransmitOffset shall be Jitter corrected. How Jitter correction is carried out is up to implementation.

# * * * * End of changes * * * *