3GPP TSG SA WG2 Meeting #156 e-meeting	S2-2304083r01

# April 17 - 21, 2023, Elbonia																revision of S2-230xxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1286 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE trigger for IMS Data Channel setup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is unclear how the UE decides to establish an IMS DC session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the UE does not establish a IMS DC session if the network has not indicated support for IMS DC. How the UE is configured with information when to establish an IMS DC session is not specified in this release. The UE can establish an IMS DC session during or after multimedia session establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete feature description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.4, AC.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# AC.4	IMS DC Channel Setup

The following principles apply when an IMS Data Channel is established: -

-	UE can establish an IMS Data Channel simultaneously while establishing an IMS audio/video session or upgrade an ongoing IMS audio/video session through a re-INVITE to an IMS Data Channel session.

NOTE 1:	An IMS Data Channel established simultaneously with an IMS audio/video session can have impact to the IMS session setup time.

-	The trigger for a UE to initiate a DC establishment request is left for UE implementation. The UE shall not initiate a DC establishment request if the network has not indicated support for IMS DC service during IMS registration.

NOTE 2:	How the UE is configured with the information whether to initiate an IMS Data Channel is not specified in this release of the specification.

-	If a UE that has not subscribed to IMS Data Channel establishes an IMS Data Channel simultaneously with an IMS audio/video IMS session, the IMS AS shall discard the Data Channel request and proceed with the audio/video IMS session establishment.

-	The DCSF provides a URL to the DCMF or MRF via the IMS AS during data channel media resource reservation for the bootstrap data channel that enables downloading a subscriber specific graphical user interface to the UE via the Bootstrap Data Channel. Once the DCMF or MRF, acting as HTTP proxy, receive a HTTP GET Request containing the root ("/") URL through the bootstrap data channel, the DCMF or MRF replaces the root ("/") URL in the HTTP GET Request with the subscriber specific URL and forward the request to the DCSF for downloading the subscriber specific graphical user interface to the UE. The UE shall maintain the HTTP session state when interacting with the DCMF.

# AC.5	Binding of DC Application with the related DC

Information for the binding of DC Application with the related DC is required to support multiple, simultaneous DC applications in a UE.

The binding information is allocated by the HPLMN or retrieved from DC application provider when a DC Application is uploaded to DCSF. DCSF may be configured with a DC application profile associated with the binding information, which indicates the DC control policy (e.g. whether the Application DC establishment follows the P2P or P2A/A2P procedure). The UE receives the binding information of a DC application via the Bootstrap DC, e.g. when the DC application is downloaded to the UE.

When the UE is establishing an IMS Application DC as defined in clause x.y, the binding information is provided by the UE in the SDP offer as described in TS 26.114 [76]. The IMS AS provides the binding information to the DCSF. The DCSF may use the binding information for controlling the Application DC setup .

Editor's note:	The reference number of clause x.y is to be updated in the subsequence meeting.

Editor's note:	The binding information and its usage is defined by SA WG4. This clause needs to be aligned with the outcome of SA WG4 work on the binding information.

**** Next Change ****

# AC.8	Procedures

## AC.8.1	IMS DC capability negotiation

The IMS network and the UE need to mutually negotiate the capability of supporting IMS data channel mutually.

IMS data channel capability negotiation includes two aspects:

-	The network discovers the data channel capability of the UE.

-	The UE discovers the data channel capability of the network.

When the UE supporting IMS data channel registers on the IMS network, it includes the media feature tag as specified in TS 26.114 [76] in the Contact header field of the initial REGISTER request to allow the home IMS network discovers its IMS data channel capability.

If the IMS network supports IMS data channel, the S-CSCF includes a Feature-Caps header field indicating its data channel capability in the 200 OK response to the initial and any subsequent REGISTER request, which is used by the UE to discover the IMS data channel capability of its home IMS network.

NOTE:	The UE can receive a Feature-Caps header field indicating its data channel capability in the 200 OK response to a subsequent REGISTER request when the network starts supporting IMS data channel after successful initial registration of the UE.

When the UE supporting IMS data channel initiates an IMS session, it includes the media feature tag as specified in TS 26.114 [76] in the Contact header field of the initial INVITE or a re-INVITE request to remote UE, regardless of data channel media being part of the SDP or not.

The UE shall not include data channel media description in the SDP offer of the initial INVITE request or any subsequent re-INVITE, if the S-CSCF has not included the data channel capability indication in the Feature-Caps header field in the 200 OK response either to the REGISTER or subsequent REGISTER request.

**** End of Changes ****