3GPP TSG-SA2 Meeting #156E	 S2-2304100

E-Meeting April 17 - 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2083 | Rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Content of home network domain when SUPI is IMSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is stated that: “The SUPI for a 5G-CRG containing IMSI is described in clause 5.9.2 of TS 23.501 [2]. The home network domain of SUPI may include the NID of SNPN as defined in clause 28.2 of TS 23.003 [14].” However, when SUPI is IMSI, then the home network domain of SUPI does not contain any NID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the statement that home network domain of SUPI may include the NID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Statement cannot be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 4.16	Access to SNPN services via wireline access network

Access to SNPN defined in clause 5.30 of TS 23.501 [2] via a wireline access network follows the same specification and procedures used for accessing a PLMN via a wireline access with the following modifications:

-	The SNPN is implicitly selected by wired physical connectivity between 5G-RG or FN-RG and W-AGF.

-	In the case of 5G-RG connected via FWA, the 5G-RG shall support the capability defined for the SNPN-enabled UE as specified in clauses 5.30.2.3 and 5.30.2.4.1 of TS 23.501 [2] where the UE is replaced by 5G-RG.

-	For FN-RG(s), the W-AGF may be configured to use a SNPN Identifier (as defined in clause 5.30.2.1 of TS 23.501 [2]) instead of a PLMN Identifier in the procedure for FN-RG Registration via W-5GAN defined in clause 7.2.1.3.

-	The SUPI for a 5G-CRG containing IMSI is described in clause 5.9.2 of TS 23.501 [2]. The SUPI in NSI type may include the NID of SNPN as defined in clause 28.7.2 of TS 23.003 [14].

-	The SUPI for a 5G-CRG and FN-CRG containing GCI is specified in clause 4.7.4 and in clause 28.15.2 of TS 23.003 [14]. The realm part of NAI format for a SUPI containing a GCI identifying the operator owning the subscription may include the NID of the SNPN.

-	The SUPI for a 5G-BRG and FN-BRG containing GLI is specified in clauses 4.7.2, 4.7.3 and 28.16.2 of TS 23.003 [14]. The realm part of NAI format for a SUPI containing a GLI identifying the operator owning the subscription may include the NID of the SNPN.

-	RG using credentials owned by a Credentials Holder separate from the SNPN is not applicable. UE behind RG accessing to SNPN with credentials owned by Credentials Holder is supported as specified in clause 5.30.2.9 of TS 23.501 [2].

-	The onboarding procedure specified in clause 5.30.2.10 of TS 23.501 [2] is not applicable in this Release for a RG.

END CHANGE