3GPP TSG SA WG2 Meeting #156-E	S2-2304116

# Electronic meeting, April 17-21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2085 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Differentiation for UEs behind 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Diffrentiated QoS may be provided for UEs behind 5G-RG. Based on the conclusions for UE behind RG in TR 23.700-17 an overlay-underlay network similarly as described in clause D.7 of TS 23.501 is used and the QoS differentiation is done based on the DSCP markings for the IPSec child SAs. Since the QoS and charging differentiation is based on network configuration and re-using existing procedures, the issue is covered in an informative annex. |  |  |  |  |  |  |  |  |
| Summary of change: |  | QoS and charging differentiation is explained in an informative annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not capture features defined in the study conclusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.316 CR#2086 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Reference to 4.10.X in Annex X refers to a new clause proposed in TS 23.316 CR#2086 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## **** First Change ****

## 4.10	UE behind 5G-RG and FN-RG

An RG connecting via W-5GAN or NG-RAN access towards 5GC can provide connectivity for a UE behind the RG to access an N3IWF or TNGF. It is assumed that the UE is 5GC capable, i.e. supports untrusted non-3GPP access and/or trusted non-3GPP access. This allows the RG, W-5GAN and the RG's connectivity via 5GC to together act as untrusted/trusted N3GPP access to support UEs behind the RG.

When FN-RG/5G-RG is serving a UE, the control and user plane packets of the UE is transported using a FN-RG/5G-RG IP PDU session and then from PSA UPF of that PDU session to an IWF. A single FN-RG/5G-RG IP PDU session can be used to serve multiple UEs.

Figure 4.10-1 shows the non-roaming architecture for a UE, behind a 5G-RG, accessing the 5GC via TNGF where the combination of 5G-RG, W-5GAN and UPF serving the 5G-RG is acting as a trusted Non-3GPP access network.

NOTE 1:	FN-RG and W-5GAN acting as trusted Non-3GPP access is not considered in this specification as it is assumed that FN-RG does not support EAP-based access control (e.g. 802.1X).

Figure 4.10-1: Non-roaming architecture for UE behind 5G-RG using trusted N3GPP access

The 5G-RG can be connected to 5GC via W-5GAN, NG-RAN or via both accesses. The UE can be connected to 5GC via 5G-RG, NG-RAN or via both accesses.

The TNGF and Ta reference point are defined in TS 23.501 [2].

NOTE 2:	The reference architecture in figure 4.10-1 only shows the architecture and the network functions directly connected to W-5GAN or TNGF, and other parts of the architecture are the same as defined in clause 4.2 of TS 23.501 [2].

NOTE 3:	The reference architecture in figure 4.10-1 supports service based interfaces for AMF, SMF and other NFs not represented in the figure.

NOTE 4:	The two N2 instances in Figure 4.10-1 apply to a single AMF for a 5G-RG which is simultaneously connected to the same 5G Core Network over 3GPP access and W-5GAN.

NOTE X: 	Support for QoS differentiation can be achieved in a similar way as it is handled when a UE connects to a PLMN via SNPN (as defined in clause 5.30.2.7 and clause D.7 of TS 23.501[2]). Also differentiated charging, both in the RG's PLMN and in the UE's PLMN, can be achieved based on existing mechanisms. This is further described in Annex X.

## **** Next Change ****

# Annex X (informative):	Support for differentiated charging and QoS for UEs behind RG

For the traffic of UEs behind a RG, QoS differentiation in the RG's PDU Session can be provided on a per UE’s-IPsec Child Security Association basis. The UE's N3IWF/TNGF determines the IPsec child SAs as defined in clauses 4.12 and 4.12a of TS 23.502 [3] as well as the DSCP value used in the outer IP header of that IPsec child SA. It is assumed that the same set of DSCP values and corresponding QoS are applicable independent of whether UE-requested or network-initiated QoS is used.

To support QoS differentiation for the UE’s traffic, QoS mapping rules between the RG's 5GC and the UE's 5GC are governed by an SLA (or network configuration in case of single operator), which includes the mapping between the DSCP marking for the IPsec child SAs and the corresponding QoS parameters and also the N3IWF/TNGF IP address(es). The non-alteration of the DSCP field on NWu/NWt is also governed by the SLA and by transport-level arrangements that are outside of 3GPP scope. The SLA also governs the RG PDU session IP addresses.

The RG’s PCF and SMF may provide PCC rules and QoS rules for the available mappings as determined by the SLA. The packet detection filters in the RG's UPF can be based on the N3IWF/TNGF IP address and the DSCP markings on NWu/NWt.

UE’s SMF/PCF may use the UE's local IP address, which is the N6 address of the RG’s PDU session, to enable differentiated QoS and charging when the UE is accessing N3IWF/TNGF via a W-5GAN.

Differentiated charging is enabled by the awareness of N3IWG/TNGF and RG PDU Session IP addresses and also the mapping between DSCP marking and QoS parameters included in the SLA.

## **** End of Changes ****