3GPP TSG-SA WG2 Meeting #156 (e-meeting)	 S2-2304148

# Elbonia, April 17 – 21, 2023																		(rev of )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0207 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Correction of mobility procedures for delivery of multicast MBS session data to RRC_INACTIVE UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | #1 Regarding SMF providing MBS Assistance Information to target RAN To provide MBS assistance information to the NG-RAN node during various mobility procedures, the following is specified in clause 7.2.3.8.1. 7.2.3.8.1 General The procedures in clause 7.2.3.2, 7.2.3.3, 7.2.3.4, 7.2.3.6 and 7.2.3.7 apply with the following additions: - If MBS assistance information is available at the SMF, depending on configuration the SMF may always provide the MBS session information comprising the MBS assistance information for the MBS session related to the MBS session that UE joined towards the target RAN node. This includes the following procedures: - For the Xn based handover procedure, if the target RAN node supports MBS, after the successful handover the SMF triggers the modification of the associated PDU session to provide the MBS assistance information for the MBS session within the MBS session information to target NG-RAN. - During the N2 based handover preparation phase, the SMF provides the MBS assistance information in the MBS session information to target NG-RAN. - During the connection resume procedure, if the target RAN node supports MBS, after acknowledging the path switch request, the SMF triggers the modification of the associated PDU session to provide the MBS assistance information for the MBS session within the MBS session information to target NG-RAN. The bullet “For the Xn based handover procedure…” reads as SMF always triggers modification of associated PDU Session, which is not aligned with clause 7.2.3.1 and clause 7.2.3.4, that is, SMF does not trigger modification of associated PDU Session in clause 7.2.3.1 but does in clause 7.2.3.4. The bullet “During the connection resume procedure…” also reads as SMF always triggers modification of associated PDU Session which is not aligned with clause 7.2.3.7 that the SMF may or may not trigger modificaton of associated PDU Session. To capture the difference from existing clauses in a simplified way, the following principles are proposed: If there is existing signaling from SMF to target RAN, the SMF includes additionally the MBS assist info. If there is no existing signaling from SMF to target RAN, the SMF may, depending on configuration, trigger modification of associated PDU Session to provide the MBS assistance info. #2 Description about RAN internal behavior In clause 7.2.3.8.1, RAN internal behaviors are described, however, the RAN aspects should be left to RAN. If there is a need to clarify, NOTE can be added by referring to the RAN spec later. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #1 Simplify the description for Xn/N2-based handover and Connection Resume regarding how SMF provides MBS Assistance Informatiion #2 Remove the RAN specific description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement incorrectly specified RAN internal function must not be specified in SA2 spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 7.2.3.8	Mobility procedures to enable delivery of multicast MBS session data to UEs in RRC_INACTIVE state

##### 7.2.3.8.1	General

The procedures in clauses 7.2.3.2, 7.2.3.3, 7.2.3.4, 7.2.3.6 and 7.2.3.7 apply with the following additions:

-	If the MBS assistance information is available at the SMF,

-	For N2-based handover, when the SMF provides the MBS session related information as part of the associated PDU Session context data as specified in the existing procedures, the SMF also includes the MBS Assistance Information in the N2 SM information.

-	For Xn-based handover or Connection Resume,

-	if, after path switch acknowledge, the SMF provides MBS session related information as part of the associated PDU Session context data as specified in the existing procedures, the SMF also includes the MBS Assistance Information in the N2 SM information.

-	If, after path switch acknowledge, the SMF does not provide MBS session related information as part of the associated PDU Session context data in the existing procedures, the SMF may, depending on configuration, initiate modification of associated PDU Session to provide the MBS assistance information.

-	The MBS session information transferred from source NG-RAN towards target NG-RAN also include MBS assistance information for the MBS session if such information is available at the source RAN node.

For a UE that is receiving multicast MBS data in CM-CONNECTED with RRC_INACTIVE state, the scenarios for UE mobility are as follows:

-	UE moves to new cell within the RAN Notification Area (RNA).

-	UE moves outside the current RAN Notification Area but within the current Registration Area (RA).

-	UE moves out of the current Registration Area.

* * * End of Changes * * *