3GPP TSG-SA WG2 Meeting #156 (e-meeting)	 S2-2304153r03

# Elbonia, April 17 – 21, 2023																		(rev of )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0210 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 Resolve EN in broadcast procedures for resource sharing in network sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clauses 7.3.1a (MBS Session Start for resource sharing across multiple broadcast MBS Sessions during network sharing) and 7.3.2a (MBS Session Release for resource sharing across multiple broadcast MBS Sessions during network sharing) contain the following EN: Editor's note: Additions with respect to N3mb tunnel management is FFS. ### Resolution of the above EN has a dependency on how to resolve the inconsistency (see below) between the steps in figures and text description in clause 7.3.1 and 7.3.2 from Rel-17 in CR0245 (Rel-17) and CR0246(Rel-18): In clause 7.3.1, step 5 in Figure 7.3.1-1 (NG-RAN joining multicast group over N3mb) is not aligned with the text description (that both unicast and multicast N3mb are described). In clause 7.3.2, same issue happens to step 6 in Figure 7.3.2-1. Besides, regarding how NG-RAN identifies the associated MBS Sessions in clauses 7.3.1a, it is proposed to replace “via a configured association of TMGIs or via the same Associated Session Identifier being received for the associated sessions” with “as specified in clause 6.18” to avoid repetition and inconsistency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clauses 7.3.1a and 7.3.2a, resolve ENs by specifying the enhancement to handle the unicast N3mb transport and the multicast N3mb transport. Clauses 7.3.1a, replace “via a configured association of TMGIs or via the same Associated Session Identifier being received for the associated sessions” with “as specified in clause 6.18” to avoid repetition and inconsistency. Align the title of referenced clause 7.3.7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN remain unresolved, risk of inconsistency |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1a, 7.3.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.247 CR0246 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The enhancement in this paper depends on the correction in TS 23.247 CR0246. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 7.3.1a	MBS Session Start for resource sharing across multiple broadcast MBS Sessions during network sharing

To support resource sharing across multiple broadcast MBS Sessions during network sharing (see clause 6.17), in addition to clause 7.3.1, the following enhancement applies:

-	Step 2: The MB-SMF includes Associated Session ID in the N2 SM information in Namf_MBSBroadcast_ContextCreate.

-	Step 4: A shared NG-RAN node identifies the associated MBS sessionsas specified in clause 6.18. The associated MBS sessions share radio resource configuration based on the existing Broadcast MBS Session context.

-	Step 5a: If the NG-RAN determines there is already an existing broadcast MBS session delivering the same content, the NG-RAN may skip joining the multicast group based on operator policy, i.e., step 5a can be skipped.

-	Step 5b: If the NG-RAN determines there is already an existing broadcast MBS session delivering the same content, a shared NG-RAN node may decide not to allocate N3mb DL tunnel.

-	Step 6: For N3mb unicast transport, if the NG-RAN does not allocate N3mb DL Tunnel in step 5b, the N3mb DL Tunnel Info is not present in MBS Session Resource Setup Response.

-	Step 8: If the N3mb unicast transport is used but N3mb DL Tunnel Info is not present, step 8 is skipped.

-	Step 9: If the NG-RAN determines that radio resource has been allocated for another broadcast MBS Session delivering the same content as specified in clause 6.18, the NG-RAN advertises the TMGI of the broadcast MBS session, and reuses such already allocated radio resource for the MBS data transmission for the broadcast MBS session.

NOTE:	The 5G QoS Profile values for the broadcast MBS sessions are assumed to be pre-agreed amongst the multiple operators participating in the network sharing, and are the same irrespective of the PLMN from which the MBS Session Start is sent.

-	Step 10 ~ step 12:

The handling is the same as steps 5, 6, 7 and 8.

-	Step 14: If the user plane of a broadcast MBS session from its MB-UPF to the shared NG-RAN is not established, the NG-RAN will not receive the MBS packets from the MB-UPF.

-	Step 15: If the NG-RAN receives multiple DL media streams for the same content represented by the same Associated Session ID via multiple CNs, the NG-RAN should deliver only one DL media stream.

* * * Next Changes * * *

### 7.3.2a	MBS Session Release for resource sharing across multiple broadcast MBS Sessions during network sharing

To support resource sharing across multiple broadcast MBS Sessions, in addition to clause 7.3.2, the following enhancement applies:

-	If the DL N3mb Tunnel for the broadcast MBS Session was not allocated previously in clause 7.3.1a, the NG-RAN does not have DL N3mb Tunnel to release.

Otherwise, if the DL N3mb Tunnel for the broadcast MBS Session was allocated previously, and if the NG-RAN determines there are other associated broadcast MBS Sessions based on the Associated Session Identifier, the NG-RAN may trigger the procedure of Transport change for resource sharing across broadcast MBS Sessions during network sharing as specified in clause 7.3.7.

-	Step 6a:

-	If the NG-RAN skipped joining the multicast group (LL SSM) in clause 7.3.1a, the NG-RAN does not send a Leave message (LL SSM),

-	Otherwise, if the NG-RAN joined the multicast group (LL SSM) previously, and if the NG-RAN determines there are other associated broadcast MBS Sessions based on the Associated Session Identifier, the NG-RAN may trigger procedure of Transport change for resource sharing across broadcast MBS Sessions during network sharing as specified in clause 7.3.7.

-	Step 6b:

-	If the DL N3mb Tunnel for the broadcast MBS Session was not allocated previously in clause 7.3.1a, step 6b is skipped.

Otherwise, if the DL N3mb Tunnel for the broadcast MBS Session was allocated previously, and if the NG-RAN determines there are other associated broadcast MBS Sessions based on the Associated Session Identifier, the NG-RAN may trigger the procedure of Transport change for resource sharing across broadcast MBS Sessions during network sharing as specified in clause 7.3.7.

* * * End of Changes * * *