# 3GPP TSG-SA WG2 Meeting #156E	S2-2304193

Electronic Meeting, 17 - 21 April, 2023	(Revision of S2-230xxxx)

Title:	[Draft]Reply LS on Network Triggered Service Request for a UE in Suspend State

Response to:	LS on Network Triggered Service Request for a UE in Suspend State (S2-2303941/C4-230664)

Release:	Rel-17

# Work Item:	     5G_CIoT, TEI17

Source:	SA2

To:	CT4

cc:	RAN3

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks CT4 for questions related to the handling of network triggered service request for UE in CM-IDLE with suspend. SA2 would like to provide following answers.

Question1: What information should the SMF include in the N2 Information for the N1N2MessageTransfer Request message?

SA2 answer: In case of CM-IDLE mode with suspend, SA2 consider it is possible to have a dummy N2 information if there is no N1 message to delivery. It’s up to CT4 to decide the details of the dummy N2 message.

Question2: Upon subsequent Connection Resume in CM-IDLE with Suspend procedure as response to the paging, how should the AMF deal with the N2 Information received in the N1N2MessageTransfer Request triggering the paging (the AMF is expected to send an N2 Connection Resume Response or N2 Path Switch Ask to NG-RAN, see clause 4.8.2.3 of 23.502)?

SA2 answer: When the UE resumes (either via NGAP Connection Resume Request or Path Switch Request), the AMF follows the flow in clause 4.8.2.3 of 23.502. The dummy N2 message can be discarded by AMF.

2. Actions:

To CT4

ACTION: 	SA2 kindly asks CT4 to take above information into consideration and discuss the possible solution. SA2 also kindly asks CT4 to further synch with SA2 once a solution is agreed.

3. Date of Next SA2 Meeting:

SA2#157                   	May 22 - 26, 2023                                 	Berlin, DE

SA2#158   		August 21 - 25, 2023 				Goteborg, SE