SA WG2 Meeting #156E	S2-2304208

# 17-21 April 2023, Electronic meeting	(was S2-220xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4240 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Closing ENs for the procedures for discontinuous coverage reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#155 a baseline was agreed for the procedure of the reporting UE out-of-coverage period in NAS and AMF reporting expected unavailability duration to the UE. Several open issues were documented and captured with related ENs. This CR is aiming to resolve the open issues and close the related ENs. - Editor's note: Capability negotiation between UE and network for the features introduced for discontinuous network coverage for satellite access is FFS. It is proposed that there are two separate " UE 5GMM Core Network Capabilities": 1) Unavailability Period reporting (existing one defined in SUECR), 2) Support for discontinuous coverage without UE being able to determine its own UE out-of-coverage period. - Editor's note: It is FFS whether the AMF can provide an expected unavailability duration in the Registration Accept also if the UE provided an out-of-coverage time the Registration Request. Assuming the format of satellite coverage availability information that is sent to the UE is standardised and therefore is reliable (from the perspective of PLMN), we don’t see the need for AMF to provide expected unavailability duration in the Registration Accept if the UE reports its own out-of-coverage period. Clause 5.4.13.1 states: To avoid signalling overload, the Mobility Registration Update procedure as described above shall be triggered during a time range before the UE loses coverage, as many UEs may lose coverage at the same time. However, there is currently no definition or support for the time range. This is possible by extending Overload control in clause 5.4.13.5. This results in sending the Mobility Registration Update sometime before an Unavailability period would start which requires adding a Start time to clause 5.4.1.4. The presence of a Start time allows use of a periodic or mobility Registration Update to send an unavailability period to the AMF which can reduce signalling. It is proposed to replace the existing text in clause 5.4.13.2 as proposed below: A UE may use satellite coverage availability information for satellite access to support discontinuous coverage operations. Satellite coverage availability information can be provided to a UE by an external server via a PDU Session or SMS as defined in CT1 specifications. - Editor's Note: The specification that will define the protocol and format of satellite coverage availability information via PDU session or SMS will be defined by CT1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Closes open issues regarding the procedures for discontinuous coverage reporting; extend overload control to UE notification of loss of coverage; add a start time to Support of Unavailability Period; allow nortification of loss of coverage using a periodic Mobility Registration Update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Open issues remain in the specification. ENs remain. Signalling load will be higher. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.4.1.4, 5.4.4a, 5.4.13.1, 5.4.13.2, 5.4.13.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

# >>>> NEXT CHANGES<<<<

#### 5.4.1.4	Support of Unavailability Period

During Registration procedure, the UE supporting the Unavailability Period feature provides "Unavailability Period Support" indication as part of 5GMM Core Network Capability in Registration Request message for initial registration and for every mobility registration. The AMF indicates whether the corresponding feature is supported in the AMF by providing the "Unavailability Period Supported" indication in Registration Accept message.

If the UE and network support Unavailability Period and an event is triggered in, or detected by, the UE that would make the UE unavailable for a certain period of time, the UE may store its MM and SM context in USIM or Non Volatile memory to be able to reuse it after its unavailability period. If the UE can store its contexts the UE may trigger Mobility Registration Update procedure otherwise the UE shall trigger UE-initiated Deregistration procedure.

NOTE:	How the UE stores its contexts depends upon the UE implementation. The UE can store some or all of its contexts in the USIM using existing USIM functionality.

Before the start of an event that makes the UE unavailable, the UE includes optionally the Start of the Unavailability Period and the Unavailability Period Duration and triggers either Mobility Registration Update or UE initiated Deregistration procedure:

Editor's note:	Coding of the Start of the Unavailability Period will be decided by CT1 and could be UTC or a time relative to the time of receipt of the Unavailability Period at the AMF.

a)	If the UE initiates Mobility or Periodic Registration Update procedure:

1)	The AMF may take the Start of the Unavailability Period (if present) and the Unavailability Period Duration into account when determining Periodic Registration Update timer value. If the Start of Unavailability Period is not present in Mobility Registration Update the AMF considers implicitly the start of the unavailability period to be the time it has received the Registration Update. The AMF may provide a Periodic Registration Update time longer than or equal to Unavailability Period Duration to avoid interfering with the UE dealing with the event that causes the unavailability;

2)	The AMF stores the information that the UE is unavailable in UE context, and considers the UE is unreachable (i.e. clear the PPF in AMF) until the UE enters CM-CONNECTED state;

3)	While the UE is unreachable, all high latency communication solutions (see clause 5.31.8) apply if supported in the network, e.g. extended data buffering, downlink data buffering status report, etc; and

4)	If there is "Loss of Connectivity" event subscription for the UE by AF, the AMF considers the remaining time in the Unavailability Period when constructing the "Loss of Connectivity" event report towards the NEF and the Unavailability Period is reported to the respective subscribed AF.

b)	If the UE initiates UE-initiated Deregistration procedure:

1)	If there is "Loss of Connectivity" event subscription for the UE by AF, the AMF considers the remaining time in the Unavailability Period when constructing the "Loss of Connectivity" event report towards the NEF and the Unavailability Period is reported to the respective subscribed AF;

Once the event which makes the UE unavailable is completed in the UE or the event is delayed to a future time or cancelled in the UE, the UE triggers Registration procedure to resume regular service. The UE shall not include the Unavailability Period in this Registration Request message. Depending on the UE state, the Registration procedure can be Initial Registration procedure or Mobility Registration Update procedure.

# >>>> NEXT CHANGES<<<<

### 5.4.4a	UE MM Core Network Capability handling

The UE MM Core Network Capability is split into the S1 UE network capability (mostly for E-UTRAN access related core network parameters) and the UE 5GMM Core Network Capability (mostly to include other UE capabilities related to 5GCN or interworking with EPS) as defined in TS 24.501 [47] and contains non radio-related capabilities, e.g. the NAS security algorithms, etc. The S1 UE network capability is transferred between all CN nodes at AMF to AMF, AMF to MME, MME to MME, and MME to AMF changes. The UE 5GMM Core Network Capability is transferred only at AMF to AMF changes.

In order to ensure that the UE MM Core Network Capability information stored in the AMF is up to date (e.g. to handle the situation when the USIM is moved into a different device while out of coverage, and the old device did not send the Detach message; and the cases of inter-RAT Registration Area Update), the UE shall send the UE MM Core Network Capability information to the AMF during the Initial Registration and Mobility Registration Update procedure within the NAS message.

The AMF shall store always the latest UE MM Core Network Capability received from the UE. Any UE MM Core Network Capability that an AMF receives from an old AMF/MME is replaced when the UE provides the UE MM Core Network Capability with Registration signalling.

If the UE's UE MM Core Network Capability information changes (in either CM-CONNECTED or in CM-IDLE state), the UE shall perform a Mobility Registration Update procedure when it next returns to NG-RAN coverage. See clause 4.2.2 of TS 23.502 [3].

The UE shall indicate in the UE 5GMM Core Network Capability if the UE supports:

-	Attach in EPC with Request type "Handover" in PDN CONNECTIVITY Request message (clause 5.3.2.1 of TS 23.401 [26]).

-	EPC NAS.

-	SMS over NAS.

-	LCS.

-	5G SRVCC from NG-RAN to UTRAN, as specified in TS 23.216 [88].

-	Radio Capabilities Signalling optimisation (RACS).

-	Network Slice-Specific Authentication and Authorization.

-	Network Slice Replacement as described in clause 5.15.19.

-	Parameters in Supported Network Behaviour for 5G CIoT as described in clause 5.31.2.

-	Receiving WUS Assistance Information (E-UTRA) see clause 5.4.9.

-	Paging Subgrouping Support Indication (NR) see clause 5.4.12.

-	CAG, see clause 5.30.3.3.

-	CAG with validity information (if UE supports CAG), see clause 5.30.3.3.

-	Subscription-based restrictions to simultaneous registration of network slices (see clause 5.15.12).

-	Support of NSAG (see clause 5.15.14).

-	Partial Network Slice support in a RA (see clause 5.15.17).

-	Minimization of Service Interruption (MINT), as described in clause 5.40.

-	Equivalent SNPNs (see clause 5.30.2.11).

-	Unavailability Period reporting, as described in clause 5.4.1.4.

-	Support for network reconnection due to RAN timing synchronization status change, see clause 5.3.4.4.

-	UE Configuration of network-controlled Slice Usage Policy (see clause 5.15.15.2).

-	Temporarily available network slices (see clause 5.15.16).

-	Support of S-NSSAI location availability information, as described in clause 5.15.18.2.

-	Support for discontinuous coverage.

The UE shall indicate "Support for discontinuous coverage" if the UE supports procedures in clause 5.4.1.4 and indicates, "Unavailability Period reporting" as supported.

If a UE operating two or more USIMs, supports and intends to use one or more Multi-USIM features (see clause 5.38) in a PLMN for a USIM, it shall indicate in the UE 5GMM Core Network Capability for this USIM in this PLMN that it supports these one or more Multi-USIM features with the following indications:

-	Connection Release Supported.

-	Paging Cause Indication for Voice Service Supported.

-	Reject Paging Request Supported.

-	Paging Restriction Supported.

Otherwise, the UE with the capabilities of Multi-USIM features but does not intend to use them shall not indicate support of these one or more Multi-USIM features.

A UE not operating two or more USIMs shall indicate the Multi-USIM features are not supported.

NOTE:	It is not necessary for a UE operating two or more USIMs to use Multi-USIM features with all USIMs.

# >>>> NEXT CHANGES<<<<

#### 5.4.13.1	Mobility Management and Power Saving Optimization

For NR satellite access that provides discontinuous network coverage, a UE out-of-coverage period may be determined, which indicates the timing information for when a UE is expected to be out of coverage, and when the UE is expected to re-gain coverage again. The UE out-of-coverage period may consider current and expected future locations of the UE.

Tracking Area or RAT specific configuration in the AMF may be used to set timer values based on typical coverage periods of a satellite system.

NOTE 1:	For example, if a satellite system only provides coverage to a UE for 20 minutes when a satellite passes, and the maximum time before a satellite passes any point on the earth is 10 hours, the AMF could configure the periodic registration timer and Mobile Reachable timer to be just greater than 20 minutes and the Implicit Deregistration timer to be greater than 10 hours to avoid unintended implicit detach due to coverage gap. Such configuration does not require AMF to be aware of detailed coverage times for each UE or for different locations.

"Discontinuous coverage" includes UE notification to the network when the UE is about to leave from or return to coverage (as described below) and overload control for discontinuous coverage as defined in clause 5.4.13.5. During Registration procedure, a UE supporting Discontinuous coverage functionalities described in this clause and clause 5.4.13.5 provides "discontinuous coverage support" indication as part of 5GMM Core Network Capability in the Registration Request message for initial registration and for every mobility registration. The AMF indicates whether Discontinuous coverage functionalities described in this clause and clause 5.4.13.5 are supported by providing the "discontinuous coverage support" indication in Registration Accept message. The procedures in this clause are applicable only if both UE and AMF indicate support for discontinuous coverage.

If the UE is able to determine its own UE out-of-coverage period, and decides to remain in no service the following applies:

-	the UE may trigger a Periodic or Mobility Registration Update procedure to inform the network of its UE out-of-coverage period, as described in clause 5.4.1.4.

NOTE 2:	The Periodic or Mobility Registration Update procedure may occur in advance of the UE out-of-coverage period in which case the UE indicates the start time of the UE out-of-coverage period as described in clause 5.4.1.4.

If the UE is not able to determine its own UE out-of-coverage period but can determine that it is about to lose coverage, the following applies:

-	The UE triggers the Mobility Registration Update procedure and indicates that the UE is about to lose coverage due to discontinuous coverage but does not provide a UE out-of-coverage period.

-	In this case, if the AMF is able to determine a UE out-of-coverage period based on satellite coverage availability information, as described below, the AMF provides an expected unavailability duration to the UE in the Registration Accept. The AMF may take the out of coverage period into account when determining Periodic Registration Update timer value for the UE. The AMF stores the information that the UE is unavailable in UE context, and considers the UE is unreachable (i.e. clear the PPF in AMF) until the UE enters CM-CONNECTED state.

-	The UE triggers a Registration Update procedure when it returns to coverage using any access type.

-	The AMF may indicate to the UE that the UE is required to perform a Registration Update procedure both when leaving and when returning to coverage. If the AMF indicates to the UE that the UE is required to perform a Registration Update procedure when leaving coverage, the  UE is required to perform a Registration Update procedure upon leaving coverage. If the AMF indicates to the UE that the UE is required to perform a Registration Update procedure when returning to coverage, the UE shall perform a Registration Update procedure upon returning to coverage.

The UE should trigger the Mobility Registration Update procedure early enough such that the procedure, under normal conditions, is able to complete before the start of the unreachability period.

NOTE 3:	A UE based on implementation can combine successive periods of no satellite coverage into a single continuous period that is notified to the network if the UE does not require network access during this period.

NOTE 4:	UE informing the network of coverage gaps would increase signalling and UE power consumption if coverage gaps are more frequent than UE's communication period.

The AMF may be aware of the satellite coverage availability information and determine UE out-of-coverage period. In this case the following applies:

-	In addition to criteria described in other applicable clauses, the AMF may take UE out-of-coverage period into account when determining e.g. Periodic Registration Update timer, extended DRX in CM-IDLE (see clause 5.31.7.2), MICO mode configuration (see clause 5.4.1.3) and Estimated Maximum Wait Time.

-	The AMF uses existing procedures to provide the UE with timers (e.g. Periodic Registration Update timer), extended DRX in CM-IDLE (see clause 5.31.7.2), and MICO mode configuration (see clause 5.4.1.3), and to the NG-RAN Extended Connected Time (see clause 5.31.7.3). This is to keep UE in power saving mode and avoid the network attempting to page the UE if it is out of satellite network coverage.

The AMF should adjust the mobile reachable timer or Implicit Deregistration timer or both such that the AMF does not implicitly deregister the UE while the UE is out of coverage, see clause 5.4.1. Features described for High latency communication in clause 5.31.8 may be used to handle mobile terminated (MT) communication with UEs being unreachable due to discontinuous coverage.

# >>>> NEXT CHANGES<<<<

# >>>> NEXT CHANGES<<<<

#### 5.4.13.5	Overload control

The AMF and UE may only use the procedure defined in this clause if both the UE and AMF indicate "Discontinuous Coverage Supported", see clause 5.4.13.1.

In order to avoid a large number of UEs causing excessive signalling load on the network when leaving coverage or re-gaining coverage after being out of coverage, the AMF may determine a maximum time offset controlling when UEs are allowed to initiate NAS signalling with the network, as described in this clause.

In this case, the AMF determines this maximum time offset based on network configuration, priority users and priority service as specified in TS 23.122 [17] and TS 24.501 [47]. The AMF sends this maximum time offset to individual UEs during the Registration procedure or UE Configuration Update procedure.

If the UE receives a maximum time offset from the network in a Registration Accept or UE Configuration Update Command message, the UE shall replace any previously received maximum time offset on the same RAT type and PLMN with this one.

When the UE knows a later time at which coverage will be lost and when the UE does not send Mobility Registration Update to the AMF in advance (see clause 5.4.13.1), the UE determines a random value up to and including the latest maximum time offset for this PLMN and RAT type and determines an earlier time equal to the later time less the random value. The UE shall send the Mobility Registration Update to the AMF indicating the loss of coverage at the earlier time.

Upon returning in coverage after being out of coverage due to discontinuous coverage, the UE sets the discontinuous coverage wait timer value to a random value up to and including the latest maximum time offset for this PLMN and RAT type, and starts this timer. The UE shall not initiate any NAS signalling on that RAT Type and PLMN while the discontinuous coverage wait timer is running.

The UE shall stop the discontinuous coverage wait timer and initiate NAS signalling if the UE receives paging message, has pending emergency services or when UE enters a TAI outside the registration area.

Editor’s note: It is for CT1 to decide whether the maximum time offset will require a new IE or reuse existing IE used for MINT i.e. disaster return wait range information. SA2 specs will be aligned based on CT1 decision.

# >>>>END OF CHANGES<<<<