3GPP TSG-WG SA2 Meeting #156E	S2-2304249

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4253 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support URSP provisioning in EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Without knowing the UE capability of supporting URSP provisioning in EPS, the SMF+PGW-C may select a PCF for the PDU Session not supporting UE Policy Association establishment towards the PCF for the UE. Without knowing the SMF+PGW-C capability of supporting ePCO and URSP Provisioning Support in EPS, when the UE sends the UE Policy Container ePCO to EPC, the UE Policy Container ePCO may be discarded by SMF+PGW-C, which may cause the URSP rules can’t be provisioned to UE in EPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE indicates its capability of supporting URSP provisioning in EPS PCO to SMF+PGW-C for the purpose of selecting a proper PCF for the PDU Session. SMF+PGW-C indicates its capability of supporting URSP provisioning in EPS PCO to UE, based on this indication, the UE initiates the UE requested bearer resource modification procedure and sends the UE Policy Container ePCO to EPC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | URSP provisioning in EPS may fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 3979 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.17.8 URSP Provisioning in EPS

When the UE first registers in 5GS, the UE includes the Indication of URSP Provisioning Support in EPS in the UE Policy Container carried in Registration Request. Only when receiving this indication in the UE Policy Container, the PCF will provision the URSP to UE in EPS.

During Initial Attach with default PDN connection establishment procedure in EPS, the UE provides the Indication of URSP Provisioning Support in EPS PCO in the PDN connectivity request to SMF+PGW-C. If the SMF+PGW-C supports URSP provisioning in EPS, it provides the Indication of URSP Provisioning Support in EPS PCO to UE in the PDN Connectivity Accept message. When the UE receives the URSP Provisioning Support in EPS PCO and ePCO support indication from EPC, it knows that SMF+PGW-C supports URSP provisioning via ePCO, then it initiates the UE requested bearer resource modification procedure and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message, the UE Policy Container ePCO will be further forwarded by MME to SMF+PGW-C. When the UE Policy Container ePCO is received by SMF+PGW-C, it forwards transparently the UE Policy Container to PCF for the PDU Session, then the PCF for the PDU Session establishes the UE Policy Association with PCF for the UE. Then the PCF for the UE generates the corresponding URSP rules in a similar way as it is done in 5GS and then sends the URSP rules to UE in the UE Policy Container as described in TS 23.502 [3] clause 4.11.0a.5. The PDN Connection used by UE and SMF/PGW-C to convey UE Policy Container ePCO shall be kept when the UE is in the CONNECTED mode. When the UE is attached to EPS, the PCF for the PDU Session retrieves the PCRTs for UE Policy from PCF for the UE and subscribes to the applicable PCRTs to SMF+PGW-C. The PCF for the PDU Session sends the UE Policy Container in the same PDN connection/PDU session in which the UE Policy Container was received.

During EPS to 5GS mobility with N26, the UE Policy Association is terminated by PCF for PDU Session when it receives the indication of RAT type change from the SMF+PGW-C.

Editor’s note: It’s FFS on how to discover the same PCF for UE serving the UE in both EPS and 5GS.

During 5GS to EPS mobility with N26, the AMF doesn’t initiate the UE Policy Association termination procedure, instead the AMF will request the PCF for the UE to delay the termination of UE Policy Association for a configured period. The PCF for the UE initiates the UE Policy Association Termination procedure either after it receives the UE Policy Association Establishment request from PCF for the PDU Session or after the configured period. The PCF for the PDU Session determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. The PCF for the PDU Session discovers the address of PCF for the UE serving the UE by querying BSF. The PCF for the UE recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS handover").

# * * * * End of changes * * * *